HANDS-ON PERFORMANCE TASK

4L60E Valve Body Installation

Student checkoff procedure for correct hydraulic stack-up, electrical connection, bolt placement, torque, filter, and pan installation.

| UNIT 4L60E | VALVE BODY 8 ft-lb | PAN 9 ft-lb | CHECKBALLS 7 |
| --- | --- | --- | --- |

| Student | ____________________________ | Date | __________________ |
| --- | --- | --- | --- |
| Partner | ____________________________ | Unit ID | __________________ |

## Performance Objective

Objective: Install the 4L60E valve body and related components in the correct order without damaging the case, gaskets, wiring, solenoids, fasteners, or hydraulic components. Meet all listed inspection and torque requirements.

| SAFETY AND WORK HABITS [ ] Wear safety glasses and protective gloves. [ ] Keep the bench, case, and parts clean. [ ] Clean fluid spills immediately. [ ] Use hand tools only until final torque. [ ] Stop if a part does not seat correctly. | REQUIRED TOOLS AND MATERIALS [ ] Inch-pound or low-range ft-lb torque wrench [ ] Correct sockets and two alignment pins [ ] Assembly lube and lint-free wipes [ ] New O-rings, filter, and pan gasket [ ] C and V gaskets, separator plate, 7 checkballs |
| --- | --- |

## Phase 1 - Case Preparation

| Done | # | Student Performance Task | Verify / Initial |
| --- | --- | --- | --- |
| [ ] | 1 | Inventory the parts. Separate bolts by length and location. Report any missing or damaged parts before beginning. |  |
| [ ] | 2 | Inspect the case, valve body, separator plate, gasket surfaces, and passages. Remove debris without scratching sealing surfaces. |  |
| [ ] | 3 | Install the 3-4 accumulator piston pin. Fit a new piston seal and apply assembly lube inside the bore. |  |
| [ ] | 4 | Install the 3-4 accumulator piston with the open side facing up, then install its spring. |  |
| [ ] | 5 | Place checkball #91 in the case pocket next to the 3-4 accumulator. |  |
| [ ] | 6 | Verify the low-reverse capsule contains its checkball. Do not add other loose case checkballs to a 4L60E. |  |
| [ ] | 7 | Confirm the band anchor pin is fully seated, then install two valve-body alignment pins in the specified holes. |  |

| STOP POINT - A - CASE READY: Instructor verifies cleanliness, accumulator orientation, #91 checkball, capsule checkball, band anchor pin, and alignment pins. | Instructor: __________________ Date/Time: ________________ |
| --- | --- |

Reference: Valvebody Installation PDF, manual pages 162-176. Always follow the instructor's unit-specific direction if the classroom unit differs.

PHASE 2 - HYDRAULIC STACK-UP ¦ Gaskets, separator plate, accumulator housing, and checkballs

Critical identification: The case gasket is marked C. The valve-body gasket is marked V. Reversing, misaligning, or pinching these gaskets can create cross-leaks and shift complaints.

4L60E separator-plate checkball map - verify all seven positions before lowering the valve body.

| Done | # | Student Performance Task | Verify / Initial |
| --- | --- | --- | --- |
| [ ] | 8 | Place the case gasket marked C onto the case. Align every passage and bolt hole. |  |
| [ ] | 9 | Verify the 4L60E separator-plate screens are present and correctly seated. |  |
| [ ] | 10 | Lower the separator plate onto the case gasket without moving the gasket. |  |
| [ ] | 11 | Place the valve-body gasket marked V on top of the separator plate. Confirm full alignment. |  |
| [ ] | 12 | Install the correct separator stiffener plate for this unit as directed by the instructor. |  |
| [ ] | 13 | Install the 1-2 accumulator housing with the correct bolts. Torque the bolts to 8 ft-lb. |  |
| [ ] | 14 | Apply small dabs of assembly lube at the seven specified checkball locations. Install exactly seven checkballs. |  |
| [ ] | 15 | Insert the S-shaped end of the manual-valve link into the manual lever. |  |

| STOP POINT - B - HYDRAULIC STACK APPROVED: Do not lower the valve body until the instructor verifies C/V gasket order, screens, separator plate, 1-2 housing torque, seven checkballs, and manual link. | Instructor: __________________ Date/Time: ________________ |
| --- | --- |

## Phase 3 - Electrical Components and Valve Body

| Done | # | Student Performance Task | Verify / Initial |
| --- | --- | --- | --- |
| [ ] | 16 | Install new O-rings on the solenoid harness case connector and TCC solenoid. Lightly lubricate each O-ring. |  |
| [ ] | 17 | Install the harness into the case. Position the TCC solenoid correctly and install its retaining bolt. |  |
| [ ] | 18 | Move the harness clear of the gasket. Carefully lower the valve body while guiding the manual-valve link into the manual valve. |  |
| [ ] | 19 | Position the pressure-switch assembly. Install the correct bolts through it, but leave them loose. |  |

PHASE 3 CONTINUED - BOLT PLACEMENT AND TORQUE ¦ Wrong bolt length can damage the transmission

High-risk warning: Check every bolt length and location before tightening. An incorrect bolt in a middle valve-body hole can contact the sun gear shell and cause wrong-gear starts or bind-ups.

Valve-body bolt identification and location chart. Match each bolt letter and length before torque.

| Done | # | Student Performance Task | Verify / Initial |
| --- | --- | --- | --- |
| [ ] | 20 | Lay the harness in its correct routing. Connect every solenoid and pressure-switch connector; confirm each latch locks. |  |
| [ ] | 21 | Install the proper bolts through the harness hold-down eyes. Remove both alignment pins and install the remaining bolts. |  |
| [ ] | 22 | Install the dipstick stop clip at the front corner of the valve body. |  |
| [ ] | 23 | Match every bolt to the bolt-location chart. Place all bolts finger-tight and obtain instructor approval before torque. |  |
| [ ] | 24 | Torque valve-body bolts to 8 ft-lb using the numbered 1-19 sequence on page 4, beginning in the middle and working outward. |  |
| [ ] | 25 | Repeat the complete torque pattern at least once. Record final torque and wrench ID below. |  |
| [ ] | 26 | Install the manual-lever detent spring and its bolt. |  |
| [ ] | 27 | Coat the neck of a new filter with assembly lube and fully install the filter. |  |

| STOP POINT - C - TORQUE VERIFIED: Instructor verifies bolt lengths, bolt locations, harness routing, connectors, dipstick stop clip, torque sequence, and final 8 ft-lb torque. | Instructor: __________________ Date/Time: ________________ |
| --- | --- |

| Torque wrench ID | ____________________ | Final setting | 8 ft-lb |
| --- | --- | --- | --- |
| First pass | [ ] Complete | Second pass | [ ] Complete |

## Phase 4 - Pan Installation and Final Inspection

| Done | # | Student Performance Task | Verify / Initial |
| --- | --- | --- | --- |
| [ ] | 28 | Place the magnet in its correct location inside the clean pan. |  |
| [ ] | 29 | Install a new pan gasket on the case rail and lower the pan into place without shifting the gasket. |  |
| [ ] | 30 | Start every pan bolt by hand before tightening any bolt. Torque the pan bolts to 9 ft-lb; repeat the pattern once. |  |
| [ ] | 31 | Complete the final inspection: filter seated, harness not pinched, connectors locked, manual linkage operating, pan gasket aligned, and work area clean. |  |

KNOWLEDGE CHECK AND INSTRUCTOR EVALUATION ¦ Complete before submitting the unit

## Torque Sequence Reference

Use the numbered 1-19 sequence for the first and second 8 ft-lb valve-body torque passes.

## Final Quality-Control Checklist

| HYDRAULIC / MECHANICAL [ ] C gasket below separator plate [ ] V gasket above separator plate [ ] Seven valve-body checkballs installed [ ] 1-2 accumulator housing torqued [ ] Manual valve link fully engaged [ ] Filter fully seated | ELECTRICAL / FASTENERS [ ] New connector and TCC O-rings installed [ ] All connector latches locked [ ] Harness routed and retained [ ] Correct bolt length in every hole [ ] Valve-body torque repeated at 8 ft-lb [ ] Pan torque repeated at 9 ft-lb |
| --- | --- |

## Knowledge Check

| 1 | What does the C identify, and where does that gasket belong? ________________________________________________________________________________________________ |
| --- | --- |
| 2 | What does the V identify, and where does that gasket belong? ________________________________________________________________________________________________ |
| 3 | Why must every valve-body bolt length be verified before tightening? ________________________________________________________________________________________________ |
| 4 | Why is the valve body torqued from the center outward and the pattern repeated? ________________________________________________________________________________________________ |
| 5 | Name two problems that a pinched harness, unlocked connector, or damaged O-ring could cause. ________________________________________________________________________________________________ |

## Instructor Evaluation

| Result | [ ] Satisfactory [ ] Needs correction [ ] Not completed |
| --- | --- |
| Corrections |  |
| Student | Signature: ________________________________ Date: ____________ |
| Instructor | Signature: ________________________________ Date: ____________ |
| Score | ________ / 100 |

Successful completion requires correct assembly, verified torque, safe work habits, and an approved final inspection.