STUDENT HANDOUT

# Automatic Transmission Hydraulics

Hydraulic flow sequence, key terms, syringe lab, and exit ticket

NAME  ________________________________     DATE  ________________     CLASS  ________________

| TRACE Follow ATF from the pan to an apply device and back. | EXPLAIN Separate pressure, flow, force, air, and leakage. | CONNECT Link syringe behavior to transmission symptoms. |
| --- | --- | --- |

BIG IDEA The geartrain cannot shift itself. Hydraulic pressure turns a command into holding force.

# Hydraulic Flow Sequence

- Pan / filter: Stores the ATF and removes debris before fluid enters the pump.

- Pump: Draws in ATF and creates fluid flow. The pump is the flow source.

- Pressure regulator: Controls line pressure by balancing pump output, spring/boost force, feedback oil, and exhaust.

- Manual / control valve: Selects a range and opens, blocks, or exhausts the needed circuit.

- Valve body: Routes fluid through spool valves, solenoids, checkballs, orifices, and passages.

- Apply device: A clutch piston or servo converts pressure into force so a clutch or band can hold.

- Cooler / lubrication / return: ATF removes heat, lubricates moving parts, and returns to the pan.

FLOW PAN/FILTER -> PUMP -> REGULATOR -> CONTROL -> VALVE BODY -> APPLY -> COOL/LUBE/RETURN

Important branch: After regulation, ATF divides into apply circuits and secondary converter/cooler/lubrication circuits. Inactive apply circuits also need an exhaust or release path.

REFERENCE

# Key Terms and Core Rules

Use the short definitions to explain what each component does in the circuit.

| TERM | MEANING | TERM | MEANING |
| --- | --- | --- | --- |
| Pressure | Force acting over an area; measured in psi or kPa. | Flow | Fluid volume moving over time. |
| Force | Push produced by pressure acting on area: F = P x A. | Pascal's Law | Pressure applied to confined liquid is transmitted equally in all directions. |
| Reaction area | Surface area on which hydraulic pressure acts. | Line pressure | Regulated main pressure feeding transmission circuits. |
| Pressure regulator | Meters or exhausts flow to maintain the target pressure. | Spool valve | Sliding valve that connects, blocks, or meters ports. |
| Land | Large-diameter section of a spool valve that blocks or reacts to pressure. | Manual valve | Range-selecting valve connected to the shifter or an actuator. |
| Solenoid | Electrical actuator that switches or meters hydraulic control. | Checkball | One-way or shuttle valve that controls flow direction. |
| Orifice | Precision restriction that controls fill or exhaust rate. | Accumulator | Piston/spring device that absorbs flow to shape pressure rise and shift feel. |
| Apply device | Clutch piston, servo, clutch, or band that converts pressure into holding action. | Aeration | Air mixed with ATF; compresses and causes soft, delayed, or erratic response. |
| Leakage | Flow escaping through a seal, bore, gasket, or clearance. | ATF | Transmits pressure, lubricates, cools, controls friction, and helps clean/seal. |

PRESSURE = FORCE / AREA     |     FORCE = PRESSURE x AREA     |     AREA = FORCE / PRESSURE

## Three Rules to Remember

| AIR CHANGES RESPONSE | Bubbles compress before pressure can move the load. |
| --- | --- |
| AREA TRADES FORCE AND TRAVEL | A larger output piston makes more force but moves a shorter distance. |
| LEAKS CONSUME FLOW | Escaping flow cannot fill the apply chamber on time. |

ACTIVITY 1 | 20 MINUTES

# Syringe Hydraulic Lab

Compare liquid, trapped air, and unequal piston areas.

SAFETY Wear safety glasses. Use colored water - never ATF - at student tables. Work over a tray, wipe spills immediately, and keep fluid away from electrical trainers.

## Materials

2 syringes  |  Clear tubing  |  Colored water  |  Tray and towels  |  Safety glasses

## Procedure

- Prepare: Put on safety glasses and place all parts inside the tray.

- Connect: Attach the clear tubing securely between two same-size syringes.

- Fill and purge: Fill the system with colored water and slowly cycle it until most trapped air is removed.

- Test liquid: Push one piston slowly. Observe the response speed, piston travel, force feel, bubbles, and leaks.

- Test trapped air: Reset the system with a small air pocket trapped inside. Repeat the same slow push and observe the change.

- Test area: Install unequal syringe sizes, purge the air, and repeat. Compare force and travel at each piston.

- Record: Complete all three rows in the observation table using specific evidence.

- Clean up: Drain the water as directed, dry the equipment and tray, and inspect the floor for spills.

## Observation Table

| SETUP | RESPONSE SPEED | RELATIVE TRAVEL | FORCE / FEEL | BUBBLES, LEAKS, OR OTHER EVIDENCE |
| --- | --- | --- | --- | --- |
| A. Liquid, same size |  |  |  |  |
| B. Trapped air |  |  |  |  |
| C. Unequal sizes |  |  |  |  |

LAB TRANSFER

# Connect the Model to the Transmission

Use observable evidence and circuit logic - not a vague answer such as 'bad transmission.'

## Lab-to-Transmission Connections

| LAB OBSERVATION | HYDRAULIC RULE | POSSIBLE TRANSMISSION EFFECT |
| --- | --- | --- |
| Immediate liquid response | Liquid is nearly incompressible. | Firm, repeatable clutch or servo application. |
| Soft or delayed air response | Air compresses before the load moves. | Delay, flare, chatter, or a bouncing pressure gauge. |
| Larger output piston | More area produces more force but needs more volume. | Greater holding force with a force/travel tradeoff. |
| Visible or felt leakage | Escaping flow cannot fill the chamber on time. | Delayed engagement, slip, flare, or low circuit pressure. |

## Analysis

- Why did the liquid-filled system respond differently from the system with trapped air?

____________________________________________________________________________________________

- With unequal syringes, which piston produced greater force? Which piston moved farther?

____________________________________________________________________________________________

- Write one evidence statement: 'I observed... Therefore... My next test will determine...'

____________________________________________________________________________________________

# Exit Ticket

1 ¦ TRACE Write the seven hydraulic stops in order from the pan/filter to the apply device and return.

____________________________________________________________________________________________

2 ¦ DIAGNOSE Line pressure is normal cold but low hot. Name two likely hydraulic causes, then state the first test or observation you would use.

____________________________________________________________________________________________

____________________________________________________________________________________________

TAKEAWAY: The pump creates ____________. Pressure develops when flow meets ____________________.