Chapter 35: Clutch Systems
Student Guided Notes Worksheet

| Name | Date | Class | Period |
| --- | --- | --- | --- |

# Learning Objectives

☐ LO 35-1: Describe clutch principles.

☐ LO 35-2: Describe the purpose and design of clutch components.

☐ LO 35-3: Describe clutch operating mechanisms.

☐ LO 35-4: Perform clutch maintenance.

☐ LO 35-5: Diagnose clutch issues.

☐ LO 35-6: Perform a clutch replacement.

# LO 35-1: Clutch Principles

1. A clutch engages the engine to the ____________________________ and transmits torque.

2. A dry clutch uses ____________________________ to transmit torque in many manual transmissions.

3. Wet clutches run in ____________________________ fluid and are common in automatic transmissions.

4. Four torque-capacity factors are coefficient of friction, diameter, number of discs, and total ____________________________ spring force.

5. Increasing clutch diameter increases leverage and torque ____________________________.

6. In low gear, input shaft speed is high, output shaft speed is low, and output shaft torque ____________________________.

7. During gear transitioning, the relative speeds of the input or output shaft must ____________________________.

8. The transmission output shaft is connected directly to the ____________________________.

# LO 35-2: Component Matching

Directions: Match each term to the best description.

| Answer | Term | Description Choices |
| --- | --- | --- |
| ____ | Flywheel | A. Smooths power pulses, stores energy, and provides a friction surface. |
| ____ | Clutch Disc | B. Driven friction disc with splined hub attached to the input shaft. |
| ____ | Pressure Plate | C. Provides clamping force so torque can be transmitted. |
| ____ | Throw-out Bearing | D. Works with the fork to compress pressure plate springs. |
| ____ | Clutch Fork | E. Connects bearing thrust face with pressure plate levers/fingers. |
| ____ | Pilot Bearing | F. Supports and aligns the transmission input shaft snout. |
| ____ | Dual-mass Flywheel | G. Absorbs torsional crankshaft vibrations and improves smoothness. |
| ____ | Waved Spring Segments | H. Allow progressive application and smoother clutch engagement. |

# LO 35-2: Purpose and Design of Components

1. A flywheel is bolted to the rear of the ____________________________.

2. A heavy flywheel slows engine acceleration, while a lighter flywheel is easier to ____________________________.

3. The starter ring gear meshes with the starter motor drive ____________________________.

4. A stepped flywheel has a recessed friction ____________________________.

5. A diaphragm pressure plate uses a diaphragm ____________________________ to create/release clamping force.

6. Coil spring pressure plates use release levers to release the clutch ____________________________.

7. Multi-plate clutches increase torque capacity by adding ____________________________ area.

8. A collar-style slave cylinder fits around the input shaft and acts directly on the ____________________________ bearing.

9. Pilot bearings may be brass/bronze bushings, needle-type bearings, or ____________________________-type bearings.

# Clutch Operating Mechanisms Comparison

| System Type | Main Parts | Strength/Advantage | Important Service Note |
| --- | --- | --- | --- |
| Linkage style |  |  |  |
| Cable style |  |  |  |
| Hydraulic style |  |  |  |

# LO 35-4: Clutch Maintenance

1. Clutch maintenance includes checking cable/linkage systems for free play and ____________________________.

2. Hydraulic clutches should be checked for fluid condition, needed flushes, and ____________________________.

3. Older clutch linings may contain ____________________________, a carcinogen.

4. A ____________________________ jack supports the transmission during removal/installation.

5. A dial indicator can measure runout and crankshaft end ____________________________.

6. A clutch alignment tool centers the clutch ____________________________ during installation.

7. Too little free play can keep the throw-out bearing in contact with pressure plate ____________________________.

8. Too much free play can cause gears to clash or ____________________________ when shifting.

9. Common clutch hydraulic bleeding methods include gravity, manual, and pressure/____________________________ bleeding.

# Clutch Diagnosis Chart

| Complaint/Symptom | What the driver may notice | Possible Cause | Inspection/Test Direction |
| --- | --- | --- | --- |
| Slippage |  |  |  |
| Binding/Drag |  |  |  |
| Pedal pulsation |  |  |  |
| Chatter |  |  |  |
| Throw-out bearing noise |  |  |  |
| Pilot bearing noise |  |  |  |

# LO 35-6: Clutch Replacement

1. Replacing a clutch requires removal of the transmission or ____________________________.

2. When disassembling a clutch, loosen pressure plate bolts gradually in a ____________________________ pattern.

3. Clutch disc discoloration may show burns caused by clutch ____________________________.

4. Flywheel inspection includes checking for bluing, hot spots, and ____________________________.

5. Flywheel runout is measured with a ____________________________ indicator.

6. Crankshaft end play is forward/rearward movement of the ____________________________.

7. Alignment ____________________________ line up the transmission and engine.

8. The clutch disc may be directional and should be installed in the correct ____________________________.

9. After installation, the vehicle should be test-driven to confirm clutch ____________________________.

# Procedure Sequence: Clutch Assembly Installation

Directions: Place the major steps in the correct order by writing 1-8.

| Order | Step |
| --- | --- |
| ____ | Install a clutch alignment tool through the clutch disc into the pilot bearing. |
| ____ | Research procedure and specifications. |
| ____ | Clean the flywheel friction surface. |
| ____ | Install and torque flywheel bolts in a star pattern. |
| ____ | Place clutch disc on flywheel in the correct direction. |
| ____ | Install pressure plate and tighten bolts gradually in a star pattern. |
| ____ | Remove alignment tool after bolts are torqued. |
| ____ | Install throw-out bearing/fork and lubricate specified points. |

# Short Answer / Review

1. Why must the clutch disconnect the engine from the transmission during a gear shift?

Answer: _______________________________________________________________________________________________

_________________________________________________________________________________________________________

2. What happens when clutch pedal free play is too little?

Answer: _______________________________________________________________________________________________

_________________________________________________________________________________________________________

3. What happens when clutch pedal free play is too much?

Answer: _______________________________________________________________________________________________

_________________________________________________________________________________________________________

4. Why should the flywheel often be resurfaced when replacing a clutch?

Answer: _______________________________________________________________________________________________

_________________________________________________________________________________________________________

5. Why should you not use bell housing bolts to pull the transmission into place?

Answer: _______________________________________________________________________________________________

_________________________________________________________________________________________________________

6. Why is asbestos dust a safety concern during older clutch service?

Answer: _______________________________________________________________________________________________

_________________________________________________________________________________________________________

# Exit Ticket

1. In your own words, explain one thing the clutch must do:

______________________________________________________________________________________________________________

2. Name one safety or service detail you should remember when working on clutch systems:

______________________________________________________________________________________________________________