# Circuit Lab Build and Measure

Electrical Fundamentals  |  Student Lab Sheet

Name: ____________________________________  Date: ____________________
Program: _________________________________  Partner: __________________
Assigned circuit or loads: __________________________________________________

Objective: Build a working circuit, document its components, and measure battery voltage, component resistance, and load voltage drop.

Materials: Instructor-approved low-voltage source, circuit protection, switch, loads, wires, multimeter, blank paper, clear tape, and pencil.

# 1 Build and Document Your Circuit

☐ Keep the power source disconnected while assembling the circuit.

☐ Spread the components across a blank sheet of paper.

☐ Connect the source, protection, switch, and assigned loads in a complete circuit.

☐ Keep wires neat and secure them with clear tape.

☐ Draw each component’s symbol beside it. Label it and describe its function.

☐ Have the instructor inspect your circuit before connecting power.

Instructor approval before power ON: ________________________________________

| Component | Draw its symbol | Describe its job |
| --- | --- | --- |
| Power source |  |  |
| Circuit protection |  |  |
| Switch |  |  |
| Load 1: ______________ |  |  |
| Load 2 if used: _________ |  |  |
| Wires or conductors |  |  |

Attach your labeled circuit layout to this sheet. Operate the switch and describe what your circuit does:

________________________________________________________________________________

________________________________________________________________________________

# 2 Measure Battery Voltage

Use DC volts with black lead in COM and red lead in V/Ω. Measure across the battery: red probe on positive and black probe on negative.

| Circuit condition | Battery voltage |
| --- | --- |
| Switch OFF | ____________ V |
| Switch ON with load operating | ____________ V |

OFF voltage − ON voltage = __________ V. What changed when the load turned on?

________________________________________________________________________________

# 3 Measure Resistance with Power Disconnected

Disconnect the battery and verify no voltage is present before selecting Ω. Isolate at least one component terminal to avoid alternate paths. Never measure resistance on an energized circuit.

Touch the probes together. Test-lead resistance: ________ Ω. Measure across the switch in each position, then across each isolated load. Record OL if displayed, not zero.

| Component or condition | Resistance |
| --- | --- |
| Switch OPEN or OFF | __________ Ω or OL |
| Switch CLOSED or ON | __________ Ω or OL |
| Load 1: __________________________ | __________ Ω or OL |
| Load 2 if used: ____________________ | __________ Ω or OL |

Use instructor-approved loads; LEDs and electronic loads may require a different test.

Which switch position completes the current path? ______________________________
What does OL indicate during a resistance test? _________________________________

# 4 Measure Load Voltage Drop with Circuit ON

Reconnect components with power disconnected. Return the meter to DC volts, connect power, and turn the circuit ON. Probe across each operating load: red toward positive and black toward the return side.

| Load | Voltage drop | Operating normally |
| --- | --- | --- |
| Load 1: __________________ | ________ V | ☐ Yes ☐ No |
| Load 2 if used: ______________ | ________ V | ☐ Yes ☐ No |

Compare each load’s voltage drop with the battery’s ON voltage:

________________________________________________________________________________

# 5 Explain and Check Your Work

Why disconnect power before measuring resistance? ______________________________
________________________________________________________________________________

How does a resistance test differ from a voltage drop test? ________________________
________________________________________________________________________________

Describe a problem you corrected, or how you confirmed correct operation: __________
________________________________________________________________________________

☐ Circuit operates   ☐ Wires secured   ☐ Labels complete   ☐ Units recorded
☐ Power disconnected and work area clean

Instructor initials: __________________________  Date: _____________________