# Circuit Lab Worksheet

Build and Measure

Name __________________________________  Date ______________  Team __________

Program _______________________________  Circuit or load _______________________

Build a working circuit, document its components, and use a digital multimeter to measure battery voltage, resistance, and voltage drop.

# 1  Build and document your circuit

Use the instructor-approved low-voltage DC source and components. Spread the circuit across a blank sheet of paper. Keep wires neat and secure them with clear tape. Leave terminals accessible for testing.

Before powering up: Check polarity, secure connections, and have your instructor inspect the circuit. Prevent a direct connection between battery positive and negative.

Draw the circuit below using electrical symbols. Label the battery, switch, load devices, and wires. Use a separate blank sheet for the physical build.

# Component identification

| Component name | Electrical symbol | Job in this circuit |
| --- | --- | --- |
| Battery / source |  |  |
| Switch |  |  |
| Load 1 ______________ |  |  |
| Load 2 if used __________ |  |  |

Build check:  □ Neat layout   □ Wires secured   □ Components labeled   □ Circuit works

Instructor initials before testing __________

# Circuit Measurements

Name __________________________________  Date ______________

Meter setup: Black lead in COM; red lead in V/Ω. Use DC volts for voltage tests. Connect the meter across the battery or component being measured.

# 2  Measure battery voltage

Measure directly across the battery terminals with the switch OFF, then ON.

| Circuit condition | Battery voltage |
| --- | --- |
| Switch OFF | ________________ V |
| Switch ON and load operating | ________________ V |

# 3  Measure resistance

Disconnect the power source before selecting Ω. Isolate at least one terminal of the component from the circuit. Never measure resistance on a powered circuit. Record OL if displayed.

| Component and condition | Resistance |
| --- | --- |
| Switch OPEN | ________________ Ω / OL |
| Switch CLOSED | ________________ Ω |
| Load 1 ______________________________ | ________________ Ω / OL |
| Load 2 if used ________________________ | ________________ Ω / OL |

# 4  Measure voltage drop

Reconnect the circuit and select DC volts. Turn the circuit ON so the loads operate. Place one probe on each side of the load and record the voltage across it.

| Load device | Voltage drop with circuit ON |
| --- | --- |
| Load 1 ______________________________ | ________________ V |
| Load 2 if used ________________________ | ________________ V |

# 5  Explain your results

1. How did battery voltage change when the circuit was turned ON?

________________________________________________________________________________

2. Compare the resistance of the open and closed switch. What does this tell you?

________________________________________________________________________________

3. What did you observe when the circuit operated? Describe any problem and correction.

________________________________________________________________________________

________________________________________________________________________________

Final check:  □ Readings include units   □ Power disconnected   □ Work area cleaned