ELECTRICAL 1 PRACTICAL ASSESSMENT

Hands-On Midterm  |  Condensed Student Packet

| STUDENT |  | DATE |  | INSTRUCTOR |  |
| --- | --- | --- | --- | --- | --- |

| STATION | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| POSSIBLE / 100 | 10 | 12 + 3 | 12 | 13 | 13 | 16 | 12 | 12 |
| SCORE |  |  |  |  |  |  |  |  |

SAFETY: Complete all 8 stations. Clarification has no penalty. If the instructor supplies the next procedure step, that station is capped at 50%. Use measured values as calculation inputs. Stop for unsafe or unexpected circuit behavior.

STATION 1  |  ID TEST TYPE    10 points

Directions: Meters 1-8 are pre-positioned. Do not move leads or change a dial. Identify the exact point/component, state, and test type: available voltage, voltage drop, resistance/continuity, or current.

| Meter | Test type | Exact point / component | Circuit state |
| --- | --- | --- | --- |
| 1 |  |  | [ ] On [ ] Off |
| 2 |  |  | [ ] On [ ] Off |
| 3 |  |  | [ ] On [ ] Off |
| 4 |  |  | [ ] On [ ] Off |
| 5 |  |  | [ ] On [ ] Off |
| 6 |  |  | [ ] On [ ] Off |
| 7 |  |  | [ ] On [ ] Off |
| 8 |  |  | [ ] On [ ] Off |

Score: ______ / 10    Initials: ______

STATION 2  |  DRAW & ID CIRCUIT 1 - SERIES    12 points + 3 bonus

Directions: Trace the board's source, fuse, switch, and 3 series loads. Connect the printed symbols terminal-to-terminal; then identify A-F.

| A: ____________________ | B: ____________________ | C: ____________________ |
| --- | --- | --- |
| D: ____________________ | E: ____________________ | F: ____________________ |

Circuit type: ______________   Evidence: ______________________________________________

Bonus - list up to 3 improvements: 1. __________________  2. __________________  3. __________________

Score: ______ / 12 (+3 bonus)    Initials: ______

STATION 3  |  DRAW & ID CIRCUIT 2 - PARALLEL    12 points

Directions: Trace the board and draw a complete schematic without a symbol bank. Label B1, F1, SW1, and L1-L3; show all junctions and the return path.

Draw the complete parallel circuit here.

Circuit type: ____________   Current paths: _____   Evidence: ___________________________________

Check: [ ] source  [ ] fuse  [ ] switch  [ ] 3 loads  [ ] junction dots  [ ] return path

Score: ______ / 12    Initials: ______

STATION 4  |  DIAGNOSE FAULT 1 - OPEN CIRCUIT    13 points

Directions: The lamp does not illuminate with the switch on. Locate the exact open without disassembly unless directed. Use the meter to narrow the fault; do not guess or replace parts. Record meaningful tests in order. De-energize before resistance/continuity tests.

| SOURCE | _____ V | METER | ________ | ENERGIZED? | [ ] Yes [ ] No |
| --- | --- | --- | --- | --- | --- |

| # | Setting / test | Lead locations / point | Reading | What this proves / next step |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |

| FAULT TYPE | [ ] Open circuit [ ] Other: ______________________________ |
| --- | --- |
| EXACT LOCATION + EVIDENCE |  |
| REPAIR + VERIFICATION |  |

Score: ______ / 13    Initials: ______

STATION 5  |  DIAGNOSE FAULT 2 - UNWANTED RESISTANCE    13 points

Directions: The lamp is noticeably dim. Locate the excessive voltage loss and identify the power side, ground side, or component. Use the meter to narrow the fault; do not guess or replace parts. Record meaningful tests in order. De-energize before resistance/continuity tests.

| SOURCE | _____ V | METER | ________ | ENERGIZED? | [ ] Yes [ ] No |
| --- | --- | --- | --- | --- | --- |

| # | Setting / test | Lead locations / point | Reading | What this proves / next step |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |

| FAULT TYPE | [ ] Unwanted resistance [ ] Other: ______________________________ |
| --- | --- |
| EXACT LOCATION + EVIDENCE |  |
| REPAIR + VERIFICATION |  |

Score: ______ / 13    Initials: ______

STATION 6  |  MEASURE AND CALCULATE - STEM BOARD    16 points

SAFETY: Measure items 1-7 first. Every calculation must substitute measured numbers only; do not use a calculated answer as another input. Source OFF/disconnected for resistance tests; return the red lead to the V/ohm jack after current testing.

| # | Symbol | Measured quantity | Measured value | Unit |
| --- | --- | --- | --- | --- |
| 1 | ET | Source / total voltage |  | V |
| 2 | IT | Total current |  | A |
| 3 | ER1 | Voltage across R1 |  | V |
| 4 | ER2 | Voltage across R2 |  | V |
| 5 | R1 meas. | Isolated R1 resistance |  | ohm |
| 6 | R2 meas. | Isolated R2 resistance |  | ohm |
| 7 | RT meas. | Total resistance; source off |  | ohm |

| # | Find | Formula | Substitute measured numbers | Answer | Unit |
| --- | --- | --- | --- | --- | --- |
| 8 | R1 calc. | ER1 / IT |  |  | ohm |
| 9 | R2 calc. | ER2 / IT |  |  | ohm |
| 10 | RT calc. | ET / IT |  |  | ohm |
| 11 | PR1 | ER1 x IT |  |  | W |
| 12 | PR2 | ER2 x IT |  |  | W |
| 13 | PT | ET x IT |  |  | W |
| 14 | V check | ER1 + ER2 |  |  | V |

Check: ER1 + ER2 approximately equals ET?  [ ] Yes  [ ] No   Difference: __________________

Score: ______ / 16    Initials: ______

STATION 7  |  COMPONENT TESTING    12 points

SAFETY: Test all 10 labeled parts. Record the setting, numeric reading with unit, condition, and evidence. Power must be off for resistance/continuity; never ohm-test a battery.

Quick guide: wire/fuse/closed switch <0.5 ohm; coin battery good >=2.9 V, weak 2.5-2.8 V; 9 V battery good >=8.3 V, usable 7.5-8.2 V, weak 6.0-7.4 V.

| # | Component | Setting | Reading + unit | Condition | Evidence |
| --- | --- | --- | --- | --- | --- |
| 1 | Fuse A |  |  | [ ] Good [ ] Faulty |  |
| 2 | Fuse B |  |  | [ ] Good [ ] Faulty |  |
| 3 | 9 V battery |  |  | [ ] Good [ ] Faulty |  |
| 4 | 3 V coin battery |  |  | [ ] Good [ ] Faulty |  |
| 5 | Switch |  |  | [ ] Good [ ] Faulty |  |
| 6 | Wire A |  |  | [ ] Good [ ] Faulty |  |
| 7 | Wire B |  |  | [ ] Good [ ] Faulty |  |
| 8 | Wire C |  |  | [ ] Good [ ] Faulty |  |
| 9 | Lamp A |  |  | [ ] Good [ ] Faulty |  |
| 10 | Lamp B |  |  | [ ] Good [ ] Faulty |  |

Summary: ____________________________________________________________________

Score: ______ / 12    Initials: ______

STATION 8  |  BUILD A CIRCUIT    12 points

SAFETY: Build the assigned circuit with a source, fuse, switch, and at least 2 loads. Draw first; do not energize before instructor approval. After sign-off, disassemble and return parts.

| TYPE | [ ] Series [ ] Parallel | SOURCE | _____ V | APPROVED | Initials: ______ |
| --- | --- | --- | --- | --- | --- |

Schematic plan - label source, fuse, switch, loads, polarity, and junctions.

Build check: [ ] fuse near source  [ ] secure  [ ] correct arrangement  [ ] polarity  [ ] neat  [ ] no short risk

| Required measurement | Meter setting | Reading | What it proves |
| --- | --- | --- | --- |
| Source available voltage |  |  |  |
| Voltage across Load 1 |  |  |  |
| Voltage across Load 2 |  |  |  |
| Instructor-selected test |  |  |  |

| HELP LEVEL | FINAL OPERATION | SIGN-OFF |
| --- | --- | --- |
| [ ] Independent [ ] Clarification [ ] Coached - 50% cap | [ ] Operates [ ] Measurements complete [ ] Disassembled / parts returned | Signature: __________________ Time: __________ |

Score: ______ / 12    Initials: ______