ELECTRICAL 1 • STUDENT WORKBOOK Electrical Components Identify parts • Build safely • Measure behavior • Diagnose faults • Explain the circuit

| STUDENT ________________________________ | INSTRUCTOR ________________________________ |
| --- | --- |
| CLASS / PERIOD ________________________________ | DATE STARTED ________________________________ |

## Your learning roadmap

| 01 IDENTIFY Name the part and terminals | 02 BUILD Create a complete current path | 03 MEASURE Use evidence, not guesses | 04 DIAGNOSE Isolate faults and explain why |
| --- | --- | --- | --- |

SUCCESS STANDARD A working circuit is only the beginning. You must also trace the current path, record a measurement, locate a likely fault, and explain the result in your own words.

## Safety agreement

- Battery disconnected while installing or moving components.

- Polarity and transistor pinout verified before power is applied.

- Every standard LED circuit includes a series current-limiting resistor.

- Instructor pre-power check completed when the lab calls for one.

| STUDENT SIGNATURE ________________________________________ | INSTRUCTOR / DATE ________________________________________ |
| --- | --- |

START HERE

# Kit Inventory + First Identification

Count the parts first. Then identify three unknown items using shape, markings, and a meter check.

| EXPECTED | COMPONENT / SUPPLY | COUNT FOUND |
| --- | --- | --- |
| 1 | Breadboard | ________ |
| 1 | 9 V battery connector for powering the breadboard | ________ |
| 1 bag | Jumper wires | ________ |
| 1 bag | Assorted fixed resistors, including 220 Ω, 470 Ω, and 100 kΩ | ________ |
| 1 box | LEDs in several colors, including one 4-lead RGB LED | ________ |
| 1 each | Thermistor, potentiometer, photoresistor (LDR), and buzzer | ________ |
| 5+ each | Transistors, tactile switches, and diodes | ________ |
| 10+ each | 10 µF and 100 µF capacitors | ________ |

MISSING OR DAMAGED PART Circle the inventory row, place the part aside, and notify the instructor before beginning a powered lab.

| UNKNOWN PART | VISUAL CLUE / MARKING | METER OR POLARITY CHECK | INSTRUCTOR |
| --- | --- | --- | --- |
| ________________ | ________________________ | ________________________ | ______ |
| ________________ | ________________________ | ________________________ | ______ |
| ________________ | ________________________ | ________________________ | ______ |

| CONFIDENCE CHECK The component I can identify fastest is… ________________________________________ ________________________________________ | QUESTION FOR INSTRUCTOR The part I still confuse with another part is… ________________________________________ ________________________________________ |
| --- | --- |

REFERENCE 1

# Power, Connections, Resistance, and Light

Use the JOB and TEST / CHECK lines to move from appearance to evidence.

| Breadboard JOB: Creates temporary electrical nodes without soldering. TEST / CHECK: Continuity-test a five-hole terminal group and each power rail. | Fixed resistor JOB: Limits current or produces a planned voltage drop. TEST / CHECK: Read the color bands, then verify resistance with power off. VEHICLE LINK: Blower resistors and sensor pull-up networks. |
| --- | --- |
| LED JOB: Produces light when forward biased. TEST / CHECK: Identify anode/cathode; verify a series resistor is present. VEHICLE LINK: Indicators, lighting, and displays. | 9 V connector + jumpers JOB: Carries source power and connects circuit nodes. TEST / CHECK: Verify connector polarity and measure source voltage before use. |

LED SIZING For a 9 V source and a typical red LED, 470 Ω is a safer starting value than 220 Ω. Estimate resistance with R = (Vsource − VLED) / ILED.

REFERENCE 2

# Sensors, Controls, and Energy Storage

| Thermistor JOB: Changes resistance with temperature. TEST / CHECK: Measure resistance, warm the part gently, and note the direction of change. VEHICLE LINK: Coolant and intake-air temperature sensing. | Photoresistor (LDR) JOB: Changes resistance with light intensity. TEST / CHECK: Compare resistance in bright light and when covered. VEHICLE LINK: Automatic headlamp concepts. |
| --- | --- |
| Potentiometer JOB: Uses a movable wiper to vary resistance or signal voltage. TEST / CHECK: Find the two ends and the wiper; measure while rotating. VEHICLE LINK: Position and level sensing. | Buzzer JOB: Converts electrical energy into sound. TEST / CHECK: Determine whether it is active (DC) or passive (pulsed signal). |
| RGB LED JOB: Combines red, green, and blue LED elements. TEST / CHECK: Identify common-anode or common-cathode and all four leads. | Capacitor JOB: Stores charge for filtering, timing, or noise control. TEST / CHECK: Identify capacitance; verify polarity on electrolytic types. VEHICLE LINK: Module power filtering and signal conditioning. |

| INPUT | SENSING / CONTROL PART | ELECTRICAL RESPONSE |
| --- | --- | --- |
| Temperature | Thermistor | Resistance changes |
| Light | LDR | Resistance changes |
| Position | Potentiometer | Resistance or signal voltage changes |

REFERENCE 3

# Semiconductors, Switching, and Polarity

| NPN transistor JOB: Uses a small base signal to switch a larger collector-emitter current. TEST / CHECK: Confirm the exact emitter-base-collector pin order before wiring. VEHICLE LINK: Lamp, relay, solenoid, and motor drivers. | Diode JOB: Conducts conventional current mainly in one direction. TEST / CHECK: Band = cathode; compare diode-test readings in both directions. VEHICLE LINK: Flyback and reverse-polarity protection. |
| --- | --- |
| Tactile pushbutton JOB: Opens or closes a signal path when pressed. TEST / CHECK: Same-side pins stay common; opposite sides switch open to closed. | Electrolytic capacitor JOB: Provides larger capacitance in a polarized package. TEST / CHECK: Match the negative stripe and lead polarity to the circuit. |

PINOUT WARNING PN2222/2N2222 and BC547 packages may not share the same lead order. Read the exact part marking and use the matching pinout before applying power.

| CHECK POINT | EXPECTED | MEASURED | PASS / FIX |
| --- | --- | --- | --- |
| Diode forward direction | One junction drop | ________ | ________ |
| Diode reverse direction | OL | ________ | ________ |
| Switch across opposite sides | OL → about 0 Ω | ________ | ________ |

BREADBOARD FUNDAMENTALS

# Read the Hidden Connections

A breadboard is a map of nodes. Your job is to know which holes share metal underneath.

| Terminal strips + center trench Groups of five are connected; the center trench separates the two sides. | Power + ground rails Red is normally positive and black is normally negative. |
| --- | --- |

## Four rules that prevent most breadboard faults

- Holes a–e share one terminal-strip node; holes f–j share a separate node.

- The center trench prevents the two sides from being connected.

- Some power rails split at the midpoint—verify continuity before assuming a full-length rail.

- Two leads placed in the same five-hole group are electrically connected, even without a jumper.

Bridge a split rail with a jumper only after continuity testing confirms the break.

| TRACE IT Circle one five-hole node on the image and describe what is connected. ________________________________________ ________________________________________ | FAULT PREDICTION What happens if both LED leads are placed in the same node? ________________________________________ ________________________________________ |
| --- | --- |

BREADBOARD FUNDAMENTALS

# Pre-Power Inspection + Circuit Logic

A complete circuit needs a source, a path through the load, and a return path. The series resistor protects the LED.

## Use this order before every powered test

- POWER OFF — inspect for a direct B+ to ground short.

- TRACE — follow the full current path from B+ through each device to ground.

- POLARITY — confirm the LED, diode, capacitor, and transistor orientation.

- PROTECTION — confirm the LED resistor is truly in series.

- POWER ON — measure source voltage, then move through the circuit one node at a time.

DIAGNOSTIC HABIT Start with the source and ground. Then divide the circuit into smaller sections. A voltage measurement tells you where electrical potential exists; it does not by itself prove current is flowing.

| INSTRUCTOR PRE-POWER STANDARD ________________________________________ | STUDENT TRACES B+ → LOAD → GROUND ________________________________________ |
| --- | --- |

LAB 1

# Three Basic Circuit Types

Build series, parallel, and series-parallel examples and explain how the paths differ.

| TIME 35–45 min | LEVEL FOUNDATION | PARTS Breadboard, LEDs, resistors, jumpers, 9 V source |
| --- | --- | --- |

| 1 PREDICT | 2 BUILD | 3 MEASURE | 4 EXPLAIN |
| --- | --- | --- | --- |

Trace each complete path before connecting the source.

| CIRCUIT | PREDICT: PATHS | OBSERVED BRIGHTNESS / VOLTAGE | EXPLAIN |
| --- | --- | --- | --- |
| Series | ________ | ________________________ | ________________ |
| Parallel | ________ | ________________________ | ________________ |
| Series-parallel | ________ | ________________________ | ________________ |

| DIAGNOSE One open load stops every load in which circuit type? Why? ________________________________________ ________________________________________ | EXPLAIN Why is branch voltage the same in a parallel circuit? ________________________________________ ________________________________________ |
| --- | --- |

| THREE BUILDS VERIFIED ________________________________________ | STUDENT TRACES EVERY CURRENT PATH ________________________________________ |
| --- | --- |

LAB 2

# Tactile Switch Continuity

Use resistance evidence to identify the correct switching pair before placing the switch in a powered circuit.

| TIME 25–35 min | LEVEL FOUNDATION | PARTS Tactile switch, DMM, breadboard, jumpers |
| --- | --- | --- |

| 1 PREDICT | 2 BUILD | 3 MEASURE | 4 EXPLAIN |
| --- | --- | --- | --- |

The two pins on the same side are internally common. Use opposite sides for switching.

| METER CONNECTION | RELEASED | PRESSED | WHAT THIS PROVES |
| --- | --- | --- | --- |
| Across opposite sides | ________ | ________ | ________________________ |
| Across same side | ________ | ________ | ________________________ |

BUILD RULE Place the tactile switch across the breadboard center trench so the two common sides occupy separate terminal strips.

| PREDICT Which pin pair should change from OL to about 0 Ω? ________________________________________ ________________________________________ | DIAGNOSE Why does the wrong pair stay near 0 Ω in both states? ________________________________________ ________________________________________ |
| --- | --- |

| METER SETUP VERIFIED ________________________________________ | CORRECT SWITCHING PAIR IDENTIFIED ________________________________________ |
| --- | --- |

LAB 3

# First LED Circuit

Build a protected LED circuit, prove the complete path, and diagnose it if the LED stays off.

| TIME 30–40 min | LEVEL FOUNDATION | PARTS LED, 470 Ω resistor, breadboard, jumpers, 9 V source, DMM |
| --- | --- | --- |

| 1 PREDICT | 2 BUILD | 3 MEASURE | 4 EXPLAIN |
| --- | --- | --- | --- |

The resistor and LED must be in series. The flat edge / shorter lead normally identifies the cathode.

- With power disconnected, connect the positive and negative rails.

- Install a 470 Ω resistor from B+ to a new terminal-strip node.

- Connect the LED anode to the resistor output and the cathode to a separate node.

- Complete the return path from the cathode node to ground.

- Trace, inspect, receive the pre-power check, then connect the source.

| CHECK POINT | EXPECTED | MEASURED | PASS / FIX |
| --- | --- | --- | --- |
| Battery | About 9 V | ________ V | ________ |
| LED anode to cathode | Color-dependent drop | ________ V | ________ |
| Ground path | Near 0 Ω, power off | ________ Ω | ________ |

IF THE LED STAYS OFF Check source voltage, rail continuity, LED polarity, resistor continuity, and the ground-side connection—in that order.

LAB 4

# LED Forward Voltage

Compare predicted and measured forward voltage for several LED colors.

| TIME 30–40 min | LEVEL MEASUREMENT | PARTS Colored LEDs, 470 Ω resistor, breadboard, 9 V source, DMM |
| --- | --- | --- |

| 1 PREDICT | 2 BUILD | 3 MEASURE | 4 EXPLAIN |
| --- | --- | --- | --- |

Place the voltmeter directly across the illuminated LED while the circuit is powered.

| COLOR | TYPICAL RANGE | PREDICT | MEASURED | DIFFERENCE |
| --- | --- | --- | --- | --- |
| Red | 1.8–2.2 V | _____ V | _____ V | _____ V |
| Yellow | 2.0–2.3 V | _____ V | _____ V | _____ V |
| Green | 2.0–3.2 V | _____ V | _____ V | _____ V |
| Blue | 2.8–3.4 V | _____ V | _____ V | _____ V |
| White | 2.8–3.4 V | _____ V | _____ V | _____ V |

INTERPRET THE RANGE LED chemistry, current, and temperature affect forward voltage. A typical range is a comparison tool, not an exact specification.

| EVIDENCE Which color produced the highest measured drop? ________________________________________ ________________________________________ | EXPLAIN How would a higher LED drop affect resistor voltage in this series circuit? ________________________________________ ________________________________________ |
| --- | --- |

LAB 5

# RGB LED Control

Identify the common lead, control each color independently, and document additive color mixing.

| TIME 40–50 min | LEVEL INTERMEDIATE | PARTS RGB LED, 3 switches, 3 resistors, breadboard, jumpers, 9 V source |
| --- | --- | --- |

| 1 PREDICT | 2 BUILD | 3 MEASURE | 4 EXPLAIN |
| --- | --- | --- | --- |

Each internal LED requires its own current-limiting resistor.

| RED | GREEN | BLUE | PREDICTED COLOR | OBSERVED COLOR |
| --- | --- | --- | --- | --- |
| ON | OFF | OFF | Red | ________ |
| OFF | ON | OFF | Green | ________ |
| OFF | OFF | ON | Blue | ________ |
| ON | ON | OFF | Yellow | ________ |
| ON | OFF | ON | Magenta | ________ |
| OFF | ON | ON | Cyan | ________ |
| ON | ON | ON | Near white | ________ |

VERIFY FIRST Identify common-anode versus common-cathode and the four-lead order before power is applied. Do not assume every RGB LED shares the diagram's pinout.

| DIAGNOSE One color never lights. List two checks that isolate the fault. ________________________________________ ________________________________________ | EXPLAIN Why does every color channel need its own resistor? ________________________________________ ________________________________________ |
| --- | --- |

LAB 6

# Potentiometer LED Dimmer

Relate wiper position, resistance, LED voltage, and brightness.

| TIME 40–50 min | LEVEL INTERMEDIATE | PARTS Potentiometer, LED(s), resistor(s), breadboard, 9 V source, DMM |
| --- | --- | --- |

| 1 PREDICT | 2 BUILD | 3 MEASURE | 4 EXPLAIN |
| --- | --- | --- | --- |

| Single-LED dimmer Changing resistance changes LED current. | Dual-LED variation One LED can brighten while the other dims. |
| --- | --- |

| KNOB POSITION | WIPER RESISTANCE | LED VOLTAGE | BRIGHTNESS |
| --- | --- | --- | --- |
| Minimum | ________ Ω | ________ V | ________ |
| Midpoint | ________ Ω | ________ V | ________ |
| Maximum | ________ Ω | ________ V | ________ |

PROTECT THE CIRCUIT Keep a fixed series resistor in the LED path so a zero-ohm potentiometer position cannot overcurrent the LED. Do not exceed the potentiometer power rating.

| GRAPH IN WORDS Describe the relationship between resistance and brightness. ________________________________________ ________________________________________ | DIAGNOSE The LED never gets bright. Which three measurements would you take? ________________________________________ ________________________________________ |
| --- | --- |

| THREE POSITIONS MEASURED ________________________________________ | STUDENT EXPLAINS THE WIPER ________________________________________ |
| --- | --- |

LAB 7

# Transistor Switch + Automatic Night Light

Use a low-current sensor network to control a separate LED load.

| TIME 45–60 min | LEVEL ADVANCED | PARTS NPN transistor, LDR, resistors, LED, breadboard, 9 V source, DMM |
| --- | --- | --- |

| 1 PREDICT | 2 BUILD | 3 MEASURE | 4 EXPLAIN |
| --- | --- | --- | --- |

| NPN transistor switch A small base signal controls collector-emitter current. | Automatic night-light Covering the LDR changes the transistor input. |
| --- | --- |

| LIGHT CONDITION | LDR RESISTANCE | BASE VOLTAGE | LED STATE |
| --- | --- | --- | --- |
| Bright | ________ Ω | ________ V | ________ |
| Covered / dark | ________ Ω | ________ V | ________ |

PINOUT WARNING Identify emitter, base, and collector using the exact transistor marking. PN2222/2N2222 and BC547 packages may use different lead orders.

| SIGNAL MAP Identify INPUT → CONTROL DEVICE → OUTPUT. ________________________________________ ________________________________________ | DIAGNOSE LED stays on in bright light. Name two likely causes. ________________________________________ ________________________________________ |
| --- | --- |

| LIGHT / DARK OPERATION VERIFIED ________________________________________ | STUDENT EXPLAINS TRANSISTOR SWITCHING ________________________________________ |
| --- | --- |

PRACTICAL ASSESSMENT

# Choose → Design → Build → Prove

Select one challenge, obtain approval, and document enough evidence that another student could reproduce your result.

| Dual pushbutton LEDs Approval: __________ | LDR night-light prototype Approval: __________ |
| --- | --- |
| Dual-transistor prototype Approval: __________ | Zener voltage ladder Approval: __________ |

| SKILL | EVIDENCE REQUIRED | SCORE |
| --- | --- | --- |
| Identify | Names components, terminals, and polarity correctly | __/4 |
| Build | Creates a safe, complete, organized current path | __/4 |
| Measure | Selects meter function and records useful evidence | __/4 |
| Diagnose | Isolates one inserted or naturally occurring fault | __/4 |
| Explain | Connects input, control, output, and result | __/4 |

| PREDICTION What should the circuit do before you build it? ________________________________________ ________________________________________ | DIAGNOSTIC PROOF Record one fault, one measurement, and the fix. ________________________________________ ________________________________________ |
| --- | --- |

| FINAL DEMONSTRATION ________________________________________ | TOTAL SCORE: ______ / 20 ________________________________________ |
| --- | --- |

APPENDIX A

# Kit Component Identification Reference

Use this manufacturer image as a visual starting point; verify the exact markings and values on the parts in your own kit.

Manufacturer identification image included in the original packet.

| PART TO RECHECK | IDENTIFICATION EVIDENCE | INSTRUCTOR |
| --- | --- | --- |
| ________________ | ________________________________________ | ________ |
| ________________ | ________________________________________ | ________ |
| ________________ | ________________________________________ | ________ |

FINAL REMINDER Appearance is only the first clue. Use part markings, polarity indicators, resistance, diode-test behavior, continuity, and the exact pinout to confirm identification.