ELECTRICAL FUNDAMENTALS • STUDENT GUIDE Electrical / Electronic Components Symbols • Purpose • Specifications • Testing • Diagnostic thinking

| STUDENT ________________________________ | INSTRUCTOR ________________________________ |
| --- | --- |
| CLASS / PERIOD ________________________________ | DATE ________________________________ |

## Learning pathway

| 01 IDENTIFY Name the part and symbol | 02 INTERPRET Read ratings and terminals | 03 TEST Choose the correct method | 04 DIAGNOSE Compare evidence to expected |
| --- | --- | --- | --- |

CORE IDEA Do not identify a component by appearance alone. Confirm its terminals, ratings, expected behavior, and test evidence.

## By the end of this guide, you should be able to…

- Recognize common electrical and electronic symbols.

- Explain what each component does in a circuit.

- Interpret the ratings that protect the component and the circuit.

- Select a safe test method and compare the result with an expected value.

- Describe one likely fault and the evidence that would confirm it.

ASSIGNMENT 1

# Component Symbol Identification

Study the symbol map. Label each symbol directly on the figure or use a separate sheet keyed to the checklist on the next page.

Functional categories overlap: an LED is both a semiconductor and a load; a relay is both electromagnetic and switching.

DIRECTIONS Use the most specific symbol name you know. Include terminal labels or normal state when the symbol requires them—for example, relay coil and contacts, N.O. / N.C., anode / cathode, or transistor terminals.

| IDENTIFICATION | REQUIRED EVIDENCE | SCORE |
| --- | --- | --- |
| Symbol name | Correct component or device name | ____ / 37 |
| Terminal / state labels | Polarity, N.O. / N.C., or terminal names where applicable | ____ / 8 |
| Neatness | Labels are readable and point to the correct symbol | ____ / 5 |

| MOST FAMILIAR Which five symbols did you recognize immediately? ________________________________________ ________________________________________ | NEEDS REVIEW Which three symbols are easiest to confuse? ________________________________________ ________________________________________ |
| --- | --- |

ASSIGNMENT 1

# Symbol Checklist

Check each item only after you can locate its symbol and state one fact about it.

| SOURCES + LOADS □ Battery □ Generator □ Bulb / lamp □ Dual-filament bulb □ LED □ Heater element □ Motor □ Servo motor □ Actuator □ Speaker / buzzer | SWITCHING □ SPST N.O. □ SPST N.C. □ SPDT on-on □ SPDT on-off-on □ DPST □ DPDT □ Multi-throw switch □ Thermostatic switch □ Pressure switch □ Motion switch □ Level switch |
| --- | --- |
| RELAYS + MAGNETICS □ 4-pin SPST relay □ 5-pin SPDT relay □ 6-pin DPST relay □ Relay coil / contacts □ Solenoid □ Transformer □ Inductor | RESISTIVE + SEMICONDUCTOR □ Fixed resistor □ Potentiometer □ Thermistor □ Photoresistor □ Diode □ Zener diode □ LED symbol □ Transistor □ Capacitor |

CLASSIFICATION NOTE The active / passive labels in introductory diagrams are helpful but simplified. In real diagnostics, classify the part by what it does in that circuit: source, path, control, protection, energy storage, signal processing, or load.

## Quick symbol rules

- A switch symbol is normally drawn in its unactuated state.

- A diode bar marks the cathode; an LED adds arrows pointing away from the junction.

- A polarized capacitor marks polarity; a resistor may use ANSI zigzag or IEC rectangle style.

- Relay and transformer drawings separate the coil or windings from the controlled / coupled circuit.

- A symbol identifies function—not the physical appearance or pin order of every real part.

| EXPLAIN Why is a symbol not enough to determine a transistor pinout? ________________________________________ ________________________________________ ________________________________________ | APPLY Choose one relay symbol and label coil, common, N.O., and N.C. ________________________________________ ________________________________________ ________________________________________ |
| --- | --- |

FOUNDATIONS

# How to Read Any Component

Use the same four questions for a resistor, relay, sensor, IC, or motor.

| QUESTION | WHAT IT REVEALS | EXAMPLE |
| --- | --- | --- |
| 1. What is its purpose? | The job it performs in this circuit | Limit current, store charge, switch a load |
| 2. What are its terminals? | Connection, polarity, direction, or normal state | A/K, E/B/C, G/D/S, COM/N.O./N.C. |
| 3. What are its ratings? | Safe electrical, thermal, and mechanical limits | Ω, W, V, A, Hz, duty cycle |
| 4. What should it do? | The measurable behavior under known conditions | Change resistance, conduct one way, switch states |

## Safe testing sequence

- Identify the circuit and expected value before touching the meter leads.

- Decide whether the test must be powered or unpowered.

- For resistance, continuity, capacitance, or diode tests: power off and discharge stored energy.

- Isolate the component or account for parallel paths when the reading is affected by the circuit.

- Select the correct meter function and lead sockets before connecting.

- Compare the result with a specification or a known-good circuit—not with a guess.

SAFETY Never measure resistance or continuity on a powered circuit. Discharge capacitors safely. Use current limiting for LEDs and Zener tests. Observe CAT rating and manufacturer procedures on vehicle circuits.

| BEFORE THE TEST What result do you expect and why? ________________________________________ ________________________________________ ________________________________________ | AFTER THE TEST Does the evidence prove the part is good, bad, or only require another test? ________________________________________ ________________________________________ ________________________________________ |
| --- | --- |

COMPONENT FAMILY 1

# Resistive + Sensing Components

| Fixed Resistor PURPOSE Limits current, creates voltage drops, and establishes bias or pull-up / pull-down conditions. KEY SPECS Resistance (Ω), power rating (W), tolerance (%), and temperature coefficient (ppm/°C). TEST / CHECK Power off. Discharge the circuit. Isolate one lead when parallel paths affect the reading; compare with tolerance. AUTOMOTIVE LINK Blower-speed resistors, current limiting, and sensor reference networks. | Potentiometer / Rheostat PURPOSE A potentiometer is a three-terminal adjustable voltage divider; a rheostat uses two terminals as a variable resistor. KEY SPECS Total resistance, taper, tolerance, power rating, rotation angle, and mechanical life. TEST / CHECK End-to-end resistance stays nearly fixed. Wiper-to-end resistance should change smoothly without opens or jumps. AUTOMOTIVE LINK Position, level, and older throttle-position sensing. |
| --- | --- |
| Thermistor PURPOSE Changes resistance with temperature. NTC resistance decreases as temperature rises; PTC resistance increases. KEY SPECS Resistance at 25°C, NTC / PTC type, beta or temperature coefficient, range, and thermal time constant. TEST / CHECK Measure at room temperature, then warm or cool the sensor. The resistance change should be smooth and repeatable. AUTOMOTIVE LINK Coolant, intake-air, evaporator, and battery temperature sensors. | Photoresistor (LDR) PURPOSE Changes resistance with light level; a common LDR decreases in resistance as light increases. KEY SPECS Light resistance, dark resistance, response time, spectral response, voltage, and power limits. TEST / CHECK Compare resistance in bright light and while covered. Look for a clear, repeatable change rather than one exact value. AUTOMOTIVE LINK Ambient-light and automatic-headlamp sensing concepts. |

DIAGNOSTIC CONNECTION A fixed resistor should stay nearly constant. A potentiometer, thermistor, and photoresistor are expected to change—but only in a smooth, predictable relationship to position, temperature, or light.

| COMPARE How is a potentiometer different from a fixed resistor? ________________________________________ ________________________________________ | DIAGNOSE A thermistor reading never changes with temperature. What should you verify next? ________________________________________ ________________________________________ |
| --- | --- |

COMPONENT FAMILY 2

# Energy Storage + Magnetics

| Capacitor PURPOSE Stores electric charge; filters ripple and noise, couples signals, and creates timing behavior. KEY SPECS Capacitance (F), voltage rating, tolerance, ESR, temperature rating, polarity, and leakage current. TEST / CHECK Safely discharge first. Check capacitance and ESR; verify polarity and inspect for swelling or leakage. AUTOMOTIVE LINK Module power filtering, sensor-signal conditioning, and suppression networks. | Inductor PURPOSE Stores energy in a magnetic field and opposes changes in current; used in filters and converters. KEY SPECS Inductance (H), current / saturation rating, DC resistance, self-resonant frequency, and temperature rating. TEST / CHECK Check continuity and DC resistance. Use an LCR meter for inductance; inspect for overheated or shorted windings. AUTOMOTIVE LINK Ignition coils, filters, DC-DC converters, and electromagnetic actuators. |
| --- | --- |
| Transformer PURPOSE Transfers AC energy between windings for voltage conversion or isolation. KEY SPECS Primary / secondary voltage, turns ratio, VA rating, frequency, regulation, and insulation rating. TEST / CHECK Power off: inspect and compare winding resistance. When safely energized, verify AC input/output and turns ratio. AUTOMOTIVE LINK Ignition-coil action, isolated chargers, and power electronics. | Solenoid PURPOSE Uses an energized coil to create linear mechanical motion or operate a valve. KEY SPECS Voltage, current draw, coil resistance, pull force, stroke, and duty cycle. TEST / CHECK Compare coil resistance and current draw to specifications; verify movement without binding. AUTOMOTIVE LINK Starter, transmission, EVAP, fuel, and hydraulic control valves. |

ENERGY REMINDER Capacitors store energy in an electric field; inductors store energy in a magnetic field. Both can affect a circuit after the source or input changes.

| COMPARE How do capacitors and inductors react differently to changing voltage or current? ________________________________________ ________________________________________ | DIAGNOSE A solenoid has correct resistance but will not move. Name two next checks. ________________________________________ ________________________________________ |
| --- | --- |

COMPONENT FAMILY 3

# Diodes + Light-Sensitive Semiconductors

| Rectifier Diode PURPOSE Conducts conventional current mainly in one direction for rectification and protection. KEY SPECS Forward voltage, average / peak forward current, repetitive reverse voltage, leakage, and power dissipation. TEST / CHECK Diode mode should show one forward junction drop and OL in reverse. In-circuit parallel paths may affect the reading. AUTOMOTIVE LINK Alternator rectifiers, flyback suppression, and reverse-polarity protection. | Zener Diode PURPOSE Operates in controlled reverse breakdown to clamp or regulate voltage. KEY SPECS Zener voltage, test current, maximum power, dynamic impedance, tolerance, and leakage. TEST / CHECK Diode mode checks the junction only. Verify Zener voltage with a current-limited reverse-bias test circuit. AUTOMOTIVE LINK Reference-voltage, surge-clamp, and module input-protection circuits. |
| --- | --- |
| LED — Light-Emitting Diode PURPOSE Produces light when forward biased and must be protected from excessive current. KEY SPECS Forward voltage, operating current, luminous intensity, wavelength / color, viewing angle, and power. TEST / CHECK Use diode mode or a current-limited supply. Confirm polarity and calculate R = (Vsource − VLED) / ILED. AUTOMOTIVE LINK Indicators, displays, exterior lighting, and optical sensing. | Photodiode PURPOSE Converts incident light into current; often operated in reverse bias for fast sensing. KEY SPECS Peak wavelength, responsivity, dark current, junction capacitance, and reverse-voltage rating. TEST / CHECK Observe current or voltage change between dark and illuminated conditions using the correct bias circuit. AUTOMOTIVE LINK Optical encoders, rain/light sensors, and communication receivers. |

FORWARD VOLTAGE A silicon diode often measures roughly 0.5–0.8 V in diode mode. LED forward voltage is commonly about 1.8–3.4 V depending on color, chemistry, current, and temperature.

| COMPARE How is normal Zener operation different from a rectifier diode? ________________________________________ ________________________________________ | CALCULATE Write the formula used to select an LED series resistor. ________________________________________ ________________________________________ |
| --- | --- |

COMPONENT FAMILY 4

# Switching + Control Devices

| BJT Transistor — NPN / PNP PURPOSE Uses base current to control collector current; works as a switch or amplifier. KEY SPECS VCE maximum, collector current, power dissipation, gain (hFE), and saturation voltage. TEST / CHECK Use diode mode to check base-emitter and base-collector junctions. Verify pinout and switching under a known load. AUTOMOTIVE LINK Low-side drivers for relays, lamps, solenoids, and module outputs. | MOSFET PURPOSE Uses gate voltage to control drain-source current with very little steady gate current. KEY SPECS VDS, drain current, RDS(on), gate threshold, required drive voltage, power, and thermal limits. TEST / CHECK Check for drain-source shorts and gate leakage; then verify that a controlled gate voltage switches the load. AUTOMOTIVE LINK High-efficiency fan, pump, injector, heater, and lighting control. |
| --- | --- |
| Relay PURPOSE Uses a low-current coil to operate electrically isolated contacts for a higher-current load. KEY SPECS Coil voltage / resistance, contact arrangement, contact current / voltage, switching time, and life cycles. TEST / CHECK Measure coil resistance, identify 30 / 85 / 86 / 87 / 87a when applicable, energize the coil, and verify contact change. AUTOMOTIVE LINK Starter, fuel pump, cooling fan, horn, and lighting circuits. | Manual / Automatic Switch PURPOSE Opens, closes, or redirects a current or signal path. Automatic switches respond to temperature, pressure, motion, or level. KEY SPECS Pole / throw arrangement, normal state, contact rating, actuation point / force, and cycle life. TEST / CHECK Power off: monitor continuity while operating the switch or changing the sensed condition. AUTOMOTIVE LINK Ignition, brake, pressure, thermostatic, level, and position switches. |

PINOUT WARNING Package shape does not guarantee terminal order. Confirm the exact BJT, MOSFET, relay, or switch pinout before applying power.

| SIGNAL PATH Choose one device and identify INPUT → CONTROL → OUTPUT. ________________________________________ ________________________________________ | DIAGNOSE A relay clicks but the load stays off. Which circuit side do you test next? ________________________________________ ________________________________________ |
| --- | --- |

COMPONENT FAMILY 5

# Power, Protection + Loads

| Battery / Generator / Power Source PURPOSE Supplies electrical energy. A battery stores chemical energy; a generator converts mechanical energy to electrical energy. KEY SPECS Voltage, current capability, capacity, internal resistance, polarity, and charging / output limits. TEST / CHECK Measure open-circuit voltage, then perform an appropriate loaded or output test while observing polarity and safety limits. AUTOMOTIVE LINK Starting, charging, and low-voltage vehicle power distribution. | Fuse PURPOSE Opens the circuit when current exceeds a safe value, protecting wiring and equipment. KEY SPECS Current and voltage ratings, time-current characteristic, package type, and interrupting capacity. TEST / CHECK Power off: continuity test. Powered: verify voltage on both test points. Replace only with the specified type and rating. AUTOMOTIVE LINK Branch-circuit and module protection throughout the vehicle. |
| --- | --- |
| Varistor (MOV) PURPOSE Changes resistance sharply at high voltage to clamp transient spikes. KEY SPECS Continuous working voltage, varistor / clamping voltage, joule rating, peak current, and response time. TEST / CHECK Inspect for cracking or heat damage. An ohmmeter can find a short, but cannot fully verify clamping performance. AUTOMOTIVE LINK Surge protection in chargers, power supplies, and electronic modules. | Load Devices PURPOSE Convert electrical energy into light, heat, sound, rotation, or linear motion. KEY SPECS Rated voltage, current / power, resistance or impedance, duty cycle, efficiency, and mechanical output. TEST / CHECK Check source voltage and ground under load, then measure current draw and compare the device's output with specification. AUTOMOTIVE LINK Bulbs, heaters, motors, actuators, speakers / buzzers, and solenoids. |

LOADED-CIRCUIT PRINCIPLE A device can show source voltage with no load and still fail under current. Voltage-drop and current measurements reveal conditions that an open-circuit voltage check can miss.

| COMPARE What is the difference between a source and a load? ________________________________________ ________________________________________ | DIAGNOSE A replacement fuse opens again. What must be found before another fuse is installed? ________________________________________ ________________________________________ |
| --- | --- |

COMPONENT FAMILY 6

# Control, Timing + Actuation

| Integrated Circuit (IC) / Control Module PURPOSE Combines many electronic functions for logic, amplification, sensing, communication, or control. KEY SPECS Supply voltage, current consumption, pinout, logic thresholds, timing, temperature range, and package. TEST / CHECK Verify power, ground, reference voltages, inputs, and outputs. Use the correct pinout and avoid probing shorts. AUTOMOTIVE LINK ECMs, TCMs, body modules, sensor interfaces, and communication networks. | Crystal / Oscillator PURPOSE Provides a stable timing reference for digital circuits and communication systems. KEY SPECS Frequency, load capacitance, tolerance / stability (ppm), drive level, temperature range, and aging rate. TEST / CHECK Use an oscilloscope or frequency counter with a low-capacitance probe; confirm supply and load capacitors first. AUTOMOTIVE LINK Module clocks, processors, communication transceivers, and timers. |
| --- | --- |
| Voltage Regulator PURPOSE Maintains a stable output voltage as input voltage or load changes. KEY SPECS Output voltage, input range, maximum current, dropout voltage, accuracy, thermal limits, and quiescent current. TEST / CHECK Measure input, output, ground voltage drop, and load current. Check for overheating, ripple, or loss of regulation. AUTOMOTIVE LINK 5 V sensor references, module power rails, USB supplies, and charging electronics. | Motor / Servo / Actuator PURPOSE Converts electrical energy into controlled rotary or linear motion; a servo includes position-control behavior. KEY SPECS Voltage, stall / running current, speed, torque or force, duty cycle, travel, and feedback type. TEST / CHECK Verify source and ground under load, compare current draw, check mechanical freedom, and test feedback where fitted. AUTOMOTIVE LINK Windows, seats, HVAC doors, throttle control, pumps, and steering systems. |

MODULE TESTING Do not condemn an IC or control module until its powers, grounds, references, inputs, network conditions, and outputs have been verified under the conditions that produce the fault.

| CONTROL LOGIC Identify one input and one output for a control module. ________________________________________ ________________________________________ | DIAGNOSE An actuator has correct voltage but moves slowly. Which two loaded tests help next? ________________________________________ ________________________________________ |
| --- | --- |

TESTING REFERENCE

# Choose the Right Meter Function

The meter setting must match the electrical quantity and the circuit condition.

| METER / TOOL MODE | CIRCUIT CONDITION | USE IT FOR | COMMON LIMIT |
| --- | --- | --- | --- |
| Resistance (Ω) | Power off; energy discharged | Resistors, coils, thermistors, potentiometers | Parallel paths can lower the reading |
| Continuity | Power off; energy discharged | Wires, fuses, switches, obvious opens | A beep does not prove low voltage drop under load |
| Diode mode | Power off; isolate as needed | Diodes, LEDs, BJT junctions | Does not verify high-voltage or high-current performance |
| Capacitance | Capacitor safely discharged | Capacitance value | May miss high ESR or leakage |
| DC voltage | Powered | Source, reference, signal, and voltage drop | Voltage present does not prove current flow |
| AC voltage / frequency | Powered and safely rated | Transformer output, alternator ripple, oscillator / sensor frequency | Meter bandwidth may limit accuracy |
| Current | Powered; meter inserted correctly | Load or circuit current draw | Wrong socket or parallel connection can blow the meter fuse |
| Oscilloscope | Powered | Waveform shape, switching, ripple, timing, intermittent signals | Probe grounding and sample rate matter |

RESISTANCE IS NOT THE WHOLE STORY A coil may have normal resistance but shorted turns. A capacitor may have correct capacitance but excessive ESR. A switch may pass continuity but develop excessive voltage drop under load.

| SELECT THE TOOL An LED does not light. Which three measurements come first? ________________________________________ ________________________________________ ________________________________________ | EXPLAIN THE LIMIT Why can continuity pass while a loaded circuit still fails? ________________________________________ ________________________________________ ________________________________________ |
| --- | --- |

DIAGNOSTIC REFERENCE

# Expected Behavior → Possible Fault → Next Test

| COMPONENT | EXPECTED BEHAVIOR | SUSPICIOUS RESULT | NEXT TEST |
| --- | --- | --- | --- |
| Fixed resistor | Within tolerance | Open, shorted, or far out of tolerance | Isolate one lead and remeasure |
| Potentiometer | Smooth wiper change | Open spots or sudden jumps | Graph resistance while rotating |
| Thermistor | Smooth temperature response | No change, open, or implausible value | Compare to temperature/resistance chart |
| Capacitor | Rated capacitance and acceptable ESR | Low capacitance, high ESR, leakage | ESR / leakage test |
| Diode / LED | Forward conduction; reverse blocking | Shorted both ways or open both ways | Isolate and retest; controlled load test |
| Relay | Coil energizes; contacts change state | No click, wrong contact state, high drop | Coil voltage/current and contact voltage drop |
| BJT / MOSFET | Input controls output current | Shorted output or no switching | Verify input drive, pinout, load, and ground |
| Motor / solenoid | Normal current and mechanical output | High current, low force/speed, no movement | Voltage drop, current draw, and mechanical check |

## Diagnostic questions

- Is the component receiving the correct input under the failure condition?

- Is the ground or return path capable of carrying current?

- Is a control device being commanded, and does its output respond?

- Is the load mechanically free and electrically within its rated current?

- Could the circuit or another parallel path be changing the measurement?

| FAULT ISOLATION Choose one row and describe how you would divide the circuit. ________________________________________ ________________________________________ ________________________________________ | EVIDENCE What reading would convince you to replace the component? ________________________________________ ________________________________________ ________________________________________ |
| --- | --- |

HANDS-ON PRACTICE

# Six Component Test Stations

Rotate through the stations. Predict first, test safely, and record evidence—not just pass / fail.

| STATION | TASK | PREDICT | MEASURED / OBSERVED | CHECK |
| --- | --- | --- | --- | --- |
| 1 — Resistor | Decode bands; measure Ω; calculate tolerance | ________ | ______________________ | ____ |
| 2 — Potentiometer | Identify ends / wiper; test smooth rotation | ________ | ______________________ | ____ |
| 3 — Thermistor / LDR | Change the input condition; record direction | ________ | ______________________ | ____ |
| 4 — Diode / LED | Identify polarity; record forward drop | ________ | ______________________ | ____ |
| 5 — Relay | Identify coil / contacts; energize and verify state | ________ | ______________________ | ____ |
| 6 — Load device | Measure source, ground drop, and current draw | ________ | ______________________ | ____ |

## One complete diagnostic explanation

| PROMPT | STUDENT RESPONSE |
| --- | --- |
| Component selected | ____________________________________________________________ |
| Expected behavior | ____________________________________________________________ |
| Test method and safety condition | ____________________________________________________________ |
| Measured evidence | ____________________________________________________________ |
| Conclusion and next step | ____________________________________________________________ |

| INSTRUCTOR VERIFICATION ________________________________________ | STUDENT EXPLAINS WHY THE TEST PROVES THE CONCLUSION ________________________________________ |
| --- | --- |

ASSESSMENT

# Identify → Interpret → Test → Diagnose

Use an instructor-selected component or circuit. Complete each section before the final demonstration.

| SKILL | PERFORMANCE EVIDENCE | SCORE |
| --- | --- | --- |
| Identify | Names the component, symbol, terminals, polarity, or normal state | ____ / 5 |
| Interpret | Explains purpose and correctly reads the important specifications | ____ / 5 |
| Plan | Selects a safe, useful test and predicts the expected result | ____ / 5 |
| Measure | Uses the meter correctly and records clear evidence with units | ____ / 5 |
| Diagnose | Compares evidence to expected behavior and chooses a justified next step | ____ / 5 |
| Explain | Communicates the current path or signal path in plain language | ____ / 5 |

| COMPONENT / CIRCUIT Name and symbol: ________________________________________ ________________________________________ ________________________________________ | PURPOSE What job does it perform here? ________________________________________ ________________________________________ ________________________________________ |
| --- | --- |

| SPECIFICATIONS Which rating matters most in this circuit? ________________________________________ ________________________________________ ________________________________________ | TEST PLAN Meter mode, circuit condition, connection points, and expected result: ________________________________________ ________________________________________ ________________________________________ |
| --- | --- |

| EVIDENCE Record the measurement with units: ________________________________________ ________________________________________ ________________________________________ | CONCLUSION Good, faulty, or more testing required—and why? ________________________________________ ________________________________________ ________________________________________ |
| --- | --- |

FINAL STANDARD A correct diagnosis connects the component's job, its ratings, the circuit condition, and a measurement. Replacing a part without that chain of evidence is a guess.

| FINAL DEMONSTRATION ________________________________________ | TOTAL SCORE: ______ / 30 ________________________________________ |
| --- | --- |