AUTOMOTIVE TECHNOLOGY  •  ELECTRICAL 2

MAZDA CX-9 LAB TASK SHEET

Locate Power-Distribution Boxes

| WEEK | TASK | VEHICLE | VIN |
| --- | --- | --- | --- |
| 1 | 1 | 2011 Mazda CX-9 | JM3TB2BA8B0308293 |

| Student |  | Date |  |
| --- | --- | --- | --- |
| Team / Bay |  | RO # |  |

OBJECTIVE

Locate the vehicle's main power-distribution points and use service information to identify assigned fuses and relays.

TOOLS, RESOURCES, AND SAFETY

| Tools & Resources Mazda service information, component-location views, flashlight, trim tools, PPE SI diagram/procedure #: ______________________________ | Safety & Setup Wear eye protection, secure the vehicle, and maintain ignition-key control. Do not force covers, trim, or connectors; restore all retainers and protective covers. Use current Mazda service information and obtain instructor approval before energizing or bypassing a circuit. |
| --- | --- |

PROCEDURE CHECKLIST

| STEP | PROCEDURE | CHECK |
| --- | --- | --- |
| 1 | Verify the vehicle, VIN, ignition status, parking brake, and key-control procedure. |  |
| 2 | Retrieve the power-distribution diagram and component-location views for this CX-9. |  |
| 3 | Locate the underhood fuse/relay box and confirm its orientation using the cover legend. |  |
| 4 | Locate the interior fuse/junction panel without damaging trim or wiring. |  |
| 5 | Identify the main fuse and assigned fuses/relays for starting, charging, and exterior lighting. |  |
| 6 | Restore all covers and record the component locations and circuit assignments below. |  |

MEASUREMENTS AND EVIDENCE

| ITEM | EXPECTED / SI SPEC | ACTUAL RESULT | P / F |
| --- | --- | --- | --- |
| Underhood box location/ID | Matches service information |  |  |
| Interior panel location/ID | Matches service information |  |  |
| Assigned fuse/relay and circuit | Correct rating and function |  |  |

ANALYSIS AND INSTRUCTOR VERIFICATION

| What did the test evidence prove? |  |
| --- | --- |
| What is the next test or action if the result is outside specification? |  |

Instructor: __________________________    Status:  PASS  /  REVISE    Date: ______________

AUTOMOTIVE TECHNOLOGY  •  ELECTRICAL 2

MAZDA CX-9 LAB TASK SHEET

Identify Modules, Grounds, and Connectors

| WEEK | TASK | VEHICLE | VIN |
| --- | --- | --- | --- |
| 1 | 2 | 2011 Mazda CX-9 | JM3TB2BA8B0308293 |

| Student |  | Date |  |
| --- | --- | --- | --- |
| Team / Bay |  | RO # |  |

OBJECTIVE

Use location and connector views to physically identify electronic modules, ground points, and harness connectors.

TOOLS, RESOURCES, AND SAFETY

| Tools & Resources Mazda service information, component-location views, flashlight, trim tools, PPE SI diagram/procedure #: ______________________________ | Safety & Setup Wear eye protection, secure the vehicle, and maintain ignition-key control. Do not force covers, trim, or connectors; restore all retainers and protective covers. Use current Mazda service information and obtain instructor approval before energizing or bypassing a circuit. |
| --- | --- |

PROCEDURE CHECKLIST

| STEP | PROCEDURE | CHECK |
| --- | --- | --- |
| 1 | Retrieve the module, ground-distribution, and connector-location information for the vehicle. |  |
| 2 | Select three modules that communicate with or control the assigned electrical system. |  |
| 3 | Locate each module and identify its primary function without disconnecting it. |  |
| 4 | Locate two assigned ground points and inspect for looseness, corrosion, or damage. |  |
| 5 | Locate two harness connectors and match the connector shape, lock, and pin numbering to the view. |  |
| 6 | Document all IDs and restore any access panels or trim removed during the inspection. |  |

MEASUREMENTS AND EVIDENCE

| ITEM | EXPECTED / SI SPEC | ACTUAL RESULT | P / F |
| --- | --- | --- | --- |
| Three module IDs/locations | Correct module and function |  |  |
| Two ground IDs/conditions | Secure, clean, no damage |  |  |
| Two connector IDs/locations | Matches connector view |  |  |

ANALYSIS AND INSTRUCTOR VERIFICATION

| What did the test evidence prove? |  |
| --- | --- |
| What is the next test or action if the result is outside specification? |  |

Instructor: __________________________    Status:  PASS  /  REVISE    Date: ______________

AUTOMOTIVE TECHNOLOGY  •  ELECTRICAL 2

MAZDA CX-9 LAB TASK SHEET

Obtain Diagrams and Connector Views

| WEEK | TASK | VEHICLE | VIN |
| --- | --- | --- | --- |
| 1 | 3 | 2011 Mazda CX-9 | JM3TB2BA8B0308293 |

| Student |  | Date |  |
| --- | --- | --- | --- |
| Team / Bay |  | RO # |  |

OBJECTIVE

Retrieve the correct diagram set and trace power, control, load, and ground paths for an assigned circuit.

TOOLS, RESOURCES, AND SAFETY

| Tools & Resources Mazda service information, wiring diagram, connector views, highlighters or colored pencils SI diagram/procedure #: ______________________________ | Safety & Setup Wear eye protection, secure the vehicle, and maintain ignition-key control. Verify the exact engine, option, and connector configuration before testing. Use current Mazda service information and obtain instructor approval before energizing or bypassing a circuit. |
| --- | --- |

PROCEDURE CHECKLIST

| STEP | PROCEDURE | CHECK |
| --- | --- | --- |
| 1 | Confirm the VIN, engine, drivetrain, and equipment options used to select service information. |  |
| 2 | Open the complete wiring diagram for the instructor-assigned electrical circuit. |  |
| 3 | Mark the power source, fuse, relay or module control, load, and ground path. |  |
| 4 | Open the connector view for each test point and verify pin numbering and terminal side. |  |
| 5 | List the connector IDs, pin numbers, wire colors, and expected circuit functions. |  |
| 6 | Explain the circuit's current path to the instructor before connecting any test equipment. |  |

MEASUREMENTS AND EVIDENCE

| ITEM | EXPECTED / SI SPEC | ACTUAL RESULT | P / F |
| --- | --- | --- | --- |
| Power source and protection | Correct fuse/feed identified |  |  |
| Control, load, and ground | Complete current path traced |  |  |
| Connector/pin test points | Correct view and terminal side |  |  |

ANALYSIS AND INSTRUCTOR VERIFICATION

| What did the test evidence prove? |  |
| --- | --- |
| What is the next test or action if the result is outside specification? |  |

Instructor: __________________________    Status:  PASS  /  REVISE    Date: ______________

AUTOMOTIVE TECHNOLOGY  •  ELECTRICAL 2

MAZDA CX-9 LAB TASK SHEET

Verify Battery State of Charge

| WEEK | TASK | VEHICLE | VIN |
| --- | --- | --- | --- |
| 1 | 4 | 2011 Mazda CX-9 | JM3TB2BA8B0308293 |

| Student |  | Date |  |
| --- | --- | --- | --- |
| Team / Bay |  | RO # |  |

OBJECTIVE

Determine whether the battery is sufficiently charged to support accurate Electrical 2 testing.

TOOLS, RESOURCES, AND SAFETY

| Tools & Resources Mazda service information, digital multimeter, battery conductance tester, charger, PPE SI diagram/procedure #: ______________________________ | Safety & Setup Wear eye protection, secure the vehicle, and maintain ignition-key control. Remove jewelry, avoid battery sparks, and observe correct tester/charger polarity. Use current Mazda service information and obtain instructor approval before energizing or bypassing a circuit. |
| --- | --- |

PROCEDURE CHECKLIST

| STEP | PROCEDURE | CHECK |
| --- | --- | --- |
| 1 | Inspect the battery case, hold-down, cables, terminals, and rating label before testing. |  |
| 2 | Remove surface charge when required and measure battery open-circuit voltage. |  |
| 3 | Enter the correct battery type and rating into the conductance tester. |  |
| 4 | Perform the conductance test and record measured CCA and tester decision. |  |
| 5 | Charge and retest the battery if the state of charge is below the testing requirement. |  |
| 6 | Confirm terminal tightness and document whether electrical diagnosis may proceed. |  |

MEASUREMENTS AND EVIDENCE

| ITEM | EXPECTED / SI SPEC | ACTUAL RESULT | P / F |
| --- | --- | --- | --- |
| Battery rating | Rating label / service information |  |  |
| Open-circuit voltage | Meets testing requirement |  |  |
| Measured CCA / decision | Tester result recorded |  |  |

ANALYSIS AND INSTRUCTOR VERIFICATION

| What did the test evidence prove? |  |
| --- | --- |
| What is the next test or action if the result is outside specification? |  |

Instructor: __________________________    Status:  PASS  /  REVISE    Date: ______________

AUTOMOTIVE TECHNOLOGY  •  ELECTRICAL 2

MAZDA CX-9 LAB TASK SHEET

Measure Source Voltage and Ground Integrity

| WEEK | TASK | VEHICLE | VIN |
| --- | --- | --- | --- |
| 2 | 1 | 2011 Mazda CX-9 | JM3TB2BA8B0308293 |

| Student |  | Date |  |
| --- | --- | --- | --- |
| Team / Bay |  | RO # |  |

OBJECTIVE

Verify that an assigned load receives adequate source voltage and has a usable ground path under operating conditions.

TOOLS, RESOURCES, AND SAFETY

| Tools & Resources Mazda service information, CAT-rated DMM, backprobe pins, test leads, fused jumper lead SI diagram/procedure #: ______________________________ | Safety & Setup Wear eye protection, secure the vehicle, and maintain ignition-key control. Never use an ohmmeter on a powered circuit; use approved backprobe methods. Use current Mazda service information and obtain instructor approval before energizing or bypassing a circuit. |
| --- | --- |

PROCEDURE CHECKLIST

| STEP | PROCEDURE | CHECK |
| --- | --- | --- |
| 1 | Select an assigned electrical load and identify its feed and ground pins on the diagram. |  |
| 2 | Verify battery state of charge and record source voltage before operating the load. |  |
| 3 | Turn the load on and measure voltage at its power-feed terminal using an approved backprobe. |  |
| 4 | Measure voltage between the load ground terminal and battery negative while the load operates. |  |
| 5 | Compare the readings with service information and calculate any voltage lost in the circuit. |  |
| 6 | State whether the feed, ground, or load requires additional testing. |  |

MEASUREMENTS AND EVIDENCE

| ITEM | EXPECTED / SI SPEC | ACTUAL RESULT | P / F |
| --- | --- | --- | --- |
| Battery/source voltage | Stable under assigned load |  |  |
| Voltage available at load | Near source voltage |  |  |
| Loaded ground voltage | Within service limit |  |  |

ANALYSIS AND INSTRUCTOR VERIFICATION

| What did the test evidence prove? |  |
| --- | --- |
| What is the next test or action if the result is outside specification? |  |

Instructor: __________________________    Status:  PASS  /  REVISE    Date: ______________

AUTOMOTIVE TECHNOLOGY  •  ELECTRICAL 2

MAZDA CX-9 LAB TASK SHEET

Backprobe Loaded-Circuit Voltage

| WEEK | TASK | VEHICLE | VIN |
| --- | --- | --- | --- |
| 2 | 2 | 2011 Mazda CX-9 | JM3TB2BA8B0308293 |

| Student |  | Date |  |
| --- | --- | --- | --- |
| Team / Bay |  | RO # |  |

OBJECTIVE

Measure circuit voltage without disconnecting the load and without damaging the connector or terminal.

TOOLS, RESOURCES, AND SAFETY

| Tools & Resources Mazda service information, CAT-rated DMM, backprobe pins, test leads, fused jumper lead SI diagram/procedure #: ______________________________ | Safety & Setup Wear eye protection, secure the vehicle, and maintain ignition-key control. Never use an ohmmeter on a powered circuit; use approved backprobe methods. Use current Mazda service information and obtain instructor approval before energizing or bypassing a circuit. |
| --- | --- |

PROCEDURE CHECKLIST

| STEP | PROCEDURE | CHECK |
| --- | --- | --- |
| 1 | Use the wiring and connector views to identify the correct connector, cavity, and terminal side. |  |
| 2 | Inspect the connector and select a backprobe that will not spread or damage the terminal. |  |
| 3 | Connect the meter reference lead to the specified reference point. |  |
| 4 | Operate the circuit and record voltage at the feed, control, and ground-side test points. |  |
| 5 | Compare connected readings with the expected circuit state on the wiring diagram. |  |
| 6 | Remove the probes, inspect the terminal seals, and confirm normal circuit operation. |  |

MEASUREMENTS AND EVIDENCE

| ITEM | EXPECTED / SI SPEC | ACTUAL RESULT | P / F |
| --- | --- | --- | --- |
| Feed-pin voltage | Expected for circuit state |  |  |
| Control/signal voltage | Matches operating command |  |  |
| Ground-side reading | Within service limit |  |  |

ANALYSIS AND INSTRUCTOR VERIFICATION

| What did the test evidence prove? |  |
| --- | --- |
| What is the next test or action if the result is outside specification? |  |

Instructor: __________________________    Status:  PASS  /  REVISE    Date: ______________

AUTOMOTIVE TECHNOLOGY  •  ELECTRICAL 2

MAZDA CX-9 LAB TASK SHEET

Perform Power-Side Voltage-Drop Test

| WEEK | TASK | VEHICLE | VIN |
| --- | --- | --- | --- |
| 2 | 3 | 2011 Mazda CX-9 | JM3TB2BA8B0308293 |

| Student |  | Date |  |
| --- | --- | --- | --- |
| Team / Bay |  | RO # |  |

OBJECTIVE

Measure unwanted resistance between battery positive and an operating electrical load.

TOOLS, RESOURCES, AND SAFETY

| Tools & Resources Mazda service information, CAT-rated DMM, backprobe pins, test leads, fused jumper lead SI diagram/procedure #: ______________________________ | Safety & Setup Wear eye protection, secure the vehicle, and maintain ignition-key control. Never use an ohmmeter on a powered circuit; use approved backprobe methods. Use current Mazda service information and obtain instructor approval before energizing or bypassing a circuit. |
| --- | --- |

PROCEDURE CHECKLIST

| STEP | PROCEDURE | CHECK |
| --- | --- | --- |
| 1 | Identify the complete power path from battery positive to the assigned load. |  |
| 2 | Verify that the circuit is operating and carrying normal current before measuring drop. |  |
| 3 | Connect the meter across the entire power side and record total voltage drop. |  |
| 4 | If drop is excessive, move one lead along the circuit to divide the path into sections. |  |
| 5 | Identify the wire, connection, fuse, relay contact, or terminal with the largest drop. |  |
| 6 | Compare with service limits and document the next repair or verification step. |  |

MEASUREMENTS AND EVIDENCE

| ITEM | EXPECTED / SI SPEC | ACTUAL RESULT | P / F |
| --- | --- | --- | --- |
| Total power-side drop | Within service limit |  |  |
| Highest section drop | Fault section isolated |  |  |
| Source voltage during test | Stable under load |  |  |

ANALYSIS AND INSTRUCTOR VERIFICATION

| What did the test evidence prove? |  |
| --- | --- |
| What is the next test or action if the result is outside specification? |  |

Instructor: __________________________    Status:  PASS  /  REVISE    Date: ______________

AUTOMOTIVE TECHNOLOGY  •  ELECTRICAL 2

MAZDA CX-9 LAB TASK SHEET

Perform Ground-Side Voltage-Drop Test

| WEEK | TASK | VEHICLE | VIN |
| --- | --- | --- | --- |
| 2 | 4 | 2011 Mazda CX-9 | JM3TB2BA8B0308293 |

| Student |  | Date |  |
| --- | --- | --- | --- |
| Team / Bay |  | RO # |  |

OBJECTIVE

Measure unwanted resistance between an operating load's ground terminal and battery negative.

TOOLS, RESOURCES, AND SAFETY

| Tools & Resources Mazda service information, CAT-rated DMM, backprobe pins, test leads, fused jumper lead SI diagram/procedure #: ______________________________ | Safety & Setup Wear eye protection, secure the vehicle, and maintain ignition-key control. Never use an ohmmeter on a powered circuit; use approved backprobe methods. Use current Mazda service information and obtain instructor approval before energizing or bypassing a circuit. |
| --- | --- |

PROCEDURE CHECKLIST

| STEP | PROCEDURE | CHECK |
| --- | --- | --- |
| 1 | Use the diagram to trace the complete ground path from the load to battery negative. |  |
| 2 | Operate the assigned load so the ground circuit carries normal current. |  |
| 3 | Connect the meter from the load ground terminal to battery negative and record total drop. |  |
| 4 | If excessive, test across each connector, splice, ground eyelet, and cable section. |  |
| 5 | Inspect the highest-drop connection for looseness, corrosion, damage, or poor contact. |  |
| 6 | Compare with service limits and identify the correct repair or next test. |  |

MEASUREMENTS AND EVIDENCE

| ITEM | EXPECTED / SI SPEC | ACTUAL RESULT | P / F |
| --- | --- | --- | --- |
| Total ground-side drop | Within service limit |  |  |
| Highest section drop | Fault section isolated |  |  |
| Load operating voltage | Correct during test |  |  |

ANALYSIS AND INSTRUCTOR VERIFICATION

| What did the test evidence prove? |  |
| --- | --- |
| What is the next test or action if the result is outside specification? |  |

Instructor: __________________________    Status:  PASS  /  REVISE    Date: ______________

AUTOMOTIVE TECHNOLOGY  •  ELECTRICAL 2

MAZDA CX-9 LAB TASK SHEET

Test Fuses Using Live Voltage

| WEEK | TASK | VEHICLE | VIN |
| --- | --- | --- | --- |
| 3 | 1 | 2011 Mazda CX-9 | JM3TB2BA8B0308293 |

| Student |  | Date |  |
| --- | --- | --- | --- |
| Team / Bay |  | RO # |  |

OBJECTIVE

Determine fuse condition and power availability by testing both fuse access points with the circuit energized.

TOOLS, RESOURCES, AND SAFETY

| Tools & Resources Mazda service information, DMM, fuse puller, test light approved for the circuit SI diagram/procedure #: ______________________________ | Safety & Setup Wear eye protection, secure the vehicle, and maintain ignition-key control. Use a circuit-safe test light and never replace a fuse with a higher rating. Use current Mazda service information and obtain instructor approval before energizing or bypassing a circuit. |
| --- | --- |

PROCEDURE CHECKLIST

| STEP | PROCEDURE | CHECK |
| --- | --- | --- |
| 1 | Use the diagram and fuse-box legend to identify the assigned fuse and its rating. |  |
| 2 | Determine the ignition switch and circuit operating state needed to energize the fuse. |  |
| 3 | Reference the meter or test light to a verified ground. |  |
| 4 | Test both exposed fuse terminals without removing the fuse and record each reading. |  |
| 5 | Classify the result as good, open, or not powered, then explain the evidence. |  |
| 6 | If the fuse is open, obtain instructor approval before replacing it or testing for a short. |  |

MEASUREMENTS AND EVIDENCE

| ITEM | EXPECTED / SI SPEC | ACTUAL RESULT | P / F |
| --- | --- | --- | --- |
| Fuse ID and rating | Matches diagram/legend |  |  |
| Test point A voltage | Expected circuit voltage |  |  |
| Test point B voltage / result | Same as A when fuse is good |  |  |

ANALYSIS AND INSTRUCTOR VERIFICATION

| What did the test evidence prove? |  |
| --- | --- |
| What is the next test or action if the result is outside specification? |  |

Instructor: __________________________    Status:  PASS  /  REVISE    Date: ______________

AUTOMOTIVE TECHNOLOGY  •  ELECTRICAL 2

MAZDA CX-9 LAB TASK SHEET

Test Relay Control and Load Circuits

| WEEK | TASK | VEHICLE | VIN |
| --- | --- | --- | --- |
| 3 | 2 | 2011 Mazda CX-9 | JM3TB2BA8B0308293 |

| Student |  | Date |  |
| --- | --- | --- | --- |
| Team / Bay |  | RO # |  |

OBJECTIVE

Verify relay coil control, switched power, and load-circuit operation using the relay terminal diagram.

TOOLS, RESOURCES, AND SAFETY

| Tools & Resources Mazda service information, DMM, fused jumper lead, relay terminal diagram, backprobe pins SI diagram/procedure #: ______________________________ | Safety & Setup Wear eye protection, secure the vehicle, and maintain ignition-key control. Use fused jumpers only with instructor approval; never bypass an unknown control circuit. Use current Mazda service information and obtain instructor approval before energizing or bypassing a circuit. |
| --- | --- |

PROCEDURE CHECKLIST

| STEP | PROCEDURE | CHECK |
| --- | --- | --- |
| 1 | Identify relay coil and contact terminals from the service-information diagram. |  |
| 2 | Verify constant or switched power at the specified relay terminals. |  |
| 3 | Command or operate the circuit and verify the relay coil control and ground path. |  |
| 4 | Measure relay output voltage and voltage drop across the closed contacts. |  |
| 5 | Use a fused jumper only if the service procedure and instructor specifically approve it. |  |
| 6 | Determine whether the fault is in the command, relay, power feed, or downstream load circuit. |  |

MEASUREMENTS AND EVIDENCE

| ITEM | EXPECTED / SI SPEC | ACTUAL RESULT | P / F |
| --- | --- | --- | --- |
| Coil control / ground | Changes with command |  |  |
| Contact input / output | Correct source and switched voltage |  |  |
| Closed-contact voltage drop | Within service limit |  |  |

ANALYSIS AND INSTRUCTOR VERIFICATION

| What did the test evidence prove? |  |
| --- | --- |
| What is the next test or action if the result is outside specification? |  |

Instructor: __________________________    Status:  PASS  /  REVISE    Date: ______________

AUTOMOTIVE TECHNOLOGY  •  ELECTRICAL 2

MAZDA CX-9 LAB TASK SHEET

Trace an Open Circuit from a Diagram

| WEEK | TASK | VEHICLE | VIN |
| --- | --- | --- | --- |
| 3 | 3 | 2011 Mazda CX-9 | JM3TB2BA8B0308293 |

| Student |  | Date |  |
| --- | --- | --- | --- |
| Team / Bay |  | RO # |  |

OBJECTIVE

Use a wiring diagram and voltage checks to locate an instructor-assigned open circuit efficiently.

TOOLS, RESOURCES, AND SAFETY

| Tools & Resources Mazda service information, wiring diagram, DMM, backprobe pins, approved load or test light SI diagram/procedure #: ______________________________ | Safety & Setup Wear eye protection, secure the vehicle, and maintain ignition-key control. Use instructor-installed faults only; do not pierce insulation unless specifically approved. Use current Mazda service information and obtain instructor approval before energizing or bypassing a circuit. |
| --- | --- |

PROCEDURE CHECKLIST

| STEP | PROCEDURE | CHECK |
| --- | --- | --- |
| 1 | Verify and document the circuit symptom before disturbing connectors or components. |  |
| 2 | Trace the current path and choose test points that divide the circuit into large sections. |  |
| 3 | Operate the circuit and test from the source toward the load until voltage is lost. |  |
| 4 | Use connector views to isolate the open wire, terminal, splice, switch, or component. |  |
| 5 | Show the instructor the root-cause test evidence before restoring the circuit. |  |
| 6 | Restore the instructor-installed fault and confirm full circuit operation. |  |

MEASUREMENTS AND EVIDENCE

| ITEM | EXPECTED / SI SPEC | ACTUAL RESULT | P / F |
| --- | --- | --- | --- |
| Last point with correct voltage | Test point recorded |  |  |
| First point without voltage | Test point recorded |  |  |
| Open-circuit location | Root cause proven |  |  |

ANALYSIS AND INSTRUCTOR VERIFICATION

| What did the test evidence prove? |  |
| --- | --- |
| What is the next test or action if the result is outside specification? |  |

Instructor: __________________________    Status:  PASS  /  REVISE    Date: ______________

AUTOMOTIVE TECHNOLOGY  •  ELECTRICAL 2

MAZDA CX-9 LAB TASK SHEET

Locate a High-Resistance Fault

| WEEK | TASK | VEHICLE | VIN |
| --- | --- | --- | --- |
| 3 | 4 | 2011 Mazda CX-9 | JM3TB2BA8B0308293 |

| Student |  | Date |  |
| --- | --- | --- | --- |
| Team / Bay |  | RO # |  |

OBJECTIVE

Use loaded voltage-drop testing to find excessive resistance in an instructor-assigned circuit.

TOOLS, RESOURCES, AND SAFETY

| Tools & Resources Mazda service information, wiring diagram, DMM, backprobe pins, approved load or test light SI diagram/procedure #: ______________________________ | Safety & Setup Wear eye protection, secure the vehicle, and maintain ignition-key control. Use instructor-installed faults only; do not pierce insulation unless specifically approved. Use current Mazda service information and obtain instructor approval before energizing or bypassing a circuit. |
| --- | --- |

PROCEDURE CHECKLIST

| STEP | PROCEDURE | CHECK |
| --- | --- | --- |
| 1 | Verify the weak, slow, dim, or intermittent circuit symptom under normal load. |  |
| 2 | Measure source voltage and circuit current or load operation for a baseline. |  |
| 3 | Measure total power-side and ground-side voltage drop while the circuit operates. |  |
| 4 | Section the side with excessive drop until the faulty connection or wire is isolated. |  |
| 5 | Inspect the isolated point and explain how resistance created the observed symptom. |  |
| 6 | Restore the fault, repeat the loaded test, and document the corrected value. |  |

MEASUREMENTS AND EVIDENCE

| ITEM | EXPECTED / SI SPEC | ACTUAL RESULT | P / F |
| --- | --- | --- | --- |
| Pre-repair total drop | Excessive side identified |  |  |
| Fault-point voltage drop | Root cause isolated |  |  |
| Post-restoration drop | Normal operation restored |  |  |

ANALYSIS AND INSTRUCTOR VERIFICATION

| What did the test evidence prove? |  |
| --- | --- |
| What is the next test or action if the result is outside specification? |  |

Instructor: __________________________    Status:  PASS  /  REVISE    Date: ______________

AUTOMOTIVE TECHNOLOGY  •  ELECTRICAL 2

MAZDA CX-9 LAB TASK SHEET

Test Starter Voltage and Current

| WEEK | TASK | VEHICLE | VIN |
| --- | --- | --- | --- |
| 4 | 1 | 2011 Mazda CX-9 | JM3TB2BA8B0308293 |

| Student |  | Date |  |
| --- | --- | --- | --- |
| Team / Bay |  | RO # |  |

OBJECTIVE

Evaluate battery voltage, starter current, and cable voltage drop during cranking.

TOOLS, RESOURCES, AND SAFETY

| Tools & Resources Mazda service information, DMM, high-current clamp, battery tester, scan tool, PPE SI diagram/procedure #: ______________________________ | Safety & Setup Wear eye protection, secure the vehicle, and maintain ignition-key control. Keep clear of belts and fans; verify Park, parking brake, and wheel chocks before cranking. Use current Mazda service information and obtain instructor approval before energizing or bypassing a circuit. |
| --- | --- |

PROCEDURE CHECKLIST

| STEP | PROCEDURE | CHECK |
| --- | --- | --- |
| 1 | Verify battery state of charge, Park, parking brake, wheel chocks, and safe cranking conditions. |  |
| 2 | Connect the DMM and high-current clamp according to the tester instructions. |  |
| 3 | Disable starting or fuel operation only when the Mazda procedure requires it. |  |
| 4 | Crank for the specified time and record minimum battery voltage and peak/stable current. |  |
| 5 | Measure positive-cable and ground-cable voltage drop during the same operating condition. |  |
| 6 | Compare all results with service information and identify the most likely next action. |  |

MEASUREMENTS AND EVIDENCE

| ITEM | EXPECTED / SI SPEC | ACTUAL RESULT | P / F |
| --- | --- | --- | --- |
| Minimum cranking voltage | Per service information |  |  |
| Starter current | Per service information |  |  |
| Positive / ground voltage drop | Within service limits |  |  |

ANALYSIS AND INSTRUCTOR VERIFICATION

| What did the test evidence prove? |  |
| --- | --- |
| What is the next test or action if the result is outside specification? |  |

Instructor: __________________________    Status:  PASS  /  REVISE    Date: ______________

AUTOMOTIVE TECHNOLOGY  •  ELECTRICAL 2

MAZDA CX-9 LAB TASK SHEET

Test Alternator Output Under Load

| WEEK | TASK | VEHICLE | VIN |
| --- | --- | --- | --- |
| 4 | 2 | 2011 Mazda CX-9 | JM3TB2BA8B0308293 |

| Student |  | Date |  |
| --- | --- | --- | --- |
| Team / Bay |  | RO # |  |

OBJECTIVE

Evaluate charging voltage and current while the alternator supplies a controlled electrical load.

TOOLS, RESOURCES, AND SAFETY

| Tools & Resources Mazda service information, DMM, high-current clamp, battery tester, scan tool, PPE SI diagram/procedure #: ______________________________ | Safety & Setup Wear eye protection, secure the vehicle, and maintain ignition-key control. Keep leads clear of rotating and hot parts; do not disconnect the battery with the engine running. Use current Mazda service information and obtain instructor approval before energizing or bypassing a circuit. |
| --- | --- |

PROCEDURE CHECKLIST

| STEP | PROCEDURE | CHECK |
| --- | --- | --- |
| 1 | Inspect belt condition, battery connections, charge indicator, and related DTCs. |  |
| 2 | Connect the DMM and current clamp with leads clear of all moving and hot components. |  |
| 3 | Start the engine and record charging voltage and current at the specified temperature and speed. |  |
| 4 | Apply approved electrical loads or the specified charging-system load test. |  |
| 5 | Record loaded voltage and current and compare them with Mazda service information. |  |
| 6 | Return loads to normal, remove equipment safely, and document the charging-system decision. |  |

MEASUREMENTS AND EVIDENCE

| ITEM | EXPECTED / SI SPEC | ACTUAL RESULT | P / F |
| --- | --- | --- | --- |
| No-load charging voltage/current | Per service information |  |  |
| Loaded charging voltage/current | Per service information |  |  |
| Charge indicator / DTC status | Normal / no related DTC |  |  |

ANALYSIS AND INSTRUCTOR VERIFICATION

| What did the test evidence prove? |  |
| --- | --- |
| What is the next test or action if the result is outside specification? |  |

Instructor: __________________________    Status:  PASS  /  REVISE    Date: ______________

AUTOMOTIVE TECHNOLOGY  •  ELECTRICAL 2

MAZDA CX-9 LAB TASK SHEET

Measure Alternator AC Ripple Voltage

| WEEK | TASK | VEHICLE | VIN |
| --- | --- | --- | --- |
| 4 | 3 | 2011 Mazda CX-9 | JM3TB2BA8B0308293 |

| Student |  | Date |  |
| --- | --- | --- | --- |
| Team / Bay |  | RO # |  |

OBJECTIVE

Check alternator rectifier performance by measuring unwanted AC content in the charging voltage.

TOOLS, RESOURCES, AND SAFETY

| Tools & Resources Mazda service information, DMM, high-current clamp, battery tester, scan tool, PPE SI diagram/procedure #: ______________________________ | Safety & Setup Wear eye protection, secure the vehicle, and maintain ignition-key control. Keep leads clear of rotating and hot parts; do not disconnect the battery with the engine running. Use current Mazda service information and obtain instructor approval before energizing or bypassing a circuit. |
| --- | --- |

PROCEDURE CHECKLIST

| STEP | PROCEDURE | CHECK |
| --- | --- | --- |
| 1 | Verify battery condition and complete a visual inspection of the charging system. |  |
| 2 | Set the DMM to AC volts or configure the scope using the approved ripple-test procedure. |  |
| 3 | Connect directly across the battery with correct polarity and safe lead routing. |  |
| 4 | Run the engine at the specified speed and apply the electrical load required by the procedure. |  |
| 5 | Record AC ripple and, when using a scope, save or sketch the waveform pattern. |  |
| 6 | Compare with service information and state whether diode or stator testing is indicated. |  |

MEASUREMENTS AND EVIDENCE

| ITEM | EXPECTED / SI SPEC | ACTUAL RESULT | P / F |
| --- | --- | --- | --- |
| Battery DC voltage during test | Charging normally |  |  |
| AC ripple voltage | Within service limit |  |  |
| Waveform pattern / decision | No abnormal missing phase or spike |  |  |

ANALYSIS AND INSTRUCTOR VERIFICATION

| What did the test evidence prove? |  |
| --- | --- |
| What is the next test or action if the result is outside specification? |  |

Instructor: __________________________    Status:  PASS  /  REVISE    Date: ______________

AUTOMOTIVE TECHNOLOGY  •  ELECTRICAL 2

MAZDA CX-9 LAB TASK SHEET

Perform Key-Off Parasitic-Draw Test

| WEEK | TASK | VEHICLE | VIN |
| --- | --- | --- | --- |
| 4 | 4 | 2011 Mazda CX-9 | JM3TB2BA8B0308293 |

| Student |  | Date |  |
| --- | --- | --- | --- |
| Team / Bay |  | RO # |  |

OBJECTIVE

Measure vehicle sleep current and isolate an excessive key-off draw without waking modules unnecessarily.

TOOLS, RESOURCES, AND SAFETY

| Tools & Resources Mazda service information, fused ammeter or low-amp clamp, DMM, scan tool, latch tools SI diagram/procedure #: ______________________________ | Safety & Setup Wear eye protection, secure the vehicle, and maintain ignition-key control. Do not crank or operate high-current loads with an ammeter installed in series. Use current Mazda service information and obtain instructor approval before energizing or bypassing a circuit. |
| --- | --- |

PROCEDURE CHECKLIST

| STEP | PROCEDURE | CHECK |
| --- | --- | --- |
| 1 | Verify the complaint, battery condition, and vehicle shut-down requirements in service information. |  |
| 2 | Prepare door, hood, and liftgate latches so the vehicle can remain closed electrically. |  |
| 3 | Connect a fused ammeter or low-amp clamp using the approved method and range. |  |
| 4 | Lock or shut down the vehicle and wait the specified time for modules to enter sleep mode. |  |
| 5 | Record stabilized draw; if excessive, isolate the affected branch using approved fuse testing. |  |
| 6 | Restore all latches and fuses, verify normal sleep current, and record the affected circuit. |  |

MEASUREMENTS AND EVIDENCE

| ITEM | EXPECTED / SI SPEC | ACTUAL RESULT | P / F |
| --- | --- | --- | --- |
| Sleep time | Per service information |  |  |
| Stabilized key-off draw | Within service limit |  |  |
| Affected fuse/circuit | Isolated if draw is excessive |  |  |

ANALYSIS AND INSTRUCTOR VERIFICATION

| What did the test evidence prove? |  |
| --- | --- |
| What is the next test or action if the result is outside specification? |  |

Instructor: __________________________    Status:  PASS  /  REVISE    Date: ______________

AUTOMOTIVE TECHNOLOGY  •  ELECTRICAL 2

MAZDA CX-9 LAB TASK SHEET

Test a 5-Volt Reference Circuit

| WEEK | TASK | VEHICLE | VIN |
| --- | --- | --- | --- |
| 6 | 1 | 2011 Mazda CX-9 | JM3TB2BA8B0308293 |

| Student |  | Date |  |
| --- | --- | --- | --- |
| Team / Bay |  | RO # |  |

OBJECTIVE

Verify reference voltage, sensor ground, and signal operation on an assigned three-wire sensor circuit.

TOOLS, RESOURCES, AND SAFETY

| Tools & Resources Mazda service information, DMM, scan tool, backprobe pins, graphing meter or lab scope SI diagram/procedure #: ______________________________ | Safety & Setup Wear eye protection, secure the vehicle, and maintain ignition-key control. Do not short the 5-volt reference; use high-impedance instruments and approved probes. Use current Mazda service information and obtain instructor approval before energizing or bypassing a circuit. |
| --- | --- |

PROCEDURE CHECKLIST

| STEP | PROCEDURE | CHECK |
| --- | --- | --- |
| 1 | Use the wiring diagram to identify the reference, signal, and sensor-ground terminals. |  |
| 2 | With KOEO, backprobe and record 5-volt reference relative to sensor ground. |  |
| 3 | Measure sensor-ground voltage relative to battery negative under connected conditions. |  |
| 4 | Measure signal voltage and compare it with the scan-tool PID. |  |
| 5 | Disconnect the sensor only if directed and observe the signal or PID response. |  |
| 6 | Reconnect, inspect terminal integrity, clear disturbed-connector DTCs as approved, and recheck. |  |

MEASUREMENTS AND EVIDENCE

| ITEM | EXPECTED / SI SPEC | ACTUAL RESULT | P / F |
| --- | --- | --- | --- |
| Reference voltage | Approximately 5 V / per SI |  |  |
| Sensor-ground voltage drop | Within service limit |  |  |
| Signal voltage and PID | Plausible and corresponding |  |  |

ANALYSIS AND INSTRUCTOR VERIFICATION

| What did the test evidence prove? |  |
| --- | --- |
| What is the next test or action if the result is outside specification? |  |

Instructor: __________________________    Status:  PASS  /  REVISE    Date: ______________

AUTOMOTIVE TECHNOLOGY  •  ELECTRICAL 2

MAZDA CX-9 LAB TASK SHEET

Graph a Thermistor Signal

| WEEK | TASK | VEHICLE | VIN |
| --- | --- | --- | --- |
| 6 | 2 | 2011 Mazda CX-9 | JM3TB2BA8B0308293 |

| Student |  | Date |  |
| --- | --- | --- | --- |
| Team / Bay |  | RO # |  |

OBJECTIVE

Observe and explain how a temperature sensor's voltage and scan-data temperature change as temperature changes.

TOOLS, RESOURCES, AND SAFETY

| Tools & Resources Mazda service information, DMM, scan tool, backprobe pins, graphing meter or lab scope SI diagram/procedure #: ______________________________ | Safety & Setup Wear eye protection, secure the vehicle, and maintain ignition-key control. Do not short the 5-volt reference; use high-impedance instruments and approved probes. Use current Mazda service information and obtain instructor approval before energizing or bypassing a circuit. |
| --- | --- |

PROCEDURE CHECKLIST

| STEP | PROCEDURE | CHECK |
| --- | --- | --- |
| 1 | Select the assigned ECT or IAT thermistor and identify its signal and ground circuits. |  |
| 2 | Before warm-up, compare the sensor PID with ambient temperature and record the cold value. |  |
| 3 | Connect a graphing meter, scope, or scan-tool graph to the sensor signal. |  |
| 4 | Warm the engine or sensor using the instructor-approved method while monitoring the graph. |  |
| 5 | Record signal voltage and PID temperature at cold, midrange, and warm conditions. |  |
| 6 | Inspect the graph for smooth change, dropouts, spikes, or implausible readings. |  |

MEASUREMENTS AND EVIDENCE

| ITEM | EXPECTED / SI SPEC | ACTUAL RESULT | P / F |
| --- | --- | --- | --- |
| Cold voltage / temperature | Plausible near ambient |  |  |
| Midrange voltage / temperature | Smooth inverse relationship |  |  |
| Warm voltage / temperature | Plausible and stable |  |  |

ANALYSIS AND INSTRUCTOR VERIFICATION

| What did the test evidence prove? |  |
| --- | --- |
| What is the next test or action if the result is outside specification? |  |

Instructor: __________________________    Status:  PASS  /  REVISE    Date: ______________

AUTOMOTIVE TECHNOLOGY  •  ELECTRICAL 2

MAZDA CX-9 LAB TASK SHEET

Sweep-Test a Potentiometer Signal

| WEEK | TASK | VEHICLE | VIN |
| --- | --- | --- | --- |
| 6 | 3 | 2011 Mazda CX-9 | JM3TB2BA8B0308293 |

| Student |  | Date |  |
| --- | --- | --- | --- |
| Team / Bay |  | RO # |  |

OBJECTIVE

Evaluate a position-sensor signal for correct range, smooth movement, and dropouts.

TOOLS, RESOURCES, AND SAFETY

| Tools & Resources Mazda service information, DMM, scan tool, backprobe pins, graphing meter or lab scope SI diagram/procedure #: ______________________________ | Safety & Setup Wear eye protection, secure the vehicle, and maintain ignition-key control. Do not short the 5-volt reference; use high-impedance instruments and approved probes. Use current Mazda service information and obtain instructor approval before energizing or bypassing a circuit. |
| --- | --- |

PROCEDURE CHECKLIST

| STEP | PROCEDURE | CHECK |
| --- | --- | --- |
| 1 | Identify the assigned APP, throttle-position, or other potentiometer signal circuit. |  |
| 2 | Confirm test conditions and connect the scan graph or scope without damaging terminals. |  |
| 3 | Record closed/rest voltage for the primary signal and the second track when equipped. |  |
| 4 | Move the control slowly through its full approved range while observing the graph. |  |
| 5 | Record full-range voltage and note any spikes, flat spots, reversals, or dropouts. |  |
| 6 | Return the control to rest and verify repeatability and correct dual-track relationship. |  |

MEASUREMENTS AND EVIDENCE

| ITEM | EXPECTED / SI SPEC | ACTUAL RESULT | P / F |
| --- | --- | --- | --- |
| Rest/closed signal | Within service range |  |  |
| Full-range signal | Within service range |  |  |
| Sweep quality / track correlation | Smooth and repeatable |  |  |

ANALYSIS AND INSTRUCTOR VERIFICATION

| What did the test evidence prove? |  |
| --- | --- |
| What is the next test or action if the result is outside specification? |  |

Instructor: __________________________    Status:  PASS  /  REVISE    Date: ______________

AUTOMOTIVE TECHNOLOGY  •  ELECTRICAL 2

MAZDA CX-9 LAB TASK SHEET

Compare Sensor PID to Meter Reading

| WEEK | TASK | VEHICLE | VIN |
| --- | --- | --- | --- |
| 6 | 4 | 2011 Mazda CX-9 | JM3TB2BA8B0308293 |

| Student |  | Date |  |
| --- | --- | --- | --- |
| Team / Bay |  | RO # |  |

OBJECTIVE

Correlate a direct electrical measurement with scan-tool data to verify the sensor, circuit, and module interpretation.

TOOLS, RESOURCES, AND SAFETY

| Tools & Resources Mazda service information, DMM, scan tool, backprobe pins, graphing meter or lab scope SI diagram/procedure #: ______________________________ | Safety & Setup Wear eye protection, secure the vehicle, and maintain ignition-key control. Do not short the 5-volt reference; use high-impedance instruments and approved probes. Use current Mazda service information and obtain instructor approval before energizing or bypassing a circuit. |
| --- | --- |

PROCEDURE CHECKLIST

| STEP | PROCEDURE | CHECK |
| --- | --- | --- |
| 1 | Choose an instructor-assigned analog sensor with an accessible signal circuit and related PID. |  |
| 2 | Identify the signal terminal, expected voltage range, and PID units from service information. |  |
| 3 | Backprobe the signal and record the DMM voltage at the first operating condition. |  |
| 4 | Record the related scan-tool PID at the same moment and operating condition. |  |
| 5 | Repeat at two additional operating conditions and compare the relationship. |  |
| 6 | Explain whether any disagreement points to the sensor, wiring, module, or conversion assumption. |  |

MEASUREMENTS AND EVIDENCE

| ITEM | EXPECTED / SI SPEC | ACTUAL RESULT | P / F |
| --- | --- | --- | --- |
| Condition 1: voltage / PID | Readings correlate |  |  |
| Condition 2: voltage / PID | Readings correlate |  |  |
| Condition 3: voltage / PID | Readings correlate |  |  |

ANALYSIS AND INSTRUCTOR VERIFICATION

| What did the test evidence prove? |  |
| --- | --- |
| What is the next test or action if the result is outside specification? |  |

Instructor: __________________________    Status:  PASS  /  REVISE    Date: ______________

AUTOMOTIVE TECHNOLOGY  •  ELECTRICAL 2

MAZDA CX-9 LAB TASK SHEET

Command an Actuator with a Scan Tool

| WEEK | TASK | VEHICLE | VIN |
| --- | --- | --- | --- |
| 7 | 1 | 2011 Mazda CX-9 | JM3TB2BA8B0308293 |

| Student |  | Date |  |
| --- | --- | --- | --- |
| Team / Bay |  | RO # |  |

OBJECTIVE

Use bidirectional control to verify whether an assigned output responds to a module command.

TOOLS, RESOURCES, AND SAFETY

| Tools & Resources Mazda-compatible scan tool, Mazda service information, DMM, backprobe pins, PPE SI diagram/procedure #: ______________________________ | Safety & Setup Wear eye protection, secure the vehicle, and maintain ignition-key control. Confirm actuator test conditions and keep hands clear of moving or hot components. Use current Mazda service information and obtain instructor approval before energizing or bypassing a circuit. |
| --- | --- |

PROCEDURE CHECKLIST

| STEP | PROCEDURE | CHECK |
| --- | --- | --- |
| 1 | Identify the actuator, control module, test prerequisites, and hazards in service information. |  |
| 2 | Complete a pre-scan and confirm battery voltage and operating conditions are acceptable. |  |
| 3 | Navigate to the correct output-control test and review the available command states. |  |
| 4 | Command the actuator OFF and ON or through the approved range while observing operation. |  |
| 5 | Record command PID, feedback PID, voltage, current, sound, or movement as applicable. |  |
| 6 | End the test, return commands to default, check DTCs, and state the next diagnostic action. |  |

MEASUREMENTS AND EVIDENCE

| ITEM | EXPECTED / SI SPEC | ACTUAL RESULT | P / F |
| --- | --- | --- | --- |
| Command state/range | Command accepted |  |  |
| Actuator response / feedback | Matches command |  |  |
| Voltage/current or observation | Within service expectation |  |  |

ANALYSIS AND INSTRUCTOR VERIFICATION

| What did the test evidence prove? |  |
| --- | --- |
| What is the next test or action if the result is outside specification? |  |

Instructor: __________________________    Status:  PASS  /  REVISE    Date: ______________

AUTOMOTIVE TECHNOLOGY  •  ELECTRICAL 2

MAZDA CX-9 LAB TASK SHEET

Test Low-Side Driver Operation

| WEEK | TASK | VEHICLE | VIN |
| --- | --- | --- | --- |
| 7 | 2 | 2011 Mazda CX-9 | JM3TB2BA8B0308293 |

| Student |  | Date |  |
| --- | --- | --- | --- |
| Team / Bay |  | RO # |  |

OBJECTIVE

Verify a module-controlled ground driver without bypassing or damaging the control module.

TOOLS, RESOURCES, AND SAFETY

| Tools & Resources Mazda service information, wiring diagram, DMM, lab scope, current probe, backprobe pins SI diagram/procedure #: ______________________________ | Safety & Setup Wear eye protection, secure the vehicle, and maintain ignition-key control. Do not ground a module driver directly; verify circuit design before connecting test equipment. Use current Mazda service information and obtain instructor approval before energizing or bypassing a circuit. |
| --- | --- |

PROCEDURE CHECKLIST

| STEP | PROCEDURE | CHECK |
| --- | --- | --- |
| 1 | Use the diagram to identify load power, driver-control wire, module pin, and ground path. |  |
| 2 | Verify power is available at the load before evaluating the low-side driver. |  |
| 3 | Backprobe the driver circuit and connect the DMM or scope using an approved reference. |  |
| 4 | Command the output OFF and ON while recording driver-circuit voltage or waveform. |  |
| 5 | Compare module command, driver voltage, and load response to determine circuit status. |  |
| 6 | If abnormal, isolate the load, wire, connector, or module only through approved pinpoint tests. |  |

MEASUREMENTS AND EVIDENCE

| ITEM | EXPECTED / SI SPEC | ACTUAL RESULT | P / F |
| --- | --- | --- | --- |
| Load power feed | Correct source voltage |  |  |
| Driver OFF / ON voltage | Switches as designed |  |  |
| Command PID / load response | Corresponding operation |  |  |

ANALYSIS AND INSTRUCTOR VERIFICATION

| What did the test evidence prove? |  |
| --- | --- |
| What is the next test or action if the result is outside specification? |  |

Instructor: __________________________    Status:  PASS  /  REVISE    Date: ______________

AUTOMOTIVE TECHNOLOGY  •  ELECTRICAL 2

MAZDA CX-9 LAB TASK SHEET

Measure Solenoid Resistance and Current

| WEEK | TASK | VEHICLE | VIN |
| --- | --- | --- | --- |
| 7 | 3 | 2011 Mazda CX-9 | JM3TB2BA8B0308293 |

| Student |  | Date |  |
| --- | --- | --- | --- |
| Team / Bay |  | RO # |  |

OBJECTIVE

Compare a solenoid's static resistance with its operating current to evaluate the coil and control circuit.

TOOLS, RESOURCES, AND SAFETY

| Tools & Resources Mazda service information, wiring diagram, DMM, lab scope, current probe, backprobe pins SI diagram/procedure #: ______________________________ | Safety & Setup Wear eye protection, secure the vehicle, and maintain ignition-key control. Do not ground a module driver directly; verify circuit design before connecting test equipment. Use current Mazda service information and obtain instructor approval before energizing or bypassing a circuit. |
| --- | --- |

PROCEDURE CHECKLIST

| STEP | PROCEDURE | CHECK |
| --- | --- | --- |
| 1 | Identify the assigned solenoid terminals, resistance specification, and activation limits. |  |
| 2 | Depower and isolate the solenoid as directed, then measure coil resistance. |  |
| 3 | Inspect the connector and reconnect the circuit before operating-current testing. |  |
| 4 | Connect a current probe or meter using the approved method and range. |  |
| 5 | Command or operate the solenoid for the permitted time and record current behavior. |  |
| 6 | Compare resistance, current, and command response and state whether the coil or circuit is suspect. |  |

MEASUREMENTS AND EVIDENCE

| ITEM | EXPECTED / SI SPEC | ACTUAL RESULT | P / F |
| --- | --- | --- | --- |
| Solenoid resistance | Within service range |  |  |
| Operating current | Within service range |  |  |
| Command / response | Consistent and repeatable |  |  |

ANALYSIS AND INSTRUCTOR VERIFICATION

| What did the test evidence prove? |  |
| --- | --- |
| What is the next test or action if the result is outside specification? |  |

Instructor: __________________________    Status:  PASS  /  REVISE    Date: ______________

AUTOMOTIVE TECHNOLOGY  •  ELECTRICAL 2

MAZDA CX-9 LAB TASK SHEET

Analyze PWM Duty Cycle

| WEEK | TASK | VEHICLE | VIN |
| --- | --- | --- | --- |
| 7 | 4 | 2011 Mazda CX-9 | JM3TB2BA8B0308293 |

| Student |  | Date |  |
| --- | --- | --- | --- |
| Team / Bay |  | RO # |  |

OBJECTIVE

Measure pulse-width modulation and relate duty-cycle changes to commanded actuator output.

TOOLS, RESOURCES, AND SAFETY

| Tools & Resources Mazda service information, wiring diagram, DMM, lab scope, current probe, backprobe pins SI diagram/procedure #: ______________________________ | Safety & Setup Wear eye protection, secure the vehicle, and maintain ignition-key control. Do not ground a module driver directly; verify circuit design before connecting test equipment. Use current Mazda service information and obtain instructor approval before energizing or bypassing a circuit. |
| --- | --- |

PROCEDURE CHECKLIST

| STEP | PROCEDURE | CHECK |
| --- | --- | --- |
| 1 | Identify the PWM-controlled actuator, control wire, expected frequency, and test conditions. |  |
| 2 | Connect the scope or duty-cycle meter with correct reference and lead routing. |  |
| 3 | Command the lowest approved output and record duty cycle, frequency, and actuator response. |  |
| 4 | Command a midrange output and repeat the measurements. |  |
| 5 | Command the highest approved output and repeat without exceeding activation limits. |  |
| 6 | Compare the three captures and explain how pulse width changed average actuator power. |  |

MEASUREMENTS AND EVIDENCE

| ITEM | EXPECTED / SI SPEC | ACTUAL RESULT | P / F |
| --- | --- | --- | --- |
| Low command: duty/frequency | Corresponds to low output |  |  |
| Mid command: duty/frequency | Corresponds to mid output |  |  |
| High command: duty/frequency | Corresponds to high output |  |  |

ANALYSIS AND INSTRUCTOR VERIFICATION

| What did the test evidence prove? |  |
| --- | --- |
| What is the next test or action if the result is outside specification? |  |

Instructor: __________________________    Status:  PASS  /  REVISE    Date: ______________

AUTOMOTIVE TECHNOLOGY  •  ELECTRICAL 2

MAZDA CX-9 LAB TASK SHEET

Measure DLC CAN-Bus Resistance

| WEEK | TASK | VEHICLE | VIN |
| --- | --- | --- | --- |
| 8 | 1 | 2011 Mazda CX-9 | JM3TB2BA8B0308293 |

| Student |  | Date |  |
| --- | --- | --- | --- |
| Team / Bay |  | RO # |  |

OBJECTIVE

Check high-speed CAN terminating resistance through the diagnostic connector with the network safely depowered.

TOOLS, RESOURCES, AND SAFETY

| Tools & Resources Mazda service information, network topology, DMM, breakout leads, Mazda-compatible scan tool SI diagram/procedure #: ______________________________ | Safety & Setup Wear eye protection, secure the vehicle, and maintain ignition-key control. Use a high-impedance meter and approved DLC pins; never ohm-test an energized network. Use current Mazda service information and obtain instructor approval before energizing or bypassing a circuit. |
| --- | --- |

PROCEDURE CHECKLIST

| STEP | PROCEDURE | CHECK |
| --- | --- | --- |
| 1 | Review the network diagram and verify that DLC pins 6 and 14 are the correct high-speed CAN pair. |  |
| 2 | Turn ignition off and follow the Mazda procedure for network sleep and battery disconnection. |  |
| 3 | Confirm the circuit is not energized before selecting the resistance function. |  |
| 4 | Measure resistance between DLC pins 6 and 14 without spreading the terminals. |  |
| 5 | Compare the reading with service information and explain what high or low resistance suggests. |  |
| 6 | Reconnect power as directed, restore settings, and complete a network scan if required. |  |

MEASUREMENTS AND EVIDENCE

| ITEM | EXPECTED / SI SPEC | ACTUAL RESULT | P / F |
| --- | --- | --- | --- |
| Network sleep/depower method | Procedure followed |  |  |
| Resistance pins 6 to 14 | Per SI; normally about 60 ohms |  |  |
| Interpretation | Termination/open/short assessment |  |  |

ANALYSIS AND INSTRUCTOR VERIFICATION

| What did the test evidence prove? |  |
| --- | --- |
| What is the next test or action if the result is outside specification? |  |

Instructor: __________________________    Status:  PASS  /  REVISE    Date: ______________

AUTOMOTIVE TECHNOLOGY  •  ELECTRICAL 2

MAZDA CX-9 LAB TASK SHEET

Measure CAN-H and CAN-L Bias Voltage

| WEEK | TASK | VEHICLE | VIN |
| --- | --- | --- | --- |
| 8 | 2 | 2011 Mazda CX-9 | JM3TB2BA8B0308293 |

| Student |  | Date |  |
| --- | --- | --- | --- |
| Team / Bay |  | RO # |  |

OBJECTIVE

Measure CAN line bias voltage and evaluate whether the network has plausible electrical activity.

TOOLS, RESOURCES, AND SAFETY

| Tools & Resources Mazda service information, network topology, DMM, breakout leads, Mazda-compatible scan tool SI diagram/procedure #: ______________________________ | Safety & Setup Wear eye protection, secure the vehicle, and maintain ignition-key control. Use a high-impedance meter and approved DLC pins; never ohm-test an energized network. Use current Mazda service information and obtain instructor approval before energizing or bypassing a circuit. |
| --- | --- |

PROCEDURE CHECKLIST

| STEP | PROCEDURE | CHECK |
| --- | --- | --- |
| 1 | Review DLC pin assignments and set the DMM to DC volts with a verified chassis ground. |  |
| 2 | Turn ignition on and allow the network to become active without starting the engine. |  |
| 3 | Measure CAN-H voltage at DLC pin 6 and record the fluctuating or averaged value. |  |
| 4 | Measure CAN-L voltage at DLC pin 14 using the same reference and conditions. |  |
| 5 | Compare both readings with service information and with each other. |  |
| 6 | If abnormal, identify whether the next test should target a short, open, ground, power, or module branch. |  |

MEASUREMENTS AND EVIDENCE

| ITEM | EXPECTED / SI SPEC | ACTUAL RESULT | P / F |
| --- | --- | --- | --- |
| CAN-H bias voltage | Plausible active-network value |  |  |
| CAN-L bias voltage | Plausible active-network value |  |  |
| Line relationship / decision | Consistent with SI |  |  |

ANALYSIS AND INSTRUCTOR VERIFICATION

| What did the test evidence prove? |  |
| --- | --- |
| What is the next test or action if the result is outside specification? |  |

Instructor: __________________________    Status:  PASS  /  REVISE    Date: ______________

AUTOMOTIVE TECHNOLOGY  •  ELECTRICAL 2

MAZDA CX-9 LAB TASK SHEET

Compare Network Module Status

| WEEK | TASK | VEHICLE | VIN |
| --- | --- | --- | --- |
| 8 | 3 | 2011 Mazda CX-9 | JM3TB2BA8B0308293 |

| Student |  | Date |  |
| --- | --- | --- | --- |
| Team / Bay |  | RO # |  |

OBJECTIVE

Use a full-vehicle scan and network topology to identify responding modules and communication faults.

TOOLS, RESOURCES, AND SAFETY

| Tools & Resources Mazda-compatible scan tool, Mazda service information, DMM, backprobe pins, PPE SI diagram/procedure #: ______________________________ | Safety & Setup Wear eye protection, secure the vehicle, and maintain ignition-key control. Confirm actuator test conditions and keep hands clear of moving or hot components. Use current Mazda service information and obtain instructor approval before energizing or bypassing a circuit. |
| --- | --- |

PROCEDURE CHECKLIST

| STEP | PROCEDURE | CHECK |
| --- | --- | --- |
| 1 | Connect the Mazda-compatible scan tool and verify stable battery voltage before scanning. |  |
| 2 | Perform a complete vehicle scan and save the pre-scan report. |  |
| 3 | Compare the responding-module list with the network topology and vehicle equipment. |  |
| 4 | Record all U-codes, their status, and the modules that reported them. |  |
| 5 | Identify any expected module that does not communicate and locate its network branch. |  |
| 6 | Develop the next three checks for module power, ground, and network integrity. |  |

MEASUREMENTS AND EVIDENCE

| ITEM | EXPECTED / SI SPEC | ACTUAL RESULT | P / F |
| --- | --- | --- | --- |
| Responding modules | Matches vehicle equipment |  |  |
| Nonresponding module(s) | None or correctly identified |  |  |
| U-codes and reporting modules | Status and source recorded |  |  |

ANALYSIS AND INSTRUCTOR VERIFICATION

| What did the test evidence prove? |  |
| --- | --- |
| What is the next test or action if the result is outside specification? |  |

Instructor: __________________________    Status:  PASS  /  REVISE    Date: ______________

AUTOMOTIVE TECHNOLOGY  •  ELECTRICAL 2

MAZDA CX-9 LAB TASK SHEET

Capture CAN Waveform with a Scope

| WEEK | TASK | VEHICLE | VIN |
| --- | --- | --- | --- |
| 8 | 4 | 2011 Mazda CX-9 | JM3TB2BA8B0308293 |

| Student |  | Date |  |
| --- | --- | --- | --- |
| Team / Bay |  | RO # |  |

OBJECTIVE

Capture and evaluate high-speed CAN-H and CAN-L waveforms for amplitude, symmetry, and interference.

TOOLS, RESOURCES, AND SAFETY

| Tools & Resources Mazda service information, lab scope, differential or two-channel leads, scan tool SI diagram/procedure #: ______________________________ | Safety & Setup Wear eye protection, secure the vehicle, and maintain ignition-key control. Use correct scope grounds and attenuation; avoid shorting CAN-H to CAN-L or chassis ground. Use current Mazda service information and obtain instructor approval before energizing or bypassing a circuit. |
| --- | --- |

PROCEDURE CHECKLIST

| STEP | PROCEDURE | CHECK |
| --- | --- | --- |
| 1 | Verify scope channel grounds, input limits, probe connections, and DLC pin assignments. |  |
| 2 | Connect one channel to CAN-H and the second to CAN-L using approved breakout leads. |  |
| 3 | Turn ignition on, select suitable voltage and time settings, and trigger on network activity. |  |
| 4 | Capture both lines during active communication and save or print the waveform. |  |
| 5 | Measure recessive and dominant levels and compare the two waveforms for mirror action. |  |
| 6 | Identify any clipping, ringing, noise, missing activity, short, or bias concern. |  |

MEASUREMENTS AND EVIDENCE

| ITEM | EXPECTED / SI SPEC | ACTUAL RESULT | P / F |
| --- | --- | --- | --- |
| CAN-H dominant/recessive | Clean transition per SI |  |  |
| CAN-L dominant/recessive | Clean mirrored transition |  |  |
| Waveform quality / decision | No abnormal noise or distortion |  |  |

ANALYSIS AND INSTRUCTOR VERIFICATION

| What did the test evidence prove? |  |
| --- | --- |
| What is the next test or action if the result is outside specification? |  |

Instructor: __________________________    Status:  PASS  /  REVISE    Date: ______________

AUTOMOTIVE TECHNOLOGY  •  ELECTRICAL 2

MAZDA CX-9 LAB TASK SHEET

Perform Connector Terminal-Tension Test

| WEEK | TASK | VEHICLE | VIN |
| --- | --- | --- | --- |
| 9 | 1 | 2011 Mazda CX-9 | JM3TB2BA8B0308293 |

| Student |  | Date |  |
| --- | --- | --- | --- |
| Team / Bay |  | RO # |  |

OBJECTIVE

Evaluate whether a connector terminal grips its mating pin with adequate tension and without damage.

TOOLS, RESOURCES, AND SAFETY

| Tools & Resources Mazda terminal kit, release tools, wire strippers, approved crimper, sealed splices, heat gun, DMM SI diagram/procedure #: ______________________________ | Safety & Setup Wear eye protection, secure the vehicle, and maintain ignition-key control. Depower the circuit before terminal work; protect nearby trim and wiring when applying heat. Use current Mazda service information and obtain instructor approval before energizing or bypassing a circuit. |
| --- | --- |

PROCEDURE CHECKLIST

| STEP | PROCEDURE | CHECK |
| --- | --- | --- |
| 1 | Identify the connector, terminal size, lock design, and approved test gauge in service information. |  |
| 2 | Depower the circuit, disconnect the connector, and inspect seals, locks, corrosion, and pin alignment. |  |
| 3 | Insert the correct terminal-tension gauge or known-good mating terminal straight into the cavity. |  |
| 4 | Compare insertion and removal drag with a known-good cavity of the same terminal size. |  |
| 5 | Document any loose, spread, pushed-out, corroded, or heat-damaged terminal. |  |
| 6 | Reconnect and lock the connector, perform a tug check, and verify circuit operation. |  |

MEASUREMENTS AND EVIDENCE

| ITEM | EXPECTED / SI SPEC | ACTUAL RESULT | P / F |
| --- | --- | --- | --- |
| Connector / cavity | Correctly identified |  |  |
| Tension comparison | Matches known-good terminal |  |  |
| Terminal/seal condition | No looseness or damage |  |  |

ANALYSIS AND INSTRUCTOR VERIFICATION

| What did the test evidence prove? |  |
| --- | --- |
| What evidence proves the repair is electrically and mechanically sound? |  |

Instructor: __________________________    Status:  PASS  /  REVISE    Date: ______________

AUTOMOTIVE TECHNOLOGY  •  ELECTRICAL 2

MAZDA CX-9 LAB TASK SHEET

Repair Wire Using a Sealed Splice

| WEEK | TASK | VEHICLE | VIN |
| --- | --- | --- | --- |
| 9 | 2 | 2011 Mazda CX-9 | JM3TB2BA8B0308293 |

| Student |  | Date |  |
| --- | --- | --- | --- |
| Team / Bay |  | RO # |  |

OBJECTIVE

Complete a durable automotive wire repair using the correct conductor size, crimp, seal, and strain relief.

TOOLS, RESOURCES, AND SAFETY

| Tools & Resources Mazda terminal kit, release tools, wire strippers, approved crimper, sealed splices, heat gun, DMM SI diagram/procedure #: ______________________________ | Safety & Setup Wear eye protection, secure the vehicle, and maintain ignition-key control. Depower the circuit before terminal work; protect nearby trim and wiring when applying heat. Use current Mazda service information and obtain instructor approval before energizing or bypassing a circuit. |
| --- | --- |

PROCEDURE CHECKLIST

| STEP | PROCEDURE | CHECK |
| --- | --- | --- |
| 1 | Confirm circuit function, wire gauge, conductor material, and allowable repair method. |  |
| 2 | Depower the circuit and remove the damaged section using clean, staggered cuts when required. |  |
| 3 | Strip only the specified insulation length without nicking or removing conductor strands. |  |
| 4 | Insert the wire fully into the approved sealed splice and crimp with the correct die. |  |
| 5 | Perform a pull test, then heat the splice evenly until the seal flows at both ends. |  |
| 6 | Check continuity or loaded voltage drop, rewrap the harness, and secure it from heat and movement. |  |

MEASUREMENTS AND EVIDENCE

| ITEM | EXPECTED / SI SPEC | ACTUAL RESULT | P / F |
| --- | --- | --- | --- |
| Wire gauge / splice size | Correctly matched |  |  |
| Pull test / seal condition | Secure and fully sealed |  |  |
| Post-repair continuity/drop | Within service limit |  |  |

ANALYSIS AND INSTRUCTOR VERIFICATION

| What did the test evidence prove? |  |
| --- | --- |
| What evidence proves the repair is electrically and mechanically sound? |  |

Instructor: __________________________    Status:  PASS  /  REVISE    Date: ______________

AUTOMOTIVE TECHNOLOGY  •  ELECTRICAL 2

MAZDA CX-9 LAB TASK SHEET

Depin and Repin a Connector Terminal

| WEEK | TASK | VEHICLE | VIN |
| --- | --- | --- | --- |
| 9 | 3 | 2011 Mazda CX-9 | JM3TB2BA8B0308293 |

| Student |  | Date |  |
| --- | --- | --- | --- |
| Team / Bay |  | RO # |  |

OBJECTIVE

Remove and reinstall a connector terminal without damaging the housing, lock, seal, or conductor.

TOOLS, RESOURCES, AND SAFETY

| Tools & Resources Mazda terminal kit, release tools, wire strippers, approved crimper, sealed splices, heat gun, DMM SI diagram/procedure #: ______________________________ | Safety & Setup Wear eye protection, secure the vehicle, and maintain ignition-key control. Depower the circuit before terminal work; protect nearby trim and wiring when applying heat. Use current Mazda service information and obtain instructor approval before energizing or bypassing a circuit. |
| --- | --- |

PROCEDURE CHECKLIST

| STEP | PROCEDURE | CHECK |
| --- | --- | --- |
| 1 | Use the connector view to verify connector ID, cavity number, terminal type, and wire color. |  |
| 2 | Depower and disconnect the connector, then release the secondary terminal lock. |  |
| 3 | Insert the correct release tool and gently remove the terminal by the wire and terminal body. |  |
| 4 | Inspect the locking tang, conductor crimp, insulation crimp, seal, and contact surface. |  |
| 5 | Repair or replace the terminal as approved, reinstall it in the correct cavity, and reset the lock. |  |
| 6 | Perform a tug check, reconnect the connector, and verify continuity and circuit operation. |  |

MEASUREMENTS AND EVIDENCE

| ITEM | EXPECTED / SI SPEC | ACTUAL RESULT | P / F |
| --- | --- | --- | --- |
| Connector/cavity/wire color | Matches connector view |  |  |
| Terminal and seal condition | No damage; lock restored |  |  |
| Continuity / circuit function | Normal after repinning |  |  |

ANALYSIS AND INSTRUCTOR VERIFICATION

| What did the test evidence prove? |  |
| --- | --- |
| What evidence proves the repair is electrically and mechanically sound? |  |

Instructor: __________________________    Status:  PASS  /  REVISE    Date: ______________

AUTOMOTIVE TECHNOLOGY  •  ELECTRICAL 2

MAZDA CX-9 LAB TASK SHEET

Verify Repair with Loaded Voltage Drop

| WEEK | TASK | VEHICLE | VIN |
| --- | --- | --- | --- |
| 9 | 4 | 2011 Mazda CX-9 | JM3TB2BA8B0308293 |

| Student |  | Date |  |
| --- | --- | --- | --- |
| Team / Bay |  | RO # |  |

OBJECTIVE

Prove the quality of a wiring repair by testing it under realistic circuit current and movement.

TOOLS, RESOURCES, AND SAFETY

| Tools & Resources Mazda terminal kit, release tools, wire strippers, approved crimper, sealed splices, heat gun, DMM SI diagram/procedure #: ______________________________ | Safety & Setup Wear eye protection, secure the vehicle, and maintain ignition-key control. Depower the circuit before terminal work; protect nearby trim and wiring when applying heat. Use current Mazda service information and obtain instructor approval before energizing or bypassing a circuit. |
| --- | --- |

PROCEDURE CHECKLIST

| STEP | PROCEDURE | CHECK |
| --- | --- | --- |
| 1 | Identify the repaired section and determine how the circuit will be operated under normal load. |  |
| 2 | Measure and record source voltage while the assigned load operates. |  |
| 3 | Connect the DMM directly across the repaired section and record loaded voltage drop. |  |
| 4 | Perform an approved wiggle test while watching the meter and circuit operation. |  |
| 5 | Compare results with service limits and inspect the repair for sealing, routing, and strain relief. |  |
| 6 | Secure the harness, restore protective coverings, and complete the repair documentation. |  |

MEASUREMENTS AND EVIDENCE

| ITEM | EXPECTED / SI SPEC | ACTUAL RESULT | P / F |
| --- | --- | --- | --- |
| Source voltage under load | Stable during test |  |  |
| Voltage drop across repair | Within service limit |  |  |
| Wiggle test / final operation | No interruption or change |  |  |

ANALYSIS AND INSTRUCTOR VERIFICATION

| What did the test evidence prove? |  |
| --- | --- |
| What evidence proves the repair is electrically and mechanically sound? |  |

Instructor: __________________________    Status:  PASS  /  REVISE    Date: ______________

AUTOMOTIVE TECHNOLOGY  •  ELECTRICAL 2

MAZDA CX-9 LAB TASK SHEET

Verify the Electrical Complaint

| WEEK | TASK | VEHICLE | VIN |
| --- | --- | --- | --- |
| 10 | 1 | 2011 Mazda CX-9 | JM3TB2BA8B0308293 |

| Student |  | Date |  |
| --- | --- | --- | --- |
| Team / Bay |  | RO # |  |

OBJECTIVE

Convert a customer statement into a repeatable, measurable symptom before beginning diagnosis.

TOOLS, RESOURCES, AND SAFETY

| Tools & Resources Mazda service information, Mazda-compatible scan tool, DMM, backprobe pins, repair order SI diagram/procedure #: ______________________________ | Safety & Setup Wear eye protection, secure the vehicle, and maintain ignition-key control. Do not erase codes or disturb evidence until all pre-scan data has been saved. Use current Mazda service information and obtain instructor approval before energizing or bypassing a circuit. |
| --- | --- |

PROCEDURE CHECKLIST

| STEP | PROCEDURE | CHECK |
| --- | --- | --- |
| 1 | Read the customer concern and rewrite it using the Condition, When, and How Often format. |  |
| 2 | Verify battery condition and perform a visual inspection without changing the symptom. |  |
| 3 | Identify the operating conditions, switch positions, temperature, and vehicle state involved. |  |
| 4 | Attempt to duplicate the concern safely and record exactly what works and what does not. |  |
| 5 | Check related systems and warning indicators to define the boundaries of the fault. |  |
| 6 | Write a verified problem statement and identify the first service-information path to follow. |  |

MEASUREMENTS AND EVIDENCE

| ITEM | EXPECTED / SI SPEC | ACTUAL RESULT | P / F |
| --- | --- | --- | --- |
| Customer concern rewritten | Condition/when/frequency included |  |  |
| Verified symptom | Repeatable and measurable |  |  |
| Operating conditions | Complete baseline recorded |  |  |

ANALYSIS AND INSTRUCTOR VERIFICATION

| What did the test evidence prove? |  |
| --- | --- |
| State the concern, cause, correction, or next diagnostic action. |  |

Instructor: __________________________    Status:  PASS  /  REVISE    Date: ______________

AUTOMOTIVE TECHNOLOGY  •  ELECTRICAL 2

MAZDA CX-9 LAB TASK SHEET

Retrieve DTCs and Freeze-Frame Data

| WEEK | TASK | VEHICLE | VIN |
| --- | --- | --- | --- |
| 10 | 2 | 2011 Mazda CX-9 | JM3TB2BA8B0308293 |

| Student |  | Date |  |
| --- | --- | --- | --- |
| Team / Bay |  | RO # |  |

OBJECTIVE

Preserve diagnostic evidence by completing and documenting a pre-scan before clearing codes or disconnecting components.

TOOLS, RESOURCES, AND SAFETY

| Tools & Resources Mazda service information, Mazda-compatible scan tool, DMM, backprobe pins, repair order SI diagram/procedure #: ______________________________ | Safety & Setup Wear eye protection, secure the vehicle, and maintain ignition-key control. Do not erase codes or disturb evidence until all pre-scan data has been saved. Use current Mazda service information and obtain instructor approval before energizing or bypassing a circuit. |
| --- | --- |

PROCEDURE CHECKLIST

| STEP | PROCEDURE | CHECK |
| --- | --- | --- |
| 1 | Connect the scan tool with stable battery voltage and confirm correct vehicle identification. |  |
| 2 | Perform a full-vehicle scan and save the original pre-scan report. |  |
| 3 | Record DTC number, description, reporting module, status, and occurrence information. |  |
| 4 | Open and record freeze-frame or failure-record data for the primary relevant DTC. |  |
| 5 | Review related PIDs and service bulletins or diagnostic notes without clearing any DTC. |  |
| 6 | Prioritize the codes and explain which one should be diagnosed first and why. |  |

MEASUREMENTS AND EVIDENCE

| ITEM | EXPECTED / SI SPEC | ACTUAL RESULT | P / F |
| --- | --- | --- | --- |
| Primary DTC / module / status | Complete code identity |  |  |
| Freeze-frame key values | Operating conditions preserved |  |  |
| Diagnostic priority | Evidence-based sequence |  |  |

ANALYSIS AND INSTRUCTOR VERIFICATION

| What did the test evidence prove? |  |
| --- | --- |
| State the concern, cause, correction, or next diagnostic action. |  |

Instructor: __________________________    Status:  PASS  /  REVISE    Date: ______________

AUTOMOTIVE TECHNOLOGY  •  ELECTRICAL 2

MAZDA CX-9 LAB TASK SHEET

Follow Pinpoint Test to Root Cause

| WEEK | TASK | VEHICLE | VIN |
| --- | --- | --- | --- |
| 10 | 3 | 2011 Mazda CX-9 | JM3TB2BA8B0308293 |

| Student |  | Date |  |
| --- | --- | --- | --- |
| Team / Bay |  | RO # |  |

OBJECTIVE

Use Mazda's diagnostic procedure and test evidence to identify a root cause without unnecessary parts replacement.

TOOLS, RESOURCES, AND SAFETY

| Tools & Resources Mazda service information, Mazda-compatible scan tool, DMM, backprobe pins, repair order SI diagram/procedure #: ______________________________ | Safety & Setup Wear eye protection, secure the vehicle, and maintain ignition-key control. Do not erase codes or disturb evidence until all pre-scan data has been saved. Use current Mazda service information and obtain instructor approval before energizing or bypassing a circuit. |
| --- | --- |

PROCEDURE CHECKLIST

| STEP | PROCEDURE | CHECK |
| --- | --- | --- |
| 1 | Select the correct symptom or DTC diagnostic path using the verified complaint and pre-scan. |  |
| 2 | Review prerequisites, connector views, expected values, and cautions before testing. |  |
| 3 | Perform each directed check in sequence and record the actual result before choosing the next branch. |  |
| 4 | When a result fails, verify the test setup and repeat the measurement before condemning a part. |  |
| 5 | Use a final isolation test to prove the wire, connector, power, ground, component, or module fault. |  |
| 6 | Present the concern, cause, evidence, and recommended repair to the instructor. |  |

MEASUREMENTS AND EVIDENCE

| ITEM | EXPECTED / SI SPEC | ACTUAL RESULT | P / F |
| --- | --- | --- | --- |
| Pinpoint test / step numbers | Correct path followed |  |  |
| Key failed measurement | Repeatable and documented |  |  |
| Root cause / proof test | Fault isolated without guessing |  |  |

ANALYSIS AND INSTRUCTOR VERIFICATION

| What did the test evidence prove? |  |
| --- | --- |
| State the concern, cause, correction, or next diagnostic action. |  |

Instructor: __________________________    Status:  PASS  /  REVISE    Date: ______________

AUTOMOTIVE TECHNOLOGY  •  ELECTRICAL 2

MAZDA CX-9 LAB TASK SHEET

Document Repair and Complete Post-Scan

| WEEK | TASK | VEHICLE | VIN |
| --- | --- | --- | --- |
| 10 | 4 | 2011 Mazda CX-9 | JM3TB2BA8B0308293 |

| Student |  | Date |  |
| --- | --- | --- | --- |
| Team / Bay |  | RO # |  |

OBJECTIVE

Verify the repair and produce a complete Concern-Cause-Correction record supported by post-repair evidence.

TOOLS, RESOURCES, AND SAFETY

| Tools & Resources Mazda service information, Mazda-compatible scan tool, DMM, backprobe pins, repair order SI diagram/procedure #: ______________________________ | Safety & Setup Wear eye protection, secure the vehicle, and maintain ignition-key control. Do not erase codes or disturb evidence until all pre-scan data has been saved. Use current Mazda service information and obtain instructor approval before energizing or bypassing a circuit. |
| --- | --- |

PROCEDURE CHECKLIST

| STEP | PROCEDURE | CHECK |
| --- | --- | --- |
| 1 | Review the verified root cause and confirm the approved repair was completed correctly. |  |
| 2 | Repeat the failed measurement under the same conditions and record the corrected value. |  |
| 3 | Clear DTCs only after all pre-repair information has been preserved. |  |
| 4 | Perform the required drive cycle, functional test, or bidirectional verification. |  |
| 5 | Complete a full post-scan and confirm no relevant current or pending DTCs return. |  |
| 6 | Write the repair order using Concern, Cause, Correction, measurements, and final verification. |  |

MEASUREMENTS AND EVIDENCE

| ITEM | EXPECTED / SI SPEC | ACTUAL RESULT | P / F |
| --- | --- | --- | --- |
| Pre- / post-repair measurement | Failure corrected |  |  |
| Post-scan DTC status | No relevant returning DTC |  |  |
| Final functional verification | Customer concern resolved |  |  |

ANALYSIS AND INSTRUCTOR VERIFICATION

| What did the test evidence prove? |  |
| --- | --- |
| State the concern, cause, correction, or next diagnostic action. |  |

Instructor: __________________________    Status:  PASS  /  REVISE    Date: ______________