MIDTERM REVIEW WORKBOOK

# Automotive Electrical

Concepts, circuits, meter skills, diagnostics, and electrical calculations

| NAME ________________________ | CLASS / PERIOD ________________ | DATE ____________ |
| --- | --- | --- |

YOUR REVIEW ROUTE

| 01 FOUNDATIONS | 02 CIRCUITS | 03 MULTIMETER | 04 DIAGNOSTICS + LAWS |
| --- | --- | --- | --- |

HOW TO USE THIS PACKET

- Complete each section from memory first. Use pencil and show every step.

- Check your work with notes or a classmate in a different color.

- Star any item you cannot explain aloud without looking at your notes.

- Bring those starred items to the review session and ask a specific question.

READY MEANS: I can select the correct formula or meter setup, label units, trace the circuit, and explain why the result makes sense.

COMPLETE  |  CHECK  |  CORRECT  |  EXPLAIN

# Section 1 | Electrical Theory Review Map

Check each target only when you can define it, give an example, and explain why it matters in a circuit.

### 1.1 What is Electricity?

#### Energy

  - Define Energy

  - Types

  - Law of Conservation

#### Matter and Atoms

  - Define matter, molecules, and atoms

  - Use the periodic table to identify atomic information

  - Atomic Structure

  - Explain the roles of protons, neutrons, and electrons

#### Ions

  - Define ions

  - Distinguish between positive and negative ions

  - Explain how ions help describe electrical behavior

#### Conductors vs. Insulators

  - Define conductors and insulators

  - Give examples of conductors and insulators

  - Explain how an insulator can become conductive

  - Describe the basic role of semiconductors

### 1.2 Electrical Flow

#### Conventional vs. Electron Flow

  - Define conventional-current and electron-flow theory

  - Explain the origin of each flow theory

#### AC vs DC

  - Define direct current (DC) and alternating current (AC)

  - Identify where DC is used

  - Identify where AC is used

  - Describe how AC and DC can be converted

#### Effects of Electrical Flow

  - Identify the effects of electrical current, explain useful applications, and recognize harmful effects.

### 1.3 Properties of Electricity

#### Current

  - Define current and state its unit of measurement

  - Explain the effects of high and low current flow

  - Explain its relationship in Ohm’s Law and Watt’s Law

#### Voltage

  - Define voltage and state its unit of measurement

  - Explain how voltage is generated

  - Explain its relationship in Ohm’s Law and Watt’s Law

#### Resistance

  - Define Resistance

  - Identify what creates resistance in a circuit

  - Explain its relationship in Ohm’s Law

  - Explain the four factors that affect resistance

#### Electrical Power

  - Define electrical power and state its unit of measurement

  - Compare electrical power with mechanical power

  - Explain its relationship in Watt’s Law

## Energy

Energy is defined by  ___________________________________________________________________________________

Types of Energy:

| Definitions: | 1-5 | Terms: | A-E | Examples |
| --- | --- | --- | --- | --- |
| - Energy stored in chemical bonds. - Energy from an object's temperature. - Energy from moving electric charges. - Energy of motion. - Stored energy due to position. |  | Kinetic Energy Potential Energy Thermal Energy Chemical Energy Electrical Energy |  | - A rolling ball - Water behind a dam - Boiling water - Gasoline in a car - Powering a lamp |

The Law of Conservation of Energy is defined as: _______________________________________________________

## Matter

_________________________	- Anything that has mass and takes up space.

_________________________	- Two or more atoms bonded together.

_________________________	- The amount of matter in an object.

_________________________	- A chemical bond where atoms share electrons.

_________________________	- A chemical bond where atoms transfer electrons.

_________________________	- The basic building blocks of matter.

- Matter is made of atoms and molecules, bonded by covalent or ionic bonds, and mass measures how much matter they contain.

- Covalent = Cooperate = Share

- Ionic = I Owe = Give or take

## Atomic Structure

- _________________________	- Contain 3 particles P_____, N______, and E_______

- _________________________	- Located at the center of an atom and contains Proton and  Neutron

- _________________________	- Are positively charged

- _________________________	- Have no charge

- _________________________	- Negatively Charged

- _______________ Rings/Shells	- Contains Electrons and can have

- _________________________	- Located in the outer ring/shell

- _________________________	- An extra electron that’s free to float to another

## Types of Atoms

|  | - Conductors - Insulators - Semiconductors - Ions - Positive Ion - Negative Ion |
| --- | --- |

## Electrical Flow

|  | - Conventional Flow - Electron Flow Theory - Alternating Current - Direct Current - Hole Theory |
| --- | --- |

## Properties of Electricity:

- _________________________ - Electrical pressure

- _________________________ - Rate of electron flow

- _________________________ - Opposition to current flow

- _________________________ - Rate at which electrical work is performed

- _________________________ - Unit used to measure voltage

- _________________________ - Unit used to measure current

- _________________________ - Unit used to measure resistance

- _________________________ - Unit used to measure electrical power

_________________________ is electrical pressure that drives _________________________ through _________________________. A load converts that electrical energy into _________________________.

- The greater the difference in electrical charge between two points, the greater the _________________________.

- With resistance held constant, the higher the __________________, the greater the current flow.

- Total circuit __________________ drop equals source voltage.

- Electrical __________________ is consumed by resistance in the circuit.

- In a series circuit, the highest resistance will have the greatest __________________ consumption.

- The intended load should provide the circuit’s primary resistance; unwanted resistance indicates a fault.

# Section 2 | Circuit Fundamentals Review Map

Use this map to identify gaps before completing the circuit-analysis activities that follow.

### 2.1 Circuit Terminology

#### Definition of a Circuit

  - Explain why a complete circuit must provide a closed path for current.

  - Distinguish among open, closed, and short circuits.

#### Circuit Symbols and Diagrams

  - Identify common electrical symbols used in schematics.

  - Trace current paths and interpret circuit diagrams.

  - Explain why standardized symbols matter in service information.

  - Practice interpreting circuit diagrams.

#### Units of Measurement

  - Review SI units used in electrical work: volts, amps, ohms, and watts.

  - Convert among milli-, base-, kilo-, and mega-unit ranges.

#### Relevant Terms

  - Open vs. Closed (Complete) Circuits

  - Continuity

  - Polarity

  - Ground-Controlled vs. Power-Controlled Circuits

### 2.2 Five Components of a Circuit

#### Power Source

  - Define a power source

  - Identify power-source types (AC supply, DC battery, solar cell).

  - Compare primary and secondary power sources

  - Recognize power-source symbols in circuit diagrams.

  - Explain internal resistance in a power source.

  - Recognize battery, generator, and other power-source symbols.

#### Conductive Path

  - Define a conductive path

  - Materials used as conductors (copper, aluminum, gold, silver).

  - Explain how conductivity affects circuit performance.

  - Explain how conductor length affects resistance

  - Recognize ground, splice, connector, and wire symbols.

#### Load Device

  - Define a load device

  - Identify resistive, inductive, and capacitive loads.

  - Explain how a load converts electrical energy into heat, light, sound, or motion.

  - Compare steady-current and pulse-controlled loads

  - Explain why the intended load should provide the circuit’s primary resistance.

  - Recognize common load-device symbols

#### Control Device

  - Define Control Device

  - Identify components used as control devices.

  - Compare common switch types used to control circuits

  - Distinguish power-side control from ground-side control

  - Symbols for the different control devices

#### Protective Device

  - Define a circuit-protection device

  - Identify fuses, circuit breakers, and fusible links.

  - Explain how protection devices prevent overcurrent damage.

  - Recognize protection-device symbols

  - Apply circuit protection to real-world fault scenarios.

### Simple Circuit

#### Characteristics of a Simple Circuit

  - Define a simple circuit

  - Explain how voltage, current, and resistance interact in a simple circuit.

  - Explain why a closed loop is required for current flow.

  - Predict the effects of opens, shorts, and overloads.

#### Building and Testing a Simple Circuit

  - Hands-on activity: Constructing a simple circuit using a breadboard.

  - Step-by-step guide on measuring voltage, current, and resistance using a multimeter.

  - Analysis of the impact of different resistances and loads on circuit performance.

#### Applications of Simple Circuits

  - Real-world examples such as simple lighting circuits, battery-operated devices.

  - Discussion on the importance of simple circuits as the foundation for more complex systems.

### Series Circuit

#### Characteristics of Series Circuits

  - Define a series circuit

  - Voltage

  - Current

  - Resistance

#### Disadvantages and Advantages

  - Predict circuit operation if one series load opens.

  - Explain how source voltage divides across series loads.

  - Identify one design advantage and one limitation of a series circuit.

#### Calculating Values in Series Circuits

  - Application of Ohm’s Law to calculate voltage drops, total resistance, and current.

  - Practice problems: Solving for unknown values in series circuits.

  - The importance of understanding power distribution in a series circuit.

#### Building and Testing a Series Circuit

  - Hands-on activity: Constructing a series circuit with multiple resistors and loads.

  - Measuring voltage drops across each component and comparing with theoretical values.

  - Troubleshooting common issues in series circuits (e.g., identifying failed components).

#### Applications of Series Circuits

  - Exploration of series circuits in real-world applications like string lights, resistive heating elements.

  - Discussion on the advantages and disadvantages of series circuits in different scenarios.

### Parallel Circuit

#### Characteristics of Parallel Circuits

  - Define a parallel circuit

  - Analysis of how voltage remains constant across all branches in a parallel circuit.

  - Exploration of current division among branches and its dependence on branch resistance.

  - Discussion on the impact of adding or removing components in a parallel circuit.

#### Advantages and Disadvantages of a Parallel Circuit

#### Calculating Values in Parallel Circuits

  - Application of Ohm’s Law to calculate total resistance, individual branch currents, and overall current.

  - Practice problems: Solving for unknown values in parallel circuits.

  - The significance of total power calculation in parallel circuits.

#### Building and Testing a Parallel Circuit

  - Hands-on activity: Constructing a parallel circuit with multiple branches.

  - Measuring voltage and current in each branch and comparing with theoretical predictions.

  - Troubleshooting parallel circuits, identifying open branches or short circuits.

#### Applications of Parallel Circuits

  - Exploration of parallel circuits in real-world applications like household electrical wiring, automotive electrical systems.

  - Discussion on the advantages of parallel circuits in maintaining functionality even if one branch fails.

### Series-Parallel Circuits

#### Characteristics of Series-Parallel Circuits

  - Define a series-parallel circuit

  - Analysis of how voltage and current are distributed across different sections of the circuit.

  - The interplay between series and parallel elements and their impact on circuit behavior.

  - Discussion on how adding or removing components affects the overall circuit.

#### Calculating Values in Series-Parallel Circuits

  - Step-by-step approach to solving complex circuits using Ohm’s Law.

  - Techniques for simplifying series-parallel circuits into simpler equivalent circuits.

  - Practice problems: Analyzing and solving for unknown values in series-parallel circuits.

#### Building and Testing a Series-Parallel Circuit

  - Hands-on activity: Constructing a series-parallel circuit and analyzing its performance.

  - Measuring voltage and current at different points in the circuit and comparing with theoretical calculations.

  - Troubleshooting complex circuits, identifying issues with specific sections.

#### Applications of Series-Parallel Circuits

  - Exploration of series-parallel circuits in real-world applications such as automotive lighting systems, power distribution networks.

  - Discussion on the advantages of series-parallel circuits in balancing the benefits of both series and parallel configurations

## Five Components of a Circuit

| 1. __________________________ | Supplies electrical energy to the circuit. |
| --- | --- |
| 2. __________________ Device | Opens the circuit during an overcurrent condition. |
| 3. __________________ Device | Turns the circuit on, off, or otherwise controls operation. |
| 4. __________________________ | Carries current from component to component. |
| 5. __________________________ | Converts electrical energy into another form such as light, heat, sound, or motion. |

Effects of Electricity

| - Illumination______________________________ |  |
| --- | --- |

## Load Devices | Match the Device to Its Function

| 1. __________________________ | Converts electrical energy into illumination. |
| --- | --- |
| 2. __________________________ | Converts electrical energy into heat through resistance. |
| 3. __________________________ | A coil of wire used to create a magnetic field or resistance. |
| 4. __________________________ | Uses electromagnetic force to produce linear motion. |
| 5. __________________________ | Converts an electrical control signal into mechanical movement. |
| 6. __________________________ | Uses electromagnetic force to produce rotational motion. |
| 7. __________________________ | Uses electromagnetic force to open or close another circuit. |
| 8. __________________________ | Converts electrical energy into sound. |
| 9. __________________________ | Transfers energy between circuits through electromagnetic induction and may change voltage. |
| 10. __________________________ | Limits or regulates current flow. |

## Circuit Terminology

- Open Circuit

- Closed (Complete) Circuit

- Short Circuit

- Continuity

- Energized Circuit

- Polarity

- Reverse Polarity

- Polarity Sensitive

- Forward Bias

- Reverse Bias

  - An LED is installed backward and does not illuminate. Identify the bias condition and explain why current does not flow.

  - A diode has been installed backward. Describe its bias condition and predict the circuit behavior.

  - Before installing a diode, which two polarity checks should be made?

  - A polarized capacitor is connected backward. Predict the failure risk and explain the safety concern.

  - Explain why diode orientation matters to current flow.

- The two sides of a circuit are the __________________ side and the __________________ side.

  - The __________________ side is located between the power source and the load input.

  - The __________________ side is located between the load output and ground.

- Component input is on the __________________ side.

- Component output is on the __________________ side.

## Circuit Type:

- _________________________	- A circuit with 1 load

- _________________________	- A circuit with at least 2 loads and one path

- _________________________	- A circuit with multiple paths

- _________________________	- A circuit with at least three loads

- _________________________	- A circuit with a switch before loads

- _________________________	- A circuit with the load before the switch

- _________________________	- A circuit with a switch after the loads

- _________________________	- A circuit with the load after the switch

## Name the circuit:

| - Wire 1: B+ to Switch Input - Wire 2: Switch Output to Load Input - Wire 3: Load Output to B- |  |  |
| --- | --- | --- |

## Connect the Circuit

| Use all loads to connect create a ground-controlled parallel circuit | Connect the dots to create a ground-controlled circuit |
| --- | --- |

Fill in Components

| Add Components Grd Series-Parallel - Falstad | Add Components - Grd Parallel - Falstad |
| --- | --- |
| Add Components - Grd Series - Falstad | Add Components - Pow Series-Parallel - Falstad |

## Wire up the Circuit

|  | Wire A. | (B+) | to | Fuse Input |
| --- | --- | --- | --- | --- |
|  | Wire B. |  | to |  |
|  | Wire C. |  | to |  |
|  | Wire D. |  | to |  |
|  | Wire E. |  | to |  |
|  | Wire F. |  | to |  |
|  | Wire G. |  | to |  |

# Section 3 | Multimeter Skills

For every measurement, identify the meter mode, lead ports, probe placement, circuit state, and expected reading.

## Symbols on a Meter

| Alternating Current |  |  | Resistance Testing Symbol |  |
| --- | --- | --- | --- | --- |
| Direct Current |  |  | Diode Testing Symbol |  |
| Voltage Testing Symbol |  |  | Continuity Testing Symbol |  |
| Amperage Testing Symbol |  |  | Frequency Testing Symbol |  |

Match the Test to the Description

| Voltmeter | Measures electrical potential difference and is connected in parallel with the circuit or component. |
| --- | --- |
| Ammeter | Measures current and must be connected in series so circuit current flows through the meter. |
| Ohmmeter | Measures resistance across an isolated component in a de-energized circuit using the meter’s internal source. |
| Continuity Tester | Checks for a complete low-resistance path and sounds an audible tone when continuity is present. |

## Multimeter Lead Placement

- Which multimeter port receives the black lead?

- Which multimeter port receives the red lead for voltage and resistance tests?

- Where are the probes placed within the circuit when performing an available voltage test?

- Where are the probes placed within the circuit when performing a voltage drop test?

- Where are the probes placed within the circuit when performing a voltage drop test of the power side?

- Where are the probes placed within the circuit when performing a resistance test?

- How must the meter be connected when performing an amperage test?

| Measuring Voltage of Battery | Measuring (V) at input of Load |
| --- | --- |

## Converting Unit Ranges

| 1mV = ______________V 1 kV = ______________V 1 MV = ______________V | 1mΩ = ______________Ω 1 kΩ = ______________Ω 1 MΩ = ______________Ω | 1mA = ______________A 1 kA = ______________A 1 MA = ______________A | 1mW = ______________W 1 kW = ______________W 1 MW = ______________W |
| --- | --- | --- | --- |

# Section 4 | Electrical Diagnostics

Diagnose from evidence: verify the complaint, predict normal values, test strategically, and confirm the repair.

## Six-Step Diagnostic Process

Step 1:

Step 2:

Step 3:

Step 4:

Step 5:

Step 6:

Circuit Malfunction Types:

Three Types of Customer Complaints

- No Operation: 		An Example: ___________________________________________________________________________

- Poor Operation: 		An Example: ___________________________________________________________________________

- Unwanted Operation: 	An Example: ___________________________________________________________________________

Define each root cause of an electrical fault and give an automotive example.

- Open Circuit

- Unwanted Resistance

- Shorted Circuit

- Intermittent Faults

- Hard Fault

## Voltage Prediction

| 1. Available Voltage at input of Load | 6. Voltage Drop of Power Side |
| --- | --- |
| 2. Available Voltage at output of Load | 7. Voltage Drop of Ground Side |
| 3. Available Voltage at input of switch | 8. Voltage Drop of Switch |
| 4. Available Voltage at output of switch | 9. Voltage Drop of all loads in a series circuit |
| 5. Voltage Drop of Load |  |

| - There should be ___V after load with power control switch open | - There should be ___V before load with power control switch open |
| --- | --- |
| - There should be ___V after load with power control switch closed | - There should be ___V before load with power control switch closed |
| - There should be ___V after load with ground control switch open | - There should be ___V before load with ground control switch open |
| - There should be ___V after load with ground control switch closed | - There should be ___V before load with ground control switch closed |

## Check Your Predictions

Compare each reading with the circuit state and explain why the meter displays that value.

## Available-Voltage Pattern Reference

Use the node pattern to connect circuit position, switch state, and expected voltage.

# Section 5 | Ohm’s Law & Watt’s Law

Formula reference + calculation practice. Write the formula, substitute values, solve, and label the unit.

## Quick Reference

| OHM’S LAW | WATT’S LAW |
| --- | --- |
| E = I x R | P = E x I |
| I = E / R | P = I² x R |
| R = E / I | P = E² / R |

Unit key: E = voltage (V)  |  I = current (A)  |  R = resistance (Ω)  |  P = power (W)

## Circuit Rules

| VALUE | SERIES CIRCUIT | PARALLEL CIRCUIT |
| --- | --- | --- |
| Voltage | Individual drops add to source voltage | Same across every branch |
| Current | Same everywhere in the circuit | Branch currents add to total current |
| Resistance | Rt = R1 + R2 + … | 1/Rt = 1/R1 + 1/R2 + … |
| Power | Individual power values add to total | Individual power values add to total |

## Solve Every Problem This Way

- List the known values and the unknown value, including units.

- Choose a formula that contains the unknown and the known values.

- Substitute, solve, and carry the correct unit through the calculation.

- Perform a reasonableness check: does the answer fit the circuit behavior?

## Calculation Practice

Use the four-step method. Show substitutions and label every final answer.

- A 12.6 V source is connected across a 4.2 Ω load. Calculate circuit current.

- A component carries 2.5 A and has 0.80 Ω of resistance. Calculate its voltage drop.

- A 12.0 V motor draws 3.0 A. Calculate electrical power.

- A 12.0 V series circuit contains 2 Ω, 4 Ω, and 6 Ω loads. Find Rt, It, and each voltage drop.

- Three parallel branches of 6 Ω, 12 Ω, and 4 Ω are connected to 12.0 V. Find each branch current, It, Rt, and total power.

## Parallel Circuits

| - Et _____ - It _____ - R1 _____ - R2 _____ - R3 _____ - IR1 _____ | - IR2 _____ - IR3 _____ - ER1 _____ - ER2 _____ - ER3 _____ |
| --- | --- |

# Section 6 | Essential Questions

Answer in complete, technically accurate sentences. Use an automotive example when it strengthens your explanation.

- What is electricity?

- Define voltage.

- What is current?

- What is the unit of resistance?

- How is an electron related to electricity?

- What are conductors and insulators? Can you provide an example of each?

- What is the difference between AC (Alternating Current) and DC (Direct Current)?

- What is a circuit?

- What does a switch do in an electrical circuit?

- What are the positive and negative battery terminals, and what does each represent?

- What is Ohm's Law?

- What is the purpose of a resistor?

- How does a light bulb work?

- What are the dangers of a short circuit?

- Why do we use fuses or circuit breakers?

- What is the function of a capacitor?

- What is the basic principle behind a transformer?

- What does a diode do?

- Why is grounding important in electrical circuits?

- What is the purpose of a transistor?

- Which meter modes measure voltage, current, and resistance, and how is each meter connected?

- Why are wires typically coated or insulated?

- What's the difference between series, parallel, and series-parallel circuits?

- What is electromagnetic induction?

- Why shouldn't you use electrical equipment near water?

- What does LED stand for and how does it differ from a traditional bulb?

- What is the role of a circuit-protection device in an automotive circuit?

- Why is measured amperage less than calculated amperage?

## Readiness Reflection

The topic I can explain most confidently is:

The topic I need to review again is:

One specific question I will ask before the midterm is:

My study plan (day / time / resource) is: