AUTOMOTIVE ELECTRICAL FUNDAMENTALS

# OHM'S LAW WORKSHEET

Voltage • Current • Resistance • Diagnostic Reasoning

| STUDENT NAME | DATE | CLASS / PERIOD |
| --- | --- | --- |

| LEARNING TARGET Solve for voltage, current, or resistance. | APPLICATION Use Ohm's Law in 12-volt vehicle circuits. | TOTAL SCORE _____ / 44 points |
| --- | --- | --- |

## Ohm's Law Formula Dashboard

| FIND VOLTAGE V = I x R volts = amps x ohms | FIND CURRENT I = V / R amps = volts / ohms | FIND RESISTANCE R = V / I ohms = volts / amps |
| --- | --- | --- |

Unit check: 1 ampere (A) = 1,000 milliamperes (mA)  |  1 kilohm (kΩ) = 1,000 ohms (Ω)

SHOW YOUR PROCESS For every calculation: (1) write the formula, (2) substitute values with units, (3) solve, and (4) label the final answer. Round to two decimal places when needed.

# Section 1 - Circuit Foundations  |  8 points

Complete each statement. One point each.

| # | Prompt | Student response |
| --- | --- | --- |
| 1 | The electrical pressure or potential difference in a circuit is called __________. | ________________ |
| 2 | The letter used for electrical current in formulas is __________. | ________________ |
| 3 | Resistance is measured in __________, represented by the symbol __________. | ________________ |
| 4 | At constant voltage, increasing resistance causes current to __________. | increase / decrease |
| 5 | At constant resistance, increasing voltage causes current to __________. | increase / decrease |
| 6 | Convert 250 mA to amperes. | ________________ A |
| 7 | Convert 0.04 A to milliamperes. | ________________ mA |
| 8 | Write the rearranged Ohm's Law formula used to solve for resistance. | R = ________________ |

# Section 2 - Select the Formula  |  6 points

Write the correct Ohm's Law formula only. Do not calculate. One point each.

| # | Situation | Formula |
| --- | --- | --- |
| 1 | Voltage and resistance are known. Find current. | ________________ |
| 2 | Current and resistance are known. Find voltage. | ________________ |
| 3 | Voltage and current are known. Find resistance. | ________________ |
| 4 | A 12 V source feeds a 6 Ω load. State the formula for the unknown current. | ________________ |
| 5 | A circuit draws 0.50 A at 5 V. State the formula for the unknown resistance. | ________________ |
| 6 | A 30 Ω resistor carries 0.20 A. State the formula for the unknown voltage. | ________________ |

# Section 3 - Calculate the Missing Quantity  |  12 points total

Show the formula and substitution. Place the final answer, with units, in the last column. One point each.

| # | Given / Find | Formula and substitution | Final answer |
| --- | --- | --- | --- |
| 1 | I = 2.50 A, R = 4.00 Ω; find V | Formula: Substitution: | ________________ |
| 2 | I = 0.40 A, R = 15 Ω; find V | Formula: Substitution: | ________________ |
| 3 | I = 0.025 A, R = 220 Ω; find V | Formula: Substitution: | ________________ |
| 4 | I = 8.00 A, R = 0.15 Ω; find V | Formula: Substitution: | ________________ |
| 5 | V = 12.0 V, R = 6.00 Ω; find I | Formula: Substitution: | ________________ |
| 6 | V = 14.4 V, R = 3.60 Ω; find I | Formula: Substitution: | ________________ |

# Section 3 - Calculate the Missing Quantity (continued)

| # | Given / Find | Formula and substitution | Final answer |
| --- | --- | --- | --- |
| 7 | V = 5.00 V, R = 100 Ω; find I | Formula: Substitution: | ________________ |
| 8 | V = 12.6 V, R = 21 Ω; find I | Formula: Substitution: | ________________ |
| 9 | V = 12.0 V, I = 3.00 A; find R | Formula: Substitution: | ________________ |
| 10 | V = 5.00 V, I = 0.020 A; find R | Formula: Substitution: | ________________ |
| 11 | V = 9.00 V, I = 0.30 A; find R | Formula: Substitution: | ________________ |
| 12 | V = 14.0 V, I = 7.00 A; find R | Formula: Substitution: | ________________ |

QUICK SELF-CHECK Does the answer make sense? At the same voltage, a lower resistance should produce a higher current. At the same resistance, a higher voltage should produce a higher current.

## Optional Challenge - Think Before You Calculate

Use these prompts for extra practice or as a fast-finisher activity. Explain your reasoning, not just the numerical answer.

|  | Prompt | Response |
| --- | --- | --- |
| A | Two loads receive 12 V. Load A is 3 Ω and Load B is 6 Ω. Calculate both currents and identify which load draws more. | ________________________ |
| B | If resistance doubles while voltage stays constant, what happens to current? Support your answer with Ohm's Law. | ________________________ |
| C | Create an automotive Ohm's Law problem using two known quantities. Solve for the third quantity with units. | ________________________ |

# Section 4 - Automotive Circuit Applications  |  10 points

Solve each vehicle-circuit problem. Show your formula, substitution, and labeled answer. Two points each.

| # | Vehicle scenario | Student work |
| --- | --- | --- |
| 1 | A blower motor has 1.50 Ω of resistance and receives 12.0 V. How much current should it draw? | Formula: Substitution: Answer: ________________ |
| 2 | A headlamp draws 4.00 A while 12.8 V is measured across it. What is the lamp's operating resistance? | Formula: Substitution: Answer: ________________ |
| 3 | A relay coil measures 80 Ω and is supplied with 12.0 V. Calculate coil current in amperes and milliamperes. | Formula: Substitution: Answer: ________________ |
| 4 | A 5.0 V sensor circuit draws 2.5 mA. What is the circuit resistance? Convert milliamperes before calculating. | Formula: Substitution: Answer: ________________ |
| 5 | A fuel pump measures 0.75 Ω and receives 13.5 V with the engine running. Calculate expected current draw. | Formula: Substitution: Answer: ________________ |

DIAGNOSTIC HABIT Before judging a component, compare calculated current with the manufacturer's specification and verify the voltage measured under load.

# Section 5 - Diagnostic Reasoning  |  8 points

Use Ohm's Law to interpret each fault. One point for each response.

SCENARIO A - ADDED RESISTANCE A blower motor is rated at 1.50 Ω. Battery voltage at the circuit is 12.0 V, but measured circuit current is only 6.0 A.

| # | Diagnostic question | Student response |
| --- | --- | --- |
| A1 | Calculate the circuit's actual total resistance. | ________________________ |
| A2 | How much unwanted resistance has been added to the circuit? | ________________________ |
| A3 | Name one likely location for the unwanted resistance. | ________________________ |
| A4 | State one expected effect on blower operation. | ________________________ |

SCENARIO B - LOW-RESISTANCE FAULT A 5.0 V bench sensor circuit should have 2,000 Ω of resistance and draw 2.5 mA. Actual measured current is 10 mA.

| # | Diagnostic question | Student response |
| --- | --- | --- |
| B1 | Calculate the circuit's actual equivalent resistance. | ________________________ |
| B2 | How far is the actual resistance from the expected resistance? | ________________________ |
| B3 | Does the evidence indicate an open/high-resistance fault or a short/low-resistance fault? | ________________________ |
| B4 | How many times greater is actual current than expected current? | ________________________ |

## Exit Ticket - Explain the Relationship

In one clear sentence, explain why current decreases when resistance increases while voltage stays constant.

| Confidence check: circle one | 1 ¦ 2 ¦ 3 ¦ 4 |
| --- | --- |

| INSTRUCTOR ANSWER KEY | REMOVE BEFORE COPYING |
| --- | --- |

Scoring: 44 points total. Accept equivalent wording and correctly rounded values with proper units.

## Section 1 - Circuit Foundations

| # | Correct response | Teaching note |
| --- | --- | --- |
| 1 | Voltage | Electrical potential difference / pressure. |
| 2 | I | Current is measured in amperes. |
| 3 | ohms; Ω | Both the written unit and symbol are required. |
| 4 | decrease | At fixed V, I = V / R. |
| 5 | increase | At fixed R, I rises with V. |
| 6 | 0.25 A | 250 / 1,000 = 0.25. |
| 7 | 40 mA | 0.04 x 1,000 = 40. |
| 8 | R = V / I | Resistance equals voltage divided by current. |

## Section 2 - Select the Formula

| # | Correct response | Teaching note |
| --- | --- | --- |
| 1 | I = V / R | Solve for current. |
| 2 | V = I x R | Solve for voltage. |
| 3 | R = V / I | Solve for resistance. |
| 4 | I = V / R | 12 V and 6 Ω are known. |
| 5 | R = V / I | 5 V and 0.50 A are known. |
| 6 | V = I x R | 0.20 A and 30 Ω are known. |

## Section 3 - Calculate the Missing Quantity

| # | Given / Find | Correct setup | Answer |
| --- | --- | --- | --- |
| 1 | I = 2.50 A; R = 4.00 Ω; find V | V = 2.50 x 4.00 | 10.0 V |
| 2 | I = 0.40 A; R = 15 Ω; find V | V = 0.40 x 15 | 6.0 V |
| 3 | I = 0.025 A; R = 220 Ω; find V | V = 0.025 x 220 | 5.50 V |
| 4 | I = 8.00 A; R = 0.15 Ω; find V | V = 8.00 x 0.15 | 1.20 V |
| 5 | V = 12.0 V; R = 6.00 Ω; find I | I = 12.0 / 6.00 | 2.00 A |
| 6 | V = 14.4 V; R = 3.60 Ω; find I | I = 14.4 / 3.60 | 4.00 A |
| 7 | V = 5.00 V; R = 100 Ω; find I | I = 5.00 / 100 | 0.05 A (50 mA) |
| 8 | V = 12.6 V; R = 21 Ω; find I | I = 12.6 / 21 | 0.60 A |
| 9 | V = 12.0 V; I = 3.00 A; find R | R = 12.0 / 3.00 | 4.00 Ω |
| 10 | V = 5.00 V; I = 0.020 A; find R | R = 5.00 / 0.020 | 250 Ω |
| 11 | V = 9.00 V; I = 0.30 A; find R | R = 9.00 / 0.30 | 30 Ω |
| 12 | V = 14.0 V; I = 7.00 A; find R | R = 14.0 / 7.00 | 2.00 Ω |

| INSTRUCTOR ANSWER KEY | REMOVE BEFORE COPYING |
| --- | --- |

Scoring: 44 points total. Accept equivalent wording and correctly rounded values with proper units.

## Section 4 - Automotive Circuit Applications

| # | Correct response | Teaching note |
| --- | --- | --- |
| 1 | I = 12.0 V / 1.50 Ω = 8.00 A | Expected blower current. |
| 2 | R = 12.8 V / 4.00 A = 3.20 Ω | Operating resistance. |
| 3 | I = 12.0 V / 80 Ω = 0.15 A = 150 mA | Accept both units. |
| 4 | 2.5 mA = 0.0025 A; R = 5.0 / 0.0025 = 2,000 Ω (2.0 kΩ) | Conversion must occur before division. |
| 5 | I = 13.5 V / 0.75 Ω = 18.0 A | Expected pump current. |

## Section 5 - Diagnostic Reasoning

| # | Correct response | Teaching note |
| --- | --- | --- |
| A1 | R = 12.0 V / 6.0 A = 2.00 Ω | Actual total circuit resistance. |
| A2 | 2.00 Ω - 1.50 Ω = 0.50 Ω unwanted resistance | Subtract rated load resistance. |
| A3 | Examples: connector, terminal, splice, power wire, or ground | Accept one reasonable high-resistance location. |
| A4 | Lower blower speed / reduced airflow; a poor connection may heat | Current is below the expected 8 A. |
| B1 | R = 5.0 V / 0.010 A = 500 Ω | 10 mA = 0.010 A. |
| B2 | 2,000 Ω - 500 Ω = 1,500 Ω below expected | Actual is one-quarter of expected resistance. |
| B3 | Short / low-resistance fault | Not an open or high-resistance fault. |
| B4 | 10 mA / 2.5 mA = 4 times expected current | The current is quadrupled. |

## Suggested Performance Guide

| Score | Level | Instructional response |
| --- | --- | --- |
| 40-44 | Mastery | Ready for multi-load circuits, voltage-drop testing, and power. |
| 35-39 | Proficient | Correct minor unit-conversion or rounding errors. |
| 31-34 | Developing | Reinforce formula selection and substitution. |
| 0-30 | Needs reteaching | Return to V-I-R meaning, units, and guided examples. |