STUDENT LAB WORKSHEET

Ohm's Law: Make the Circuit Prove It

Predict  •  Build  •  Measure  •  Explain

| DURATION 3–4 HOURS | TEAM 2 STUDENTS | INVESTIGATIONS 4 | TOTAL 100 POINTS |
| --- | --- | --- | --- |

| NAME |  | PARTNER |  |
| --- | --- | --- | --- |
| DATE |  | CLASS / BLOCK |  |

MISSION Use a digital multimeter and a working DC circuit to gather evidence that current changes predictably when voltage or resistance changes.

## Learning Targets

| □ Select and rearrange the correct Ohm's law equation. | □ Build a simple DC circuit and verify it before energizing. |
| --- | --- |
| □ Measure voltage, current, and resistance correctly. | □ Compare predicted and measured values using evidence. |

## Formula & Unit Card

| FIND VOLTAGE V = I × R volts (V) | FIND CURRENT I = V ÷ R amperes (A) | FIND RESISTANCE R = V ÷ I ohms (Ω) |
| --- | --- | --- |

mA → A: divide by 1,000    |    A → mA: multiply by 1,000    |    % error = |measured − predicted| ÷ predicted × 100

## Safety & Quality Gate

| □ | Power OFF before measuring resistance or moving meter leads. |
| --- | --- |
| □ | Ammeter connects in series; voltmeter connects in parallel. |
| □ | Never place the ammeter directly across a battery or power source. |
| □ | Begin on a safe current range and verify the correct meter jack. |
| □ | Use the instructor-assigned supply and load; do not exceed ratings. |
| □ | Open the switch between tests and ask for an inspection before power. |

Student initials: ________________    Instructor safety approval: ________________

# LAB 1  |  Meter & Resistor Reality Check

Prove your meter setup and component values before energizing a circuit.

MATERIALS DMM, jumper wires, knife switch, battery holder or 0–5 V supply, four assigned resistors, and safety glasses as directed.

## A. Quick Readiness Check

| 1. Write the unit for each quantity: voltage = ______; current = ______; resistance = ______. |  |
| --- | --- |
| 2. A 4.5 V source is connected to 330 Ω. Predict current in amperes and milliamperes. |  |
| 3. If voltage doubles while resistance stays constant, current will ____________________. |  |
| 4. If resistance doubles while voltage stays constant, current will ____________________. |  |
| 5. Circle the correct rule: resistance is measured with power ON / OFF. |  |

## B. Correct Meter Placement

If only one DMM is available, de-energize the circuit before changing the meter mode, jack, or connection.

| METER LEADS | COM + correct V/Ω or A/mA jack | APPROVAL | Instructor initials: ____________ |
| --- | --- | --- | --- |

## C. Resistor Verification — Power OFF

Identify four assigned resistors, calculate each acceptable tolerance range, then compare the DMM reading with the labeled value.

| ID | Color Bands / Label | Nominal R (Ω) | Tolerance (%) | Min–Max Acceptable (Ω) | Measured R (Ω) | Pass? Y / N |
| --- | --- | --- | --- | --- | --- | --- |
| A |  |  |  |  |  |  |
| B |  |  |  |  |  |  |
| C |  |  |  |  |  |  |
| D |  |  |  |  |  |  |

Tolerance formula: minimum = nominal × (1 − tolerance); maximum = nominal × (1 + tolerance)

EVIDENCE 1. Which resistor was closest to its nominal value? Support your answer with numbers.

EVIDENCE 2. Why can a good resistor measure slightly above or below its labeled value?

# LAB 2  |  Voltage Drives Current

Change voltage while keeping resistance constant. Record the effect on current.

CONTROLLED VARIABLE Use the same resistor for every trial. Recommended: 330 Ω, ¼ W. Record your measured resistor value below.

| RESISTOR USED | Nominal: ______ Ω ¦ Measured: ______ Ω | VOLTAGE LEVELS | 1.5 V, 3.0 V, and 4.5 V |
| --- | --- | --- | --- |

## Build & Test

- Open the switch. Build a series path: source → switch → resistor → ammeter → source.

- Connect the voltmeter in parallel across the resistor. If using one DMM, take voltage and current readings separately with power OFF between changes.

- Set the source to 1.5 V. Measure actual voltage. Calculate predicted current before closing the switch.

- Ask for inspection, close the switch briefly, record current, then open the switch.

- Repeat at 3.0 V and 4.5 V without changing the resistor.

## Data Table

| Trial | Source Setting | Measured V (V) | Measured R (Ω) | Predicted I (mA) | Measured I (mA) | % Error |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 1.5 V |  |  |  |  |  |
| 2 | 3.0 V |  |  |  |  |  |
| 3 | 4.5 V |  |  |  |  |  |

## Show One Complete Calculation

| Predicted current | I = V ÷ R = __________ V ÷ __________ Ω = __________ A = __________ mA |
| --- | --- |
| % error | ¦__________ − __________¦ ÷ __________ × 100 = __________ % |

## Graph & Explain

| 1. Relationship Circle one: DIRECT / INVERSE / NO CLEAR PATTERN | 2. Evidence When voltage increased from ______ V to ______ V, current changed from ______ mA to ______ mA. |
| --- | --- |
| 3. Prediction At 6.0 V with the same resistor, predicted current = __________ mA. | 4. Explanation Describe why your measured values may not exactly match the calculated values. |

# LAB 3  |  Resistance Controls Current

Hold voltage steady. Change resistance and observe how current responds.

FIXED SUPPLY Use 4.5 V (three AA cells or the instructor-set supply). Use only ¼ W or higher resistors: 220 Ω, 470 Ω, 680 Ω, and 1,000 Ω.

## Build & Test

- Open the switch and install the 220 Ω resistor. Measure its actual resistance with power OFF.

- Calculate predicted current using the measured supply voltage and measured resistance.

- Inspect the meter mode and leads. Close the switch briefly, measure current, then open the switch.

- Repeat with 470 Ω, 680 Ω, and 1,000 Ω. Keep the supply voltage unchanged.

- Touch components only after power is removed. Report unusual heat immediately.

## Data Table

| Trial | Resistor Label (Ω) | Measured R (Ω) | Measured V (V) | Predicted I (mA) | Measured I (mA) | % Error |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 220 |  |  |  |  |  |
| 2 | 470 |  |  |  |  |  |
| 3 | 680 |  |  |  |  |  |
| 4 | 1,000 |  |  |  |  |  |

## Show One Complete Calculation

| Predicted current | I = V ÷ R = __________ V ÷ __________ Ω = __________ A = __________ mA |
| --- | --- |
| Resistance check | R = V ÷ I = __________ V ÷ __________ A = __________ Ω |

## Graph & Explain

| 1. Relationship Circle one: DIRECT / INVERSE / NO CLEAR PATTERN | 2. Rank Largest current → smallest current: ______ Ω, ______ Ω, ______ Ω, ______ Ω |
| --- | --- |
| 3. Pattern When resistance increased, current ______________________________________. | 4. Automotive link Added resistance from corrosion causes component current to __________ and performance to __________. |

# LAB 4  |  The Miniature Lamp: A Changing Resistance

Compare a filament's resistance when it is cold and when it is operating.

CAUTION Use only the instructor-assigned bulb and supply. The bulb rating must match the test voltage. Open the switch if the lamp becomes unusually bright or hot.

## A. Cold-Filament Check — Power OFF

| BULB RATING ________ V / ________ A (if marked) | COLD RESISTANCE R₍cold₎ = __________ Ω | METER MODE Ω ¦ Leads removed from power source | TEAM CHECK Partner verified power OFF: □ Yes |
| --- | --- | --- | --- |

## B. Operating Test

- Build a series circuit with source → switch → lamp → ammeter → source.

- Connect the voltmeter in parallel across the lamp, or take voltage and current readings separately with power OFF between meter changes.

- Predict current using the cold resistance. Mark this prediction as an estimate only.

- Ask for inspection. Energize the lamp long enough to obtain a steady reading, then open the switch.

- Calculate the lamp's operating resistance and power from the measured values.

| Supply V (V) | Lamp V (V) | Current I (A) | Current I (mA) | Calculated Hot R = V ÷ I (Ω) | Lamp Power P = V × I (W) |
| --- | --- | --- | --- | --- | --- |

## C. Calculation Workspace

| Cold-current estimate | I = V ÷ R₍cold₎ = __________ V ÷ __________ Ω = __________ A = __________ mA |
| --- | --- |
| Hot resistance | R₍hot₎ = V ÷ I = __________ V ÷ __________ A = __________ Ω |
| Lamp power | P = V × I = __________ V × __________ A = __________ W |

## D. Compare & Explain

| 1. Compare values | R₍hot₎ was __________ times R₍cold₎. (R₍hot₎ ÷ R₍cold₎) |
| --- | --- |
| 2. Choose the evidence | The filament's resistance: □ increased □ decreased □ stayed constant |
| 3. Explain the physics | As the filament heated, its resistance ________________________, which caused current to ________________________. |

CER. Write one claim, one measured piece of evidence, and one sentence of reasoning showing why an incandescent lamp is not a constant-resistance load.

# CHALLENGE  |  Find the Fault

Use expected values, voltage checks, and power-off resistance checks to identify one hidden circuit fault.

FAULT BANK Instructor may install one safe fault: open connection, high-resistance connection, reduced source voltage, incorrect resistor, or loose connection. Do not create your own fault.

## 1. Establish the Normal Baseline

| SOURCE Expected: ______ V CURRENT Expected: ______ A or ______ mA | LOAD Component / value: __________________ OPERATION Expected behavior: ______________________ | CALCULATION I = V ÷ R = __________________ APPROVAL Baseline verified: □ Yes Initials: ______ |
| --- | --- | --- |

## 2. Record the Symptom Before Touching the Circuit

OBSERVE. What is the circuit doing—or not doing? Be specific.

COMPARE. How does the symptom differ from the baseline?

## 3. Diagnostic Measurements

| Check Point | Expected | Measured | Difference | What This Suggests |
| --- | --- | --- | --- | --- |
| Source voltage |  |  |  |  |
| Voltage before load |  |  |  |  |
| Voltage across load |  |  |  |  |
| Voltage after load / ground side |  |  |  |  |
| Circuit current |  |  |  |  |
| Power-off resistance check |  |  |  |  |

## 4. Diagnose, Repair, Verify

| Initial hypothesis | I think the fault is __________________________________ because __________________________________. |
| --- | --- |
| Root cause found | The actual fault was ______________________________________________________________. |
| Safe correction | We corrected it by ________________________________________________________________. |
| Verification | After repair: voltage = ______ V; current = ______; operation = ______________________. |

DIAGNOSTIC RULE Measure voltage on an energized circuit only when instructed. Measure resistance only with power OFF and the component isolated as directed. Return the meter lead to the V/Ω jack when finished.

CER. Claim the fault, cite two measurements as evidence, and explain how Ohm's law supports your diagnosis.

# PERFORMANCE TASK  |  Design to a Current Target

Use Ohm's law to select resistance, build the circuit, and prove that it meets the target.

DESIGN TARGET With a 4.5 V source, create a resistor circuit that draws 8–12 mA. Use only instructor-approved resistors. Series combinations are allowed.

## 1. Plan Before You Build

| Available resistor values | ____________________________________________________________ |
| --- | --- |
| Chosen value(s) | ____________________________________________________________ |
| Equivalent resistance | Rₑq = __________________ Ω |
| Predicted current | I = 4.5 V ÷ __________ Ω = __________ A = __________ mA |
| Predicted resistor power | P = V × I = __________ W ¦ Rating safe? □ Yes □ No |

## 2. Draw & Label Your Circuit

Draw standard symbols. Label the source, switch, resistor value(s), current direction, ammeter location, and voltage-measurement points.

Instructor build approval: __________________    Partner cross-check: □ Complete

## 3. Build, Measure & Decide

| Measured V (V) | Measured I (mA) | Calculated R (Ω) | % Error | Target Met? Y / N | Safe & Stable? Y / N |
| --- | --- | --- | --- | --- | --- |

DECISION. Did the circuit meet the target? If not, state the change you would make and support it with Ohm's law.

## Exit Ticket

| 1. A 12 V circuit has 6 Ω total resistance. Current = ____________________. |  |
| --- | --- |
| 2. A component has 5 V across it and 0.020 A through it. Resistance = ____________________. |  |
| 3. At constant voltage, increasing resistance causes current to ____________________. |  |
| 4. Why must an ammeter never be connected directly across a source? |  |
| 5. Write one sentence using your own data that proves Ohm's law. |  |

## Cleanup & Submission

| □ Switch open / power OFF | □ Meter lead returned to V/Ω | □ Components sorted | □ Work area clean |
| --- | --- | --- | --- |
| □ All tables complete | □ Units shown | □ Partner names listed | □ Worksheet submitted |

Instructor score: ________ / 100