POWER FLOW SEQUENCE

Reverse Engagement & Power Flow

From engine start to vehicle movement

| 01 BUILD PRESSURE Engine starts | 02 APPLY REVERSE Driver selects R | 03 MOVE VEHICLE Brake is released |
| --- | --- | --- |

Application note: This example uses Shift Solenoids A/B, a reverse-input clutch, and a low/reverse clutch; exact solenoid states and circuit names vary by transmission model.

# Engine Starts: Pressure Is Established

- Torque converter and pump: The engine turns the impeller and drives the transmission pump at engine speed. The pump draws automatic transmission fluid (ATF) from the sump and establishes line pressure.

- Pressure control: The pressure regulator valve controls line pressure and recirculates or returns excess fluid.

- Electronic command: For this example, the PCM commands Shift Solenoids A and B ON. The manual valve remains in Park, so the reverse-input circuit is not yet applied.

- Low/reverse clutch: The Park/Reverse circuit applies the low/reverse clutch, holding the reaction carrier for Reverse.

# Driver Selects Reverse: Apply Devices Engage

- Manual valve and pressure: Moving the selector to R shifts the manual valve and routes line pressure into the Reverse apply circuit.

- Reverse-input clutch: Apply pressure engages the reverse-input clutch pack, while the reverse-boost circuit increases pressure for the load.

- Power flow: The reverse-input clutch drives the reaction sun gear while the low/reverse clutch holds the reaction carrier. The reaction internal (ring) gear and output shaft rotate in the opposite direction, producing Reverse.

# Brake Is Released: The Vehicle Accelerates

- Brake applied: With the brakes holding the vehicle stationary in Reverse, the turbine is held by the driveline while the impeller turns. This is a stall condition.

- Initial movement: Releasing the brake allows the vehicle to creep backward at idle. Pressing the accelerator increases impeller speed and fluid flow.

- Torque multiplication: While the turbine is much slower than the impeller, the stator redirects returning fluid and multiplies torque.

- Coupling phase: As turbine and vehicle speed increase, the speed difference decreases, torque multiplication fades, and the converter moves toward fluid coupling.

KEY CORRECTION  |  Stall speed is not a threshold the vehicle must exceed before moving. Torque multiplication is strongest when the impeller-to-turbine speed difference is greatest, then decreases as the turbine catches up.