STUDENT LAB WORKSHEET

Stall-Speed Testing

Pre-test research, safe procedure, and diagnostic interpretation

| Student |  | Date |  |
| --- | --- | --- | --- |
| Vehicle / VIN |  | Transmission / Mileage |  |

Purpose: Safely prepare for, perform, record, and interpret a stall-speed test using current vehicle-specific service information.

SAFETY GATE: Do not perform the test until the instructor confirms that the manufacturer permits it and approves the RPM specification, loaded-test time, cooldown, wheel-chocking plan, and test area.

# A  Research Before Testing

Complete this page before starting or moving the vehicle.

1. Define stall speed in your own words.

2. List the precautions that must be verified before performing a stall test.

Vehicle-Specific Service Information

| Stall test permitted? | YES / NO | Approved RPM source |  |
| --- | --- | --- | --- |
| Reverse specification | ________ RPM | Drive specification | ________ RPM |
| Maximum loaded time | ________ seconds | Required cooldown |  |
| Required ECT / TFT |  | Wheel-chocking plan |  |
| Service information source |  | Instructor approval | Approved / Not approved |

# B  Pre-Test Inspection and Setup

Document what you observed—not only whether the item was checked.

STOP IF: The fluid level is unsafe, a severe leak is present, relevant DTCs require diagnosis first, the vehicle cannot be secured, or the manufacturer procedure is unavailable.

| # | Pre-Test Check | Student Findings | Complete |
| --- | --- | --- | --- |
| 3 | Park the vehicle on a level surface. | Location / bay: | Initials |
| 4 | Check transmission-fluid level and condition using the approved procedure. | Level: Color: Odor: Foam / debris: | Initials |
| 5 | If the fluid is low, inspect for leaks and document the location and severity. | Leak findings: | Initials |
| 6 | Explain what the observed fluid condition may indicate. | Interpretation: | Initials |
| 7 | Select and verify a readable RPM source. | Dash tach / Scan tool / External tach Method used: | Initials |
| 8 | Check all relevant modules for DTCs before testing. | Codes / discovery: | Initials |
| 9 | Verify coolant level and bring the engine and transmission to the required temperature. | ECT: TFT: Coolant condition: | Initials |

Instructor review: Confirm the pre-test inspection and authorize the loaded portion of the test.

| Instructor initials |  | Date / time |  |
| --- | --- | --- | --- |
| Authorization | Proceed / Do not proceed | Conditions |  |

# C  Perform and Record the Stall Test

One operator controls the vehicle. The instructor controls the test area and stop decision.

CRITICAL: No one may stand in front of or behind the vehicle. Release the throttle immediately if the vehicle moves, the wheels spin, RPM exceeds the expected limit, or any unsafe condition develops.

| # | Procedure | Record / Verify |
| --- | --- | --- |
| 10 | Apply the parking brake and install wheel chocks exactly as approved. Confirm the test area is clear. | Instructor check: |
| 11 | Start the engine in Park. Warm the engine and transmission to the specified temperature. | ECT: TFT: |
| 12 | Hold the service brake firmly and select Reverse. | Vehicle held securely: |
| 13 | Apply full throttle only for the approved duration. Record the highest RPM and release the throttle immediately. | Reverse max RPM: |
| 14 | Shift to Park or Neutral and complete the approved cooldown before another loaded test. | Cooldown completed: |
| 15 | If approved, select Drive and repeat the same controlled procedure. Record the highest RPM. | Drive max RPM: |

## Test Results

| Range | Specification | Recorded RPM | Difference | Classification |
| --- | --- | --- | --- | --- |
| Reverse |  |  |  | Within / High / Low |
| Drive |  |  |  | Within / High / Low |
| Other (approved) |  |  |  | Within / High / Low |

14a. What is the purpose of shifting to Park or Neutral and completing the required cooldown between loaded tests?  Explain heat control and component protection.

# D  Interpret the Results

A stall test narrows the diagnostic path; it does not replace service information or follow-up tests.

16. What may be indicated if the recorded stall speed is above specification?  Use the tested range and clutch chart.

17. What may be indicated if the recorded stall speed is below specification?  Separate engine-output concerns from converter/transmission concerns.

18. Find the clutch-application chart for this vehicle and record the components applied in each tested range.

| Range | Applied Clutches / Bands / One-Way Devices | Shared Components and Diagnostic Notes |
| --- | --- | --- |
| Reverse |  |  |
| Drive |  |  |
| Other approved gear |  |  |

19. Describe any manufacturer-approved method for testing other gears or ranges.  Do not invent a method; cite the service-information procedure.

## Diagnostic Conclusion and Next Test

Based on the readings and clutch chart, state your most likely diagnostic direction and the next test that would confirm or reject it.

| Student signature |  | Date |  |
| --- | --- | --- | --- |
| Instructor sign-off |  | Result | Satisfactory / Repeat |